# Vulnerability Ranking – Ground Water

Directions for using the spreadsheet

General Notes:

The list in the spreadsheet (GW Vulnerability Ranking List.xls, sheet “GW”) comes from the PCA checklists.  Each PCA is listed four (4) times: Zone A, Zone B5, Zone B10, and Unknown.

PCA risk points (Column C) have been assigned based on the risk of the PCA (VH, H, M, or L) for that zone.

To use the Vulnerability Ranking Spreadsheet follow these steps:

- Conduct the PCA inventory.

- Make a copy of the spreadsheet “GW” and give the new sheet a descriptive name.

- On the new sheet, delete rows for any PCAs that do not exist in any of the zones.  (Remember that each PCA is listed 4 times).

- Delete rows for PCAs that don’t occur in a particular zone (i.e. if the PCA exists only in Zone A, delete the rows for Zone B5, B10 and Unknown).

- For PCAs whose existence is unknown, delete the appropriate rows (i.e. if you have no idea whether the PCA exists, keep the Unknown row and delete the other three.  Similarly, if you know the PCA exists in Zone A but you aren’t sure about B5 and B10, delete the rows for B5 and B10 and keep the rows for Zone A and Unknown.)

- The remaining rows should represent all of the PCAs that exist or whose existence is unknown within the zones.  (For some sources, especially in urban areas, this may still be a very long list.)

- Calculate Physical Barrier Effectiveness for the source.  Insert the corresponding points 
(i.e., H =1, M = 3, L = 5) into column E for all the rows.

- Determine the Vulnerability Score in Column F (sum of columns C + D + E)

- Sort the list by the Vulnerability Score, from highest to lowest.

- The source is considered vulnerable to all PCAs with vulnerability score   8.  Shade these yellow.

- Review the list.  This is a good time to review the assessment with the water supplier.  Items to consider:

  - Are there detected contaminants in the source water?  What are the PCAs associated with the contaminants?  Are those PCAs at the top of the list?  If not, move them there with a note or asterisk.

  - What were the perceived biggest problems before doing the assessment?  Are these PCAs at the top of the list?  Should they be?

  - Are there PCAs at the top of the list that don’t seem particularly important?

- If there are any concerns with the vulnerability ranking, go back and review the PCA inventory.  Revise the inventory as necessary.

- Revise the vulnerability ranking as necessary.

- Print the final list, save to disk, and submit with the remainder of the assessment.