CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN FRANCISCO BAY REGION

SELF-MONITORING PROGRAM

for

Bolinas Community Public Utility District

Marin County, California

ORDER  R2-2004-0058

July 21, 2004

I.	GENERAL

Reporting responsibilities of waste dischargers are specified in Sections 13225(a), 13267(b), 13268, 13383, and 13387(b) of the California Water Code.

The principle purposes of a monitoring program by a waste discharger, also referred to as a 		self-monitoring program, are:

1.	To document compliance with wastewater requirements and prohibitions established by 			this Board; and

2.	To facilitate self-policing by the discharger in the prevention and abatement of pollution 			arising from wastewater treatment and disposal.

II.	SAMPLING AND ANALYTICAL METHODS

Sample collection, storage, and analyses shall be performed according to Code of Federal Regulations Title 40, Section 136 (40 CFR S136), or other methods approved and specified by the Executive Officer of this Board.

Wastewater analyses shall be performed by a laboratory approved for these analyses by the State Department of Health Services (DHS), or a laboratory waived by the Executive Officer from obtaining a DHS certification for these analyses.

The director of the laboratory whose name appears on the certification, or his/her laboratory supervisor who is directly responsible for the analytical work performed shall supervise all analytical work including appropriate quality assurance/quality control procedures in his/her laboratory and shall sign all reports of such work submitted to the Board.

All monitoring instruments and equipment shall be properly calibrated and maintained to ensure accuracy of measurements.

III.		DEFINITION OF TERMS

### Recycled Water Production and Distribution Areas

Bolinas Community Pubic Utility District (BCPUD) is the producer of recycled water that will be applied to athletic fields on Mesa Park. Recycled water may also be distributed to other sites in the future subject to the Executive Officer’s approval. The BCPUD is responsible for the quality of the recycled water released for distribution to designated water reuse sites and for the oversight of operation and maintenance of the recycled water distribution facilities. The BCPUD shall require Users to submit self-monitoring data to BCPUD to be incorporated into the BCPUD’s SMP as required by the Producer/User water recycling agreement. The BCPUD is responsible for submittal of the required monitoring reports to the Board.

### SAMPLES

1.	A grab sample is an individual sample collected in a short period of time not exceeding 15 minutes. Grab samples represent only conditions existing at the time of sample collection. Grab samples are used primarily in determining compliance with daily or instantaneous maximum limits.

2.	A flow sample is the accurate measurement of the average flow volume over a given period of time, using a properly calibrated and maintained flow measuring device. Flows calculated from properly maintained pump useage records for accurately calibrated pumps are acceptable.

### STANDARD OBSERVATIONS

Recycled Water Use Areas

1.	Evidence of any recycled water escaping the recycled water use area through airborne spray (show affected area on a sketch).

2.	Evidence of any prolonged ponding of recycled water, or of mosquitoes breeding within the use area due to ponding.

3.	Observations recorded and reported along irrigation distribution system including but not limited to broken or poorly adjusted spray equipment.

IV.	DESCRIPTION OF SAMPLING AND OBSERVATION STATIONS

NOTE: A sketch showing identification and locations of all stations described below shall accompany the first monitoring report, and subsequent reports when locations are added or changed, or a violation is reported.

- WASTEWATER TREATMENT SYSTEM

STATION	DESCRIPTION

A-1	= At a point prior to the dissolved air flotation unit, and after the discharge point from

Pond 3.

A-2	= At a point prior to the media filters, and after the dissolved air flotation unit.

E-1	= At a point prior to the sodium hypochlorite injector, and after the media filters.

E-2	= At a point prior to the chlorine contact chamber, and after the sodium hypochlorite injector.

E-3	= At a point prior to the recycled water wetwell which discharges to the RW storage tanks, and after the chlorine contact chamber.

E-4	= At a point in the irrigation system after the RW storage tanks.

2.	RECYCLED WATER USE AREA

STATION	DESCRIPTION

MP-1 thru MP -‘n’	Located at about 200 foot intervals around the perimeter of the

(Mesa Park) recycled water use areas (Obs. Intervals adjusted to area being irrigated).

#### V.		SCHEDULE OF SAMPLING, ANALYSES AND OBSERVATIONS

Sampling, analyses and observations shall be conducted according to the schedule given in Table 1 and Table 1 Footnotes (SMP Attachment A).

Initial System Monitoring

To document that the new wastewater treatment and distribution system is working effectively and can maintain long-term treatment reliability, the chemical and coliform sampling and analyses program in Table 1 shall be performed on a more frequent basis until the system establishes a stable and consistent system performance level.

VI. 	GENERAL OPERATION and MAINTENANCE (O&M)

The O & M Manual shall include scheduling of specific tasks to ensure the treatment and distribution systems will consistently and reliably perform according to the design criteria. All monitoring practices identified in the O&M Manual must be implemented.

VII.	REPORTS TO BE FILED WITH THE  BOARD

A.	Self-Monitoring Reports (SMR)

The BCPUD shall submit written reports to the Board for each calendar month. Reports shall be submitted to this Board’s office no later than the thirtieth day of the following month. Each SMR shall include the following:

1.		Letter of Transmittal, including:

(a).		Discharger’s name, address, phone number & contact person;

(b).		The monitoring period being reported, by month and year;

(c).		The name of the responsible Board staff member;

(d).		Tabulations of the results from all required sampling, analyses and observations specified in Table 1 and its Footnotes (SMP Attachment A) by date, sample type and station.

(e).		Discussion of violations found during the monitoring period, including causes and corrective actions taken or planned in order to prevent future violations;

(f).		Discussion of any special events pertinent to maintaining compliance with water reuse requirements, such as equipment repair or replacement, or operational changes;

(g).		Signatory statement by the BCPUD’s Recycled Water Supervisor or designated agent, under penalty of perjury, that to the best of the signer’s knowledge the report is true, accurate and complete

2.		SMP in Electronic Form:

The Discharger has the option to submit all monitoring results in an electronic reporting format approved by the Executive Officer.  Most dischargers in Region 2 are currently submitting SMRs electronically in a format approved by the Executive Officer in a letter dated December 17, 1999, Official Implementation of Electronic Reporting System (ERS). The ERS format includes, but is not limited to, a transmittal letter, summary of violation details and corrective actions, and transmittal receipt.  If there are any discrepancies between the ERS requirements and the "hard copy" requirements listed in the SMP, then the approved ERS requirements supercede.

B.	Report of Permit Violation

In the event the BCPUD violates, or threatens to violate the conditions of water reuse requirements due to:

        - Maintenance work, power failure, or breakdown of wastewater treatment or transport equipment;

        - Accidents caused by human error or negligence; or

        - Other causes such as acts of nature;

The BCPUD shall:

a.			Notify the Board office by telephone, as soon as the BCPUD, or agent, has knowledge of the event; and

b.			Submit a follow-up written report within two weeks of the event. The written report shall include pertinent information explaining reasons for the non-compliance, actions taken to correct the problem and dates thereof, and actions being taken to prevent the problems from recurring.

I, Bruce H. Wolfe, Executive Officer, do hereby certify that the foregoing Self-monitoring Program is effective on the date shown below and may be reviewed and amended at any time subsequent to the effective date upon written notice from the Executive Officer or upon written request from the BCPUD.

Bruce H. Wolfe

Executive Officer

Effective Date: July 21, 2004

Attachment A: Table 1

ATTACHMENT  A

BCPUD, Marin County

WATER REUSE REQUIREMENTS

Self-Monitoring Program

TABLE 1

SCHEDULE FOR SAMPLING, MEASUREMENTS, AND ANALYSES

| Sampling Station |  | A-1 | A-2 | E-1 | E-2 | E-3 | E-4 | MPs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Units |  |  |  |  |  |  |  |
| Flow Rate | Gal/day | C/X |  |  |  |  |  |  |
| BODS | Mg/L | G/W |  |  |  | G/W |  |  |
| Tot. Susp. Solids | Mg/L | G/W |  |  |  | G/W |  |  |
| Nitrate Nitrogen as N | Mg/L |  |  |  |  | G/W |  |  |
| Total Coliform | MPN/100ml |  |  |  |  | G/D | G/M |  |
| E. Coli | MPN/100ml |  |  |  |  |  | G/M |  |
| Turbidity | NTU |  | C/X | C/X |  |  |  |  |
| pH | SU | C/X |  |  |  | C/X |  |  |
| Chlorine Residual | Mg/L |  |  |  | C/X | C//X |  |  |
| CT Value | Min-mg/L |  |  |  |  | C/X |  |  |
| Standard Observations |  |  |  |  |  |  |  | Ob/2W |

FOOTNOTES FOR TABLE 1:

Type of Samples			Frequency of Sampling

G = grab sample				D= Daily, when TRWP is in operation

C= Continuously Measured/Recorded	W = Once each week

Ob=Std Observations			2/W = Two days per week

X = Continuously Monitored

M= Once per Month

Type of Stations

A-1  = TRWP Influent (Pond 3 Effluent)

A-2  = DAF effluent  (media filter influent)

E-1  = Media filter effluent prior to sodium hypochlorite injector

E-2  = Media filter effluent (chlorine contact chamber influent)

E-3  = Filtered and disinfected effluent from TRWP

E-4  = Recycled water in the distribution system

MPs = Recycled water use area observations