ATTACHMENT C

SELF-MONITORING PROGRAM

A.   	GENERAL

Reporting responsibilities of water recycling agencies are specified in

Sections 13255(a), 13267(b), 13268, 13383, and 13387(b) of the California Water

Code and this Regional Board’s Resolution No. 73-16.

The principal purposes of a monitoring program by water recyclers, also referred to as

the self-monitoring program, are:

1)   To document compliance with water quality requirements and prohibitions

established by this Regional Board; and

2)   To facilitate self-policing by the water recyclers.

B.   	RECYCLED WATER MONITORING

A Producer shall develop and implement a water reuse monitoring program. When

the User(s) is other than the Producer, delegation of responsibilities must be clearly

spelled out and included in the Producer’s Water Use Permits.

Recycled Water Effluent Quality - Producer Program

The Producer’s self-monitoring program is applicable during periods when recycled

water is in use. The SMP program shall include the observations, sampling,

measurements, and analyses prescribed in Table 1.

Description of Sampling and Observation Stations

a.   Recycled Water

Station        	Description

E—001      	 	Location at the wastewater treatment plant where a representative sample of treated 					effluent being diverted for

reuse can he obtained and total diverted flow can he

measured.

Monitoring of Recycled Water Users - Producer/User Program

Self-Monitoring Program

The Producer will set individual user monitoring requirements based on the size,

volume used, complexity, etc. of each use area. Producer conducted monitoring, or

user self-monitoring if approved by the Producer, shall be conducted at least annually.

All recycled use areas, while recycled water is being used, shall be inspected at a

frequency, specified by the Producer’s Water Reuse Permit , for the following

deficiencies or violations of use:

1.   Standard Observations

a.   Evidence of runoff of recycled water from the site (show affected area on

a sketch, estimate volume).

b.   Odor of wastewater origin from irrigation site. If present, indicate

apparent source, characterization, and direction of travel.

c.  	Evidence of ponding of recycled water, and evidence of mosquitoes

breeding within the irrigation area due to ponded water.

d.  	Warning signs properly posted to inform public that irrigation or water

use is recycled water which is not safe for drinking.

e.  	Evidence of leaks or breaks in the irrigation system pipelines or tubing.

f.  	Evidence of broken, or otherwise faulty drip irrigation system emitters or

spray irrigation sprinklers.

g.  	Evidence of overflows, leaks, erosion of dikes, etc. of storage pond(s) or

impoundment(s).

All violations shall be followed by a discussion of when and how deficiencies

were corrected.

2.  Description of Sampling and Observation Stations

a.  	Land Observation Stations

Station          	Description

L- 1 to L-n     Locations at a sufficient number of points at

reuse areas in order to ensure compliance with

water reuse requirements.

b.  Impoundment Stations

Station          	Description

P-1 to P-n     	Locations at points along the periphery of each

storage, ornamental, golf course, or other pond or

impoundment.

2

Inspection Program

A Producer shall also conduct periodic random inspections of Users to ensure

compliance with DHS reuse criteria and the Water Reuse Permit. Inspections shall be

performed when recycled water is being used. Any significant repairs or modifications

made to the system involving compliance with this Order shall be described in the

Annual Report.

C.  REPORTING REQUIREMENTS

1.  	Significant Violation Reporting

Violations of the DHS reuse criteria that impact or threaten to impact public

health or water quality shall be reported to the Board by phone within 24

hours, followed by a written report within 15 days describing corrective actions

taken.

2.  	Annual Report to the Regional Board

An annual report for each calender year shall be submitted to the Board by the

Producer by March 15 of each year. The report shall contain a statement by

the reporting official, under penalty of perjury, that to the best of the signer’s

knowledge the report is true and correct.

The report shall include:

a.  	Tabulation of SMP recycled water analyses (see Table 1).

b. 	 A tabular summary of recycled water use by billing period by each User.

c.  	A list of new authorized recycled water Users, including the name of

customers, application, source and projected annual flow to be delivered.

d.  	A summary of the total daily recycled water delivered by the Producer.

e.  	Tabulation of User site inspections conducted by the Producer.

f. 	 A summary of effluent violations related to recycled water use, violations

found during inspection of reuse sites, corrective actions taken and any

changes to, or revoking of User authorizations by the Producer.

In addition there shall be a comprehensive discussion of the progress

and results of the water recycling program. The discussion shall also

include:

g.  	An update regarding current and future development of the water

recycling program, including planning, design and construction of

facilities, preparation of required reports and technical documents

and progress toward regulatory approvals.

3

h. 	Progress and evaluation of any special studies or projects being

undertaken related to the program.

I, Loretta Barsamian, Executive Officer, do hereby certify that the foregoing Self-Monitoring

Program is effective on the date shown below and may be reviewed at any time subsequent

to the effective date upon written notice from the Executive Officer or request from the

Producers. Revisions to the SMP will be authorized in writing by the Executive Officer.

Loretta K. Barsamian

Executive Officer

Attachment:

Table 1 — Producer Schedule for Sampling, Measurements, and Analyses

File No. 2107.00

January 17, 1996

4