REGIONAL WATER QUALITY CONTROL BOARD

LOS ANGELES REGION

REGIONAL

TOXIC HOT SPOT CLEANUP PLAN

FEBRUARY 1999

REGIONAL TOXIC HOT SPOT

CLEANUP PLAN

REGIONAL WATER QUALITY CONTROL BOARD

LOS ANGELES REGION

Part I

I.	INTRODUCTION

In 1989, the California State legislature established the Bay Protection and Toxic Cleanup Program (BPTCP).    The BPTCP has four major goals: (1) to provide protection of present and future beneficial uses of the bays and estuarine waters of California; (2) identify and characterize toxic hot spots; (3) plan for toxic hot spot cleanup or other remedial or mitigation actions; (4) develop prevention and control strategies for toxic pollutants that will prevent creation of new toxic hot spots or the perpetuation of existing ones within the bays and estuaries of the State.

This Regional Toxic Hot Spot Cleanup Plan (Cleanup Plan) is intended to provide direction for the remediation or prevention of toxic hot spots in the Los Angeles Region (pursuant to Water Code Sections 13390 et seq.).      Pursuant to Sections 13140 and 13143 of the Water Code, this Cleanup Plan is necessary to protect the quality of waters and sediments of the State from discharges of waste, in-place sediment pollution and contamination, and any other factor that can impact beneficial uses of enclosed bays, estuaries and coastal waters.

This Cleanup Plan includes a specific definition of a Toxic Hot Spot and site ranking criteria from the Water Quality Control Policy for Guidance on the Development of Regional Toxic Hot Spot Cleanup Plans (Part I).    In Part II of the Cleanup Plan, the list of candidate toxic hot spots and the ranking matrix are presented.    The last section of the Cleanup Plan contains a characterization of the high priority candidate toxic hot spots and the preliminary assessment of actions to address the problems identified at the sites.

Region Description

The Los Angeles Region encompasses all coastal drainages flowing to the Pacific Ocean between Rincon Point (on the coast of western Ventura County) and the eastern Los Angeles County line, as well as the drainages of five coastal islands (Anacapa, San Nicolas, Santa Barbara, Santa Catalina and San Clemente).    In addition, the region includes all coastal waters within three miles of the continental and island coastlines.

The enclosed bays, estuaries and coastal waters of the Los Angeles Region subject to the provisions of the Bay Protection and Toxic Cleanup Program are listed in Table 1.    The region contains two large deepwater harbors (Los Angeles and Long Beach Harbors) and one smaller deepwater harbor (Port Hueneme).    There are small craft marinas within the harbors, as well as tank farms, naval facilities, fish processing plants, boatyards, and container terminals.    Several small-craft marinas also occur along the coast (e.g., Marina del Rey, King Harbor, Ventura Harbor);    these contain boatyards, other small businesses and dense residential development.

Several large, primarily concrete-lined rivers (e.g., Los Angeles River, San Gabriel River) lead to unlined tidal prisms which are influenced by marine waters.    Salinity may be greatly reduced following rains since these rivers drain large urban areas composed of mostly impermeable surfaces.    Some of these tidal prisms receive a considerable amount of freshwater throughout the year from publicly-owned treatment plants discharging tertiary-treated effluent.    Lagoons are located at the mouths of other rivers draining relatively undeveloped areas (e.g., Mugu Lagoon, Malibu Lagoon, Ventura River Estuary, Santa Clara River estuary).    There are also a few isolated coastal brackish water bodies receiving runoff from agricultural or residential areas.

Santa Monica Bay, which includes the Palos Verdes Shelf for the purposes of the Bay Protection and Toxic Cleanup Program, dominates a large portion of the open coastal waters in the region.    The region's coastal waters also include the areas along the shoreline of Ventura County and the waters surrounding the five offshore islands in the region.

TABLE 1.	LOS ANGELES REGION - WATERBODIES COVERED UNDER 		BAY PROTECTION AND TOXIC CLEANUP PROGRAM

| WATER BODY OR SEGMENT NAME | HYDROLOGIC UNIT | TOTAL AREAL EXTENT |
| --- | --- | --- |
| ESTUARIES |  |  |
| Ormond Beach Wetlands | 400.00 | 20 acres |
| Ventura River Estuary | 402.10 | 10 acres |
| Santa Clara River Estuary | 403.00 | 60 acres |
| Calleguas Creek Tidal Prism | 403.11 | 10 acres |
| McGrath Lake Estuary | 403.11 | 40 acres |
| Mugu Lagoon-East & West Arms | 403.11 | 1500 acres |
| Malibu Lagoon | 404.31 | 29 acres |
| Colorado Lagoon | 405.12 | 13 acres |
| Dominguez Channel Tidal Prism | 405.12 | 8 miles |
| Los Angeles River Tidal Prism/Queensway Bay | 405.12 | 3 miles |
| Los Cerritos Channel Tidal Prism/Wetland | 405.12 | 5 acres |
| Sim’s Pond | 405.12 | 1 acre |
| Ballona Wetlands | 405.13 | 150 acres |
| Venice Canals | 405.13 | 20 acres |
| San Gabriel River Tidal Prism | 405.15 | 3 miles |
| ENCLOSED BAYS |  |  |
| Channel Islands Harbor | 403.11 | 220 acres |
| Port Hueneme | 403.11 | 121 acres |
| Ventura Harbor | 403.11 | 423 acres |
| Alamitos Bay | 405.12 | 285 acres |
| King Harbor | 405.12 | 90 acres |
| Long Beach Harbor (Inner) | 405.12 | 840 acres |
| Long Beach Marina | 405.12 | 100 acres |
| Los Angeles Harbor (Inner) | 405.12 | 1,260 acres |
| San Pedro Bay | 405.12 | 10,700 acres |
| Shoreline Marina | 405.12 | 25 acres |
| Marina Del Rey Harbor | 405.13 | 354 acres |
| OPEN BAYS/OCEAN |  |  |
| Nearshore - Point Mugu to Latigo Point | 400.00 | 11,710 acres |
| Santa Monica Bay (L.A. County Line to Pt. Fermin) | 405.13 | 256,000 acres |
| Anacapa Island ASBS | 406.10 | 21,280 acres |
| San Nicolas Island/Begg Rock ASBS | 406.20 | 102,528 acres |
| Santa Barbara Island ASBS | 406.30 | 14,000 acres |
| Santa Catalina Island ASBS | 406.40 | 17,936 acres |
| San Clemente Island ASBS | 406.50 | 80,512 acres |

Legislative Authority

California Water Code, Division 7, Chapter 5.6 established a comprehensive program to protect the existing and future beneficial uses of California's enclosed bays and estuaries.    SB 475 (1989), SB 1845 (1990), AB 41 (1989), and SB 1084 (1993) added and modified Chapter 5.6 [Bay Protection and Toxic Cleanup (Water Code Sections 13390-13396.5)] to Division 7 of the Water Code.

The BPTCP has provided a new focus on RWQCBs efforts to control pollution of the State's bays and estuaries by establishing a program to identify toxic hot spots and plan for their cleanup.

Water Code Section 13394 requires that each RWQCB complete a toxic hot spot cleanup plan.    Each cleanup plan must include:    (1) a priority listing of all toxic hot spots covered by the plan; (2) a description of each toxic hot spot including a characterization of the pollutants present at the site; (3) an assessment of the most likely source or sources of pollutants; (4) an estimate of the total costs to implement the cleanup plan; (5) an estimate of the costs that can be recovered from parties responsible for the discharge of pollutants that have accumulated in sediments; (6) a preliminary assessment of the actions required to remedy or restore a toxic hot spot; and (7) a two-year expenditure schedule identifying State funds needed to implement the plan.

Limitations

This regional toxic hot spot cleanup plan contains information on sites that are believed to be the worst sites in the Region.    The candidate toxic hot spots identified in this Cleanup Plan are not considered known toxic hot spots until approved by the State Water Resources Control Board (SWRCB) in the consolidated toxic hot spot cleanup plan.    Many of the actions presented in this plan are general and may be specified in more detail as the actions are implemented through RWQCB actions.

II.	TOXIC HOT SPOT DEFINITION

Codified Definition of A Toxic Hot Spot

Section 13391.5 of the Water Code defines toxic hot spots as:

"...[L]ocations in enclosed bays, estuaries, or adjacent waters in the 'contiguous zone' or the 'ocean' as defined in Section 502 of the Clean Water Act (33. U.S.C. Section 1362), the pollution or contamination of which affects the interests of the State, and where hazardous substances have accumulated in the water or sediment to levels which (1) may pose a substantial present or potential hazard to aquatic life, wildlife, fisheries, or human health, or (2) may adversely affect the beneficial uses of the bay, estuary, or ocean waters as defined in the water quality control plans, or (3) exceeds adopted water quality or sediment quality objectives."

Specific Definition of A Toxic Hot Spot

The following specific definition provides a mechanism for identifying and distinguishing between "candidate" and "known"    toxic hot spots.    A candidate toxic hot spot is considered to have enough information to designate a site as a known toxic hot spot except that the candidate hot spot has not been approved by the RWQCB and the SWRCB.    Once a candidate toxic hot spot has been adopted into the consolidated statewide toxic hot spot cleanup plan then the site shall be considered a known toxic hot spot and all the requirements of the Water Code shall apply to that site.

Candidate and known toxic hot spots are locations (sites in waters of the State) in enclosed bays, estuaries or the ocean.    Dischargers (e.g., publicly owned treatment works, industrial facilities, power generating facilities, agricultural land, storm drains, etc.) are not toxic hot spots.

Candidate Toxic Hot SpotCandidate Toxic Hot Spot

A site meeting any one or more of the following conditions is considered to be a "candidate" toxic hot spot.

1.	The site exceeds water or sediment quality objectives for toxic pollutants that are contained in appropriate water quality control plans or exceeds water quality criteria promulgated by the U.S. Environmental Protection Agency (U.S. EPA).

This finding requires chemical measurement of water or sediment, or measurement of toxicity using tests and objectives stipulated in water quality control plans.    Determination of a toxic hot spot using this finding should rely on recurrent measures over time (at least two separate sampling dates).    Suitable time intervals between measurements must be determined.

2.	The water or sediment exhibits toxicity associated with toxic pollutants that is significantly different from the toxicity observed at reference sites (i.e., when compared to the lower confidence interval of the reference envelope, or, in the absence of a reference envelope, is significantly toxic as compared to controls (using a t-test) and the response is less than 90 percent of the minimum significant difference for each specific test organism), based on toxicity tests acceptable to the SWRCB or the RWQCBs.

To determine whether toxicity exists, recurrent measurements (at least two separate sampling dates) should demonstrate an effect.    Appropriate reference and control measures must be included in the toxicity testing.    The methods acceptable to and used by the BPTCP may include some toxicity test protocols not referenced in water quality control plans (e.g., the Bay Protection and Toxic Cleanup Program Quality Assurance Project Plan).    Toxic pollutants should be present in the media at concentrations sufficient to cause or contribute to toxic responses in order to satisfy this condition.

3.	The tissue toxic pollutant levels of organisms collected from the site exceed levels established by the United States Food and Drug Administration (FDA) for the protection of human health, or the National Academy of Sciences (NAS) for the protection of human health or wildlife.    When a health advisory against the consumption of edible resident non-migratory organisms has been issued by Office of Environmental Health Hazard Assessment (OEHHA) or Department of Health Services (DHS), on a site or water body, the site or water body is automatically classified a "candidate" toxic hot spot if the chemical contaminant is associated with sediment or water at the site or water body.

Acceptable tissue concentrations are measured either as muscle tissue (preferred) or whole body residues.    Residues in liver tissue alone are not considered a suitable measure for known toxic hot spot designation.    Animals can either be deployed (if a resident species) or collected from resident populations.    Recurrent measurements in tissue are required.    Residue levels established for one species for the protection of human health can be applied to any other consumable species.

Shellfish:    Except for existing information, each sampling episode should include a minimum of three replicates.    The value of interest is the average value of the three replicates.    Each replicate should be comprised of at least 15 individuals.    For existing State Mussel Watch information related to organic pollutants, a single composite sample (20-100 individuals), may be used instead of the replicate measures.    When recurrent measurements exceed one of the levels referred to above, the site is considered a candidate toxic hot spot.

Fin-fish:    A minimum of three replicates is necessary.    The number of individuals needed will depend on the size and availability of the animals collected; although a minimum of five animals per replicate is recommended.    The value of interest is the average of the three replicates.    Animals of similar age and reproductive stage should be used.

4.	Impairment measured in the environment is associated with toxic pollutants found in resident individuals.

Impairment means reduction in growth, reduction in reproductive capacity, abnormal development, histopathological abnormalities.    Each of these measures must be made in comparison to a reference condition where the endpoint is measured in the same species and tissue is collected from an unpolluted reference site.    Each of the tests shall be acceptable to the SWRCB or the RWQCBs.

Growth Measures:    Reductions in growth can be addressed using a suitable bioassay acceptable to the State or Regional Boards or through measurements of field populations.

Reproductive Measures:    Reproductive measures must clearly indicate reductions in viability of eggs or offspring, or reductions in fecundity.    Suitable measures include:    pollutant concentrations in tissue, sediment, or water which have been demonstrated in laboratory tests to cause reproductive impairment, or significant differences in viability or development of eggs between reference and test sites.

Abnormal Development:    Abnormal development can be determined using measures of physical or behavioral disorders or aberrations.    Evidence that the disorder can be caused by toxic pollutants, in whole or in part, must be available.

Histopathology:    Abnormalities representing distinct adverse effects, such as carcinomas or tissue necrosis, must be evident.    Evidence that toxic pollutants are capable of causing or contributing to the disease condition must also be available.

5.	Significant degradation in biological populations and/or communities associated with the presence of elevated levels of toxic pollutants.

This condition requires that the diminished numbers of species or individuals of a single species (when compared to a reference site) are associated with concentrations of toxic pollutants.    The analysis should rely on measurements from multiple stations.    Care should be taken to ensure that at least one site is not degraded so that a suitable comparison can be made.

Known Toxic Hot SpotKnown Toxic Hot Spot

A site meeting any one or more of the conditions necessary for the designation of a "candidate" toxic hot spot that has gone through a full SWRCB and RWQCB hearing process, is considered to be a "known" toxic hot spot.    A site will be considered a "candidate" toxic hot spot until approved as a known toxic hot spot in a Regional Toxic Hot Spot Cleanup Plan by the RWQCB and approved by the SWRCB.

III.	MONITORING APPROACH

As part of the legislative mandates, the BPTCP has implemented regional monitoring programs to identify toxic hot spots (Water Code Section 13392.5).    The BPTCP has pioneered the use of effects-based measurements of impacts in California's enclosed bays and estuaries.    The Program has used a two-step process to identify toxic hot spots.    The first step is to screen sites using toxicity tests or chemical tests or benthic community analysis.    In the second step, the highest priority sites with observed toxicity are retested to confirm the effects.    This section presents descriptions of the BPTCP monitoring objectives and sampling strategy.

Monitoring Program ObjectivesMonitoring Program Objectives

The four objectives of BPTCP regional monitoring are:

1.	Identify locations in enclosed bays, estuaries, or the ocean that are potential or candidate toxic hot spots.    Potential toxic hot spots are defined as suspect sites with existing information indicating possible impairment but without sufficient information to be classified further as a candidate toxic hot spot.

2.	Determine the extent of biological impacts in portions of enclosed bays and estuaries not previously sampled (areas of unknown condition).

3.	Confirm the extent of biological impacts in enclosed bays and estuaries that have been previously sampled.

4.	Assess the relationship between toxic pollutants and biological effects.

Sampling StrategySampling Strategy

Screening Sites and Confirming Toxic Hot Spots Screening Sites and Confirming Toxic Hot Spots

In order to identify toxic hot spots a two step process was used.    Both steps are designed around an approach with three measures (sediment quality triad analysis) plus an optional bioaccumulation component.    The triad analysis consists of toxicity testing, benthic community analysis, and chemical analysis for metals and organic chemicals.

The first step is a screening phase that consists of measurements using toxicity tests or benthic community analysis or chemical tests or bioaccumulation data to provide sufficient information to list a site as a potential toxic hot spot or a site of concern.    Sediment grain size, total organic carbon (TOC), NH3 and H2S concentration are measured to differentiate pollutant effects found in screening tests from natural factors.

A positive result or an effect in any of the triad tests would trigger the confirmation step (depending on available funding).    The confirmation phase consists of performing all components of the sediment quality triad:    toxicity, benthic community analysis, and chemical analysis, on the previously sampled site of concern.    Assessment of benthic community structure may have not be completed if there was difficulty in measuring or interpreting the information for a water body.

Special Studies Performed in the Region

1) Los Angeles/Long Beach Harbor Study

In 1992, the State Water Resources Control Board and the National Oceanic and Atmospheric Administration entered into a three-year cooperative agreement to assess the potential adverse biological effects in several coastal bays and harbors in Southern California.    This study was performed in San Pedro Bay, Los Angeles Harbor, Long Beach Harbor, Anaheim Bay, Alamitos Bay and Huntington Harbor.    The objectives of the study were to:    1) characterize the magnitude and relative spatial extent of toxicant-associated bioeffects in these nearshore areas;    2) determine relationships between concentrations and mixtures of sediment-associated toxicants and the occurrence and severity of bioeffects;    and 3) distinguish more severely impacted sediments from less severely impacted sediments.    Samples were collected on several occasions:    July 30, 1992 (Leg 1), August 19, 1992 (Leg 2), September 10 and 15, 1992 (Legs 3 and 4), May 7 and 28, 1993 (Legs 18 and 19), June 18, 1993 (Leg 20), August 3-5 and 17-19, 1993 (Legs 22 and 23), February 3 and 14, 1994 (Legs 25 and 26), April 18, 1994 (Leg 30) and May 22, 1994 (Leg 32).

Amphipod survival and abalone larval development toxicity tests were performed on sediment and pore water samples.    Significant amphipod mortality compared to laboratory controls was observed at the majority of sites in the Los Angeles and Long Beach inner harbor areas.    Most of the outer harbor site sediments were not toxic to amphipods.    Several chemicals (e.g., acenanaphthene, phenanthrene, fluoranthene, copper, lead, zinc) or chemical groups (e.g., total PAHs) were significantly correlated with amphipod survival.

2) Biomarker Study

This study was conducted in 1993 as part of the Bay Protection and Toxic Cleanup Program.    The project was designed to assess the impact of sediment contaminants on fish, with special emphasis on evaluation of bay gobies (Lepidogobius lepidus) as a potential indicator species for the California coast.    Unfortunately, bay gobies could not be collected and other species were substituted.    The study area included nine sites in the Los Angeles Harbor area.    Sediments were analyzed for metals, pesticides, polynuclear aromatic hydrocarbons (PAHs) and polychlorinated biphenyls (PCBs).    In addition, pore water was tested for metals and sediments were assayed for toxicity to amphipods.    Although we were unable to identify a single "indicator" species which could be used throughout California coastal waters, all five fish species examined did have lesions which appeared to be consistent with contaminant exposure.    Additional studies would be needed to validate the technique.

3)    BPTCP Monitoring

Under this program, monitoring studies were performed throughout the Los Angeles Region over several years.    The results are summarized in a report entitled “Sediment Chemistry, Toxicity, and Benthic Community Conditions in Selected Water Bodies of the Los Angeles Region”, produced in August 1998 in collaboration between the California Department of Fish and Game, Los Angeles Regional Water Quality Control Board and State Water Resources Control Board.    The following paragraphs briefly describe the monitoring programs and the major findings.

On January 15, 1993 (Leg 11), several northern Los Angeles County stations were screened for sediment toxicity.    On February 10, 1993 (Leg 13), Ventura River Lagoon sites were sampled.    Screening of Mugu Lagoon and McGrath Lake was conducted on April 18, 1994 (Leg 30).    Sampling for Mugu Lagoon, McGrath Lake, Port Hueneme, Shoreline Marina and Marina del Rey was performed on June 24, 1996 (Leg 45).    On July 22, 1996, the areal extent of the contamination problem in Consolidated Slip was investigated (Leg 46).    On February 10, 1997, samples were collected in Los Angeles/Long Beach Harbor, Marina del Rey, Shoreline Marina and King Harbor (Leg 48).    The May 12, 1997, sampling focused on Los Angeles/Long Beach Harbor and the Cabrillo Pier area (Leg 53).    The Kaiser Pier area was investigated on August 22, 1997 (Leg 54).    The location of all of the sampling stations is shown on Figures 1-8.

Copper, mercury, zinc, tributyltin, total chlordane, total PCBs and high molecular weight PAHs were found to be the chemicals or chemical groups of greatest concern at the Industrial Harbor stations.    Of the 192 Industrial Harbor stations where toxicity was assessed, 29% showed significant toxicity to amphipods.    Toxicity was highest in the inner Industrial Harbor sediment samples.    Of the 102 Industrial Harbor stations where benthic community structure was characterized, 13% were considered to be degraded based upon a Relative Benthic Index.    Bioaccumulation measurements in fish collected from the West Basin and Cabrillo Pier areas of Los Angeles Harbor indicated DDT and PCB tissue concentrations that exceeded EPA screening values.    Bivalves exposed to sediment from the Cabrillo Pier area accumulated significantly higher concentrations of DDT and PCB than clams exposed to control sediment.

Copper, lead, mercury, zinc, total chlordane and total PCBs were found to be the chemicals or chemical groups of greatest concern at the Marina stations.    Of the 35 Marina stations tested for amphipod toxicity, sediment samples were toxic to amphipods at 31% of the stations.    Only 1 of 22 Marina stations was considered to have a degraded benthic community.

PCBs and several pesticides, including chlordane, dieldrin, endosulfan and DDT, were the chemicals of greatest concern in the Lagoon stations.    Of the 33 Lagoon stations where toxicity was assessed, 58% were toxic to amphipods.    All 6 of the Lagoon samples assessed (all located within Mugu Lagoon) had a degraded benthic community.    Fish collected from one station in Mugu Lagoon had elevated levels of PCBs compared to EPA screening values.

Figure 1.    Sampling Stations in Outer Los Angeles and Long Beach Harbor, Port Hueneme and Palos Verdes.

Figure 2.    Sampling Stations in Inner Los Angeles and Long Beach Harbor and Consolidated Slip.

Figure 3.    Sampling Stations in Shoreline Marina and Los Alamitos Bay.

Figure 4.    Sampling Stations in King Harbor and Marina Del Rey.

Figure 5.    Sampling Stations in Ventura Marina and Channel Islands Harbor.

Figure 6.    Sampling Stations in Mugu Lagoon.

Figure 7.    Sampling Stations in McGrath Lake, Ballona Creek and Colorado Lagoon/Sim's Pond.

Figure 8.    Sampling Stations in Ventura River Estuary, Santa Clara River Estuary and Malibu Lagoon.

IV.	CRITERIA FOR RANKING TOXIC HOT SPOTS

A value for each criterion described below shall be developed provided    appropriate information exists or estimates can be made.    Any criterion for which no information exists shall be assigned a value of “No Action”.    The RWQCB shall create a matrix of the scores of the ranking criteria.    The RWQCBs shall determine which sites are “High” priority based on the five general criteria (below) keeping in mind the value of the water body.    The RWQCBs shall provide a justification or reason a ranking was assigned if the value is an estimate based on best professional judgement.

Human Health Impacts

If a Human Health Advisory has been issued for consumption of non-migratory aquatic life from the site, assign a “High” priority. If tissue residues in aquatic organisms exceed FDA/DHS action level and U.S. EPA screening levels, assign a “Moderate” priority.

Aquatic Life Impacts

For aquatic life, site ranking shall be based on an analysis of substantial information available.    The measures that shall be considered are:    sediment chemistry, sediment toxicity, biological field assessments    (including benthic community analysis), water toxicity, toxicity identification evaluations (TIEs), and bioaccumulation.

Stations with hits in any two of the biological measures if associated with high chemistry, assign a “High” priority.    A hit in one of the measures associated with high chemistry is assigned “moderate”, and high sediment or water chemistry only shall be assigned “low”.    In analyzing the substantial information available, RWQCBs should take into consideration that impacts related to biological field assessments (including benthic community structure) are of more importance than other measures of impact.

Water Quality Objectives

Any chemistry data used for ranking under this section shall be no more than 10 years old, and shall have been analyzed with appropriate analytical methods and quality assurance.

Water quality objective or water quality criterion:    Exceeded regularly (assign a “High” priority), occasionally exceeded (“Moderate”), infrequently exceeded (“Low”).    [Water quality objectives to be used are found in the Los Angeles Regional Water Quality Control Board’s Basin Plan or the California Ocean Plan (depending on which plan applies to the water body being addressed).    Where the Basin Plan contains a more stringent value than the statewide plan, the regional water quality objective will be used.]

Areal Extent of Toxic Hot Spot

Select one of the following values:    More than 10 acres, 1 to 10 acres,    less than 1 acre.

Natural Remediation Potential

Select one of the following values:    Site is unlikely to improve without intervention (“High”), site may or may not improve without intervention (“Moderate”), site is likely to improve without intervention (“Low”).

V.	FUTURE NEEDS

Additional monitoring should be conducted at sites of concern to determine whether such sites meet the criteria for designation as candidate toxic hot spots in the future.    Monitoring of candidate toxic hot spots also will be required to determine whether remediation efforts are successful in eliminating the hot spots or whether conditions improve without any directed remediation efforts.

Sites of Concern (Sites that do not qualify as Candidate Toxic Hot Spots)

| Water body name | Segment Name |  | Site Identification |  | Reason for Listing |  | Pollutants present at the site |  | Report reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Los Angeles Inner Harbor | Inner Fish Harbor | 44019.0, 40019.2, 40019.3 |  | sediment concentrations; sediment toxicity |  | DDT, PCB, metals (Cu, Hg, Zn) |  | [1] |  |
| Los Angeles Inner Harbor | Kaiser International | 49004.0 |  | sediment concentrations |  | DDT, PCB, PAH, Cu, Endosulfan |  | [1] |  |
| Los Angeles Inner Harbor | Hugo Neu Proler | 46001.0, 46002.0 |  | sediment concentrations |  | PCB |  | [1] |  |
| Los Angeles Inner Harbor | Southwest Slip | 40001.2, 40001.3 |  | sediment concentrations; sediment toxicity |  | DDT, PCB, PAH, metals (Hg, Cr), Benz[a]anthracene, Benzo[a]pyrene, Dibenz[a,h] anthracene |  | [1] |  |
| Long Beach Inner Harbor | Cerritos Channel | 44011.0 |  | sediment concentrations; sediment toxicity; accumulation in mussel tissue |  | DDT, PCB, metals, Chlordane, TBT |  | [1], [2] |  |
| Colorado Lagoon |  | 44017.0 |  | sediment concentrations; sediment toxicity; accumulation in mussel and fish tissue |  | DDT, PCB, metals (Pb, Zn), Chlordane, Dieldrin |  | [1], [2], [3] DFG |  |
| Shoreline Marina |  | 44020.0, 48006.0, 48008.0 |  | sediment concentrations; sediment toxicity |  | Zn, DDT, PCB, Chlordane, Phenanthrene |  | [1], [3] |  |
| McGrath Lake |  | 44024.0, 44027.0 |  | sediment concentrations; sediment toxicity |  | DDT, Chlordane, Dieldrin, Toxaphene, Endosulfan |  | [1] |  |

Sites of Concern (Sites that do not qualify as Candidate Toxic Hot Spots)

| Water body name | Segment Name | Site Identification | Reason for Listing |  | Pollutants present at the site | Report reference |
| --- | --- | --- | --- | --- | --- | --- |
| Port Hueneme |  | 44012.0, 44013.0 | sediment concentrations; sediment toxicity; accumulation in mussel tissue; | DDT, PCB, PAH, metals (Zn, Cr), Benz[a]anthracene; Benzo[a]pyrene; Dibenz[a,h] anthracene |  | [1], [2] |
| Long Beach Outer Harbor |  | 44018.1, 44018.2, 44018.3, 44020.1, 44020.2, 44020.3 | sediment concentrations; sediment toxicity | DDT, Chlordane |  | [1] |
| Long Beach Inner Harbor | West Basin | 40009.0, 40009.1, 40009.2, 40009.3 | sediment concentrations; sediment toxicity; accumulation in clam tissue | DDT, PCB |  | [1] |
| Alamitos Bay |  | 40021.1, 40021.2, 40021.3, 40022.1, 40022.2, 40022.3, 40023.1, 40023.2, 40023.3 | sediment concentrations | DDT, Chlordane |  | [1] |
| King Harbor |  | 48011.0 | sediment concentrations; sediment toxicity | DDT, PCB |  | [1] |
| Channel Islands Harbor |  | 44023.0 | sediment concentrations; sediment toxicity | DDT, Ag |  | [1] |

Reference list

[1]          Anderson, B. et al.    1998.    Sediment chemistry, toxicity and benthic community conditions in selected water bodies of the Los Angeles Region.    Final Report to State Water Resources Control Board, Bay Protection and Toxic Cleanup Program Monitoring.

[2]          State Mussel Watch Program, Los Angeles Region

[3]          Water Quality Assessment [303(d) List], Los Angeles Region, December 1995

Several sites have been listed in the table above as “Sites of Concern”.    These are sites that displayed signs of sediment contamination problems, primarily based upon data collected as part of the Bay Protection and Toxic Cleanup Program, but did not meet the criteria for designation as “Candidate Toxic Hot Spots”.    Although designation as a “Site of Concern” does not trigger any specific action under the Bay Protection and Toxic Hot Spot Program, these sites have been identified by the Regional Board so that they can be targeted for additional monitoring as funding becomes available.

Several of the Sites of Concern could not be designated as “Candidate Toxic Hot Spots” due to the lack of recurrent toxicity;    in some cases, the sites were only sampled on one occasion, while in other cases, toxicity was observed on only one of the sampling events.    Inner Fish Harbor, Southwest Slip, Cerritos Channel, Colorado Lagoon, Shoreline Marina, McGrath Lake, Port Hueneme, Long Beach Outer Harbor, West Basin, King Harbor and Channel Islands Harbor all fall into this category.

Hugo Neu Proler, Kaiser International, Alamitos Bay are listed as sites of concern due to sediment contamination, but sediment toxicity was not observed on any occasion.

Part II

Candidate Toxic Hot Spot List

| Water body name | Segment Name | Site Identification | Reason for Listing | Pollutants present at the site | Report reference |
| --- | --- | --- | --- | --- | --- |
| Santa Monica Bay | Palos Verdes Shelf | BPTCP 40031.1, 40031.2, 40031.3 | Human health advisory; NAS level exceeded for DDT; sediment concentrations + sediment toxicity; degraded benthic community | DDT, PCB | [1], [2], [3], [4] |
| Mugu Lagoon/ Calleguas Creek Tidal Prism | Eastern Arm, Main Lagoon, Western Arm/ Tidal Prism | BPTCP 44050.0, 44052.0, 44053.0, 44054.0; 44016.0, 48013.0, 48014.0, 48015.0, 48016.0, 48017.0, 48018.0, SMW 507.8; TSM 403.11.04, 403.12.06 | Reproductive impairment; OEHHA level exceeded for Hg; NAS level exceeded for DDT; sediment concentrations + sediment toxicity; degraded benthic community | DDT, PCB, metals, Chlordane, Chlorpyrifos | [4], [5], [6], [7] |
| Los Angeles Inner Harbor | Dominguez Channel/ Consolidated Slip | BPTCP 40006.1, 40006.2 | Human health advisory; NAS level exceeded for DDT; sediment concentrations + sediment toxicity; degraded benthic community | DDT, PCB, PAH, metals (Cd, Cu, Pb, Hg, Zn), dieldrin, chlordane | [4], [8], [9], [10] |
| Los Angeles Outer Harbor | Cabrillo Pier | BPTCP 40010.1, 40010.2, 40010.3 | Human health advisory; NAS level exceeded for DDT; sediment concentrations + sediment toxicity | DDT, PCB, Cu | [2], [4], [10] |
| Los Angeles River | Estuary | BPTCP 40013.1 | Sediment concentrations + sediment toxicity | DDT, PAH, Chlordane | [4] |
| Ballona Creek | Entrance Channel | BPTCP 44024.0, COE | Sediment concentrations + sediment toxicity | DDT, metals (Zn, Pb), Chlordane, Dieldrin, Chlorpyrifos | [4] |
| Marina del Rey |  | BPTCP 44014.0, 48001.0, 48002.0, 48003.0, 48004.0, 48005.0 | Sediment concentrations + sediment toxicity | DDT, PCB, Metals (Cu, Hg, Ni, Pb, Zn), Chlordane | [4] |

Reference list

[1]	Los Angeles Regional Water Quality Control Board.    Santa Monica Bay:    State of the Watershed.    June, 1997.

[2]	California Department of Fish and Game.    1996 California Sport Fishing Regulations.

[3]	Santa Monica Bay Restoration Project, Comprehensive Conservation and Management Plan, 1996.

[4]	Anderson, B. et al.    1998.    Sediment chemistry, toxicity and benthic community conditions in selected water bodies of the Los Angeles Region.    Final Report to State Water Resources Control Board, Bay Protection and Toxic Cleanup Program Monitoring.

[5]	Ledig, D.    Preliminary report on the ecology of the light-footed clapper rail at Mugu Lagoon, Ventura Co., California.

[6]	Final report to California Department of Fish and Game FG 8555, 1990.

[7]	Los Angeles Regional Water Quality Control Board.    Calleguas Creek Watershed.    June, 1996.

[8]	Malins, D.C. et al.    Toxic chemicals, including aromatic and chlorinated hydrocarbons and their derivatives, and liver lesions in white croaker (Genyonemus lineatus) from the vicinity of Los Angeles.    Environ. Sci. and Tech., August 1987, pp. 765-770.

[9]	MEC Analytical Systems, Inc.    Biological baseline and ecological evaluation of existing habitats in Los Angeles Harbor and adjacent waters, Vol. II, Final Report.    Page 4-74.    Sept. 1988.    Prepared for Port of Los Angeles, Environmental Management Division.

[10]	Cross, J.N. et al.    Contaminant concentrations and toxicity of sea-surface microlayer near Los Angeles, California.    Mar. Environ. Research:    23 (1987) 307-323.

Ranking Matrix

| Waterbody Name | Site Identification | Human Health Impacts | Aquatic Life Impacts | Water Quality Objectives | Areal Extent | Remediation Potential | Overall Ranking |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Santa Monica Bay | Palos Verdes Shelf | High | High | Low | > 10 acres | High | High |
| Mugu Lagoon | Eastern Arm, Main Lagoon, Western Arm/ Tidal Prism | Moderate | High | Low | > 10 acres | High | High |
| Los Angeles Inner Harbor | Dominguez Channel/ Consolidated Slip | High | High | Low | 1 - 10 acres | High | High |
| Los Angeles Outer Harbor | Cabrillo Pier | High | Moderate | Low | 1 - 10 acres | High | High |
| Los Angeles River | Estuary | Moderate | Moderate | Low | 1-10 acres | High | Moderate |
| Marina Del Rey | Entrance Channel | Low | Moderate | Low | < 1 acre | High | Moderate |
| Marina del Rey |  | Moderate | Moderate | Low | 1-10 acres | High | Moderate |

# Part III

High Priority Candidate Toxic Hot Spot Characterization

This section of the cleanup plan contains a characterization of the four high priority candidate toxic hot spots identified (Santa Monica Bay/Palos Verdes Shelf, Mugu Lagoon/Calleguas Creek Tidal Prism, Los Angeles Outer Harbor/Cabrillo Pier, Los Angeles Inner Harbor/Dominguez Channel, Consolidated Slip).    This section also contains a preliminary assessment of actions to address the problems identified at these sites.

| Candidate Toxic Hot Spot | Areal Extent | Estimated Remediation Cost |
| --- | --- | --- |
| Palos Verdes Shelf | 9 million cubic meters | $13 - 67 million |
| Mugu Lagoon Calleguas Creek Tidal Prism | 725,000 cubic yards 50,000 - 100,000 cubic yards | $72.5 million $1-5 million |
| Cabrillo Pier | 25,000 - 50,000 cubic yards | $0.5 - 50 million |
| Consolidated Slip | 50,000 cubic yards | $1 - 50 million |

Three areas were designated as moderate priority candidate toxic hot spots (Los Angeles River Estuary, Marina del Rey Entrance Channel, Marina del Rey), based on lower ratings for human health impacts and aquatic life impacts (refer to ranking matrix).    The Cleanup Plan is not required to contain a detailed characterization report and preliminary assessment of remediation actions for “moderate” candidate toxic hot spots.    However, these needs would be addressed in the future after remediation plans have been initiated at the high priority sites.

1.    Santa Monica Bay/Palos Verdes Shelf

The contaminated sediments on the Palos Verdes Shelf    appear to significantly impact the marine community and may pose a serious risk to individuals who regularly consume fish from the area.    Currently, elevated levels of DDT and PCBs are found in the organisms that live in the area of the contaminated sediments, including bottom feeding fish such as white croaker, and water column feeders such as kelp bass.    Marine mammals and birds could be affected through the consumption of contaminated fish [Draft Ecological Risk Evaluation Report for the Palos Verdes Shelf, U.S. Environmental Protection Agency, September 1998].

The ongoing release of these hazardous substances from the sediment into the environment and the resulting accumulation of DDT and PCB in food chain organisms may persist if no action is taken.    Commercial fishing and recreational fishing have been affected by the contamination.    The State of California has issued a health advisory warning against the consumption of white croaker and kelp bass and closed commercial fishing for white croaker on the Palos Verdes Shelf.

A)    Areal Extent of Toxic Hot Spot

In July 1996, the United States Environmental Protection Agency initiated a response action under Superfund site and began an evaluation to address the large deposit of DDT and PCB contaminated sediments on the Palos Verdes Shelf.    The contaminated sediment footprint identified as the study area for this evaluation was defined as the boundary for one part-per-million (mg/kg) sediment DDT concentration described by the United States Geological Survey (USGS), covering portions of the continental shelf and continental slope between Point Vicente in the northwest and Point Fermin to the southeast (Figure 9).    This entire area is proposed as a candidate know toxic hot spot.

Studies by the USGS in 1992 and 1993 indicated that this layer of contaminated sediments is about two inches to two feet thick and covers an area of more than 15 square miles, with the highest concentrations located in a 3-square mile band near the outfall pipes.    The total volume of contaminated sediments on the Palos Verdes Shelf is approximately 9 million cubic meters and covers a surface area of approximately 40 square kilometers, with approximately 70% of this volume present on the continental slope in water depths less than 100 meters.    The total mass of p,p’-DDE in the contaminated sediments is estimated to be greater than 67 metric tons.

In addition to the large volume of monitoring data evaluated as part of the Superfund evaluation, limited sampling was conducted as part of the Bay Protection and Toxic Cleanup Program.    BPTCP monitoring data showed that on September 10, 1992, sediment concentrations at stations 40031.1, 40031.2 and 40031.3 exceeded the ERM thresholds for Total DDT and Total PCB.    Samples collected on August 17-19, 1993, and February 3. 1994, at station 40031.2 (Replicates 1, 2 and 3) also exceeded the ERM thresholds for Total DDT and Total PCB.    Amphipod toxicity was recorded with whole sediments at station 40031.2 on February 3, 1994.    Porewater toxicity to abalone was recorded at station 40031.2 on September 10, 1992.    A degraded benthic community was observed at station 40031.2 on August 17-19, 1993.

Palos Verdes Shelf

BPTCP Stations With Sediment Chemistry Concentrations Exceeding ERM Threshold

| BPTCP Station | Sampling Date | Total DDT (ppb) | Total PCB (ppb) |
| --- | --- | --- | --- |
| 40031.1 | 9/10/92 | 2729.3 | 268.7 |
| 40031.2 | 9/10/92 | 3337.5 | 271.3 |
| 40031.3 | 9/10/92 | 2520.7 | 204.1 |
| 40031.2 Rep 1 | 8/17-19/93 | 2525.8 | 259.5 |
| 40031.2 Rep 2 | 8/17-19/93 | 3569.3 | 301.4 |
| 40031.2 Rep 3 | 8/17-19/93 | 2604.6 | 302.5 |
| 40031.2 Rep 1 | 2/3/94 | 3344.2 | 271.3 |
| 40031.2 Rep 2 | 2/3/94 | 3331.5 | 312.8 |
| 40031.2 Rep 3 | 2/3/94 | 2063.9 | 221.8 |

Figure 9.    Areal extent of toxic hot spot on the Palos Verdes Shelf.

## B)    Sources of Pollutants

From 1947 to 1983, the Montrose Chemical Corporation of California, Inc., manufactured the pesticide dichloro-diphenyl-trichloroethane (DDT) at its plant at 20201 Normandie Avenue in Los Angeles.    Wastewater containing significant concentrations of DDT was discharged from the Montrose plant into the sewers, flowed through the Los Angeles County Sanitation Districts' wastewater treatment plant and was discharged to the Pacific Ocean waters on the Palos Verdes Shelf through subsurface outfalls offshore of Whites Point.    Montrose's discharge of DDT stopped around 1972, and the plant was shut down and dismantled in 1983.

Polychlorinated biphenyls (PCBs) also were present in the wastewater discharged from the LACSD wastewater treatment plant and are found along with DDT in the effluent-affected deposits on the ocean floor along the Palos Verdes Shelf.    Historically, PCB contamination entered the sewer system as the result of discharges from several industrial sources.

Although DDT and PCBs were banned in the early 1970s, release of contaminants from historically deposited sediments continues to be a source of these toxic chemicals.    Concentrations of total DDT and p,p'-DDE (the predominant metabolite of DDT) in the surface sediments have remained relatively high since the late 1980s.    This suggests that historical deposits are brought to the sea floor surface by a combination of natural physical, chemical or biological processes.

Besides DDT and PCB, there has been little evidence that the concentrations of    other toxic organic compounds, such as PAHs and heavy metals (including copper, cadmium, chromium, nickel, silver, zinc and lead), discharged from the LACSD wastewater treatment plant have caused impacts to marine organisms.    However, the concentrations of heavy metals in the sediments on the Palos Verdes Shelf are significantly higher than the background levels found in most parts of Santa Monica Bay and other parts of the Southern California Bight.

C)    Actions by Regional Board

The Los Angeles Regional Board's Water Quality Assessment identifies the Palos Verdes Shelf as an impaired waterbody.    The aquatic life beneficial use was listed as impaired due to sediment toxicity, tissue bioaccumulation of pollutants (DDT, PCBs, silver, chromium, lead), sediment contamination (DDT, PCBs, cadmium, copper, lead, mercury, nickel, zinc, PAHs, chlordane), and a health advisory warning against consumption of fish (white croaker).    The Regional Board believes that the impairment is due to the effects of historical discharges of these pollutants, since the concentrations presently discharged are very low.

The Santa Monica Bay Restoration Project (SMBRP) was formed in 1988 under the National Estuary Program in response to the critical problems facing Santa Monica Bay.    The Los Angeles Regional Board has been an active participant in this program.    The SMBRP was charged with the responsibility for assessing the Bay's problems, developing solutions and putting them into action.    The scientific characterization of the Bay is described in the SMBRP's "State of the Bay, 1993" report and other technical investigations.    This report, along with the Project's recommendations for action, comprises the Bay Restoration Plan which was approved in 1995.    With over 200 recommended actions (74 identified as priorities), the plan addresses the need for pollution prevention, public health protection, habitat restoration and comprehensive resource management throughout Santa Monica Bay, including the Palos Verdes Shelf area. The Los Angeles Regional Board is the lead agency responsible for implementation of    several recommended actions.

The Los Angeles Regional Board has adopted a watershed management approach, which is expected to regulate pollutant loads from point sources through permits that better focus on issues relevant to each watershed.    The Regional Board also expects that pollutant loads from nonpoint sources can be better controlled through the participation of the public in the management of their watersheds.    During the 1996-97 Fiscal Year, the watershed management approach was used to renew selected NPDES permits within the Santa Monica Bay Watershed.    The NPDES permit for the Los Angeles County Sanitation Districts’ Joint Water Pollution Control Plant, which discharges a mixture of advanced primary and secondary effluent through an ocean outfall onto the Palos Verdes Shelf, was renewed with appropriate limits, performance goals and mass emission caps to limit the discharge of pollutants of concern.

D) Preliminary Assessment of Remediation Actions

In July 1996, the U.S. Environmental Protection Agency decided to undertake a Superfund response (under the Comprehensive Environmental Response, Compensation and Liability Act) called a removal action to address the contaminated sediment problem on the Palos Verdes Shelf.    EPA initiated the preparation of an Engineering Evaluation/Cost Analysis (EE/CA) of possible response actions.    The EE/CA will evaluate the need for Superfund action and will use the three broad criteria of effectiveness, implementability and cost to evaluate the alternatives for addressing hazardous substances being released into the environment.

As an initial step in the EE/CA process, EPA has prepared the "Screening Evaluation of Response Actions for Contaminated Sediments on the Palos Verdes Shelf".    The Screening Evaluation describes the range of potential cleanup and disposal technologies for contaminated sediments and makes an initial determination about which technologies will be incorporated into the alternatives evaluated in detail in the EE/CA.    General response actions which were evaluated included:

-removal (i.e., dredging) and treatment or disposal;

-institutional controls;    and

-in situ (or in-place) capping;

-no action..

While sediment removal (i.e., dredging) is technically feasible, it could possibly result in the dispersal of contaminated sediment, thereby increasing short-term risks.    Once dredged, the sediment would require disposal, possibly preceded by treatment, which could be both expensive and very difficult to implement.    Upland disposal facilities are very limited, and disposal options along the coastline or in the open ocean would likely violate Federal and State environmental laws.    For these reasons, EPA has decided not to consider dredging and treatment or disposal options further in the EE/CA.

Institutional control measures, such as warning notices or fishing restrictions, intended to protect human health already have been established for certain coastal areas including the Palos Verdes Shelf by the State of California, although their effectiveness is uncertain.    Additional institutional controls could include measures to 1) expand the scope of existing State controls by increasing the area affected;    2) increase the awareness of and effectiveness of existing controls through additional public outreach efforts;    and 3) enhance State enforcement of the commercial fishing closure.

In situ, or in-place, capping can be used to prevent or reduce direct human or ecological exposure to contaminants and to prevent migration of contaminants into the water.    The cap could reduce or eliminate adverse impacts through 1) physical isolation of the contaminated sediment from the benthic environment, reducing the exposure of organisms to contaminants and limiting the potential for bioaccumulation and movement of contaminants into the food chain;    2) physical stabilization of the contaminated layer to retard resuspension and transport of contaminated sediment;    and 3) reducing the flux of dissolved contaminants from the sediments into the water column (e.g., due to waves and currents).    Large caps for areas like the Palos Verdes Shelf typically would consist of clean dredged material (i.e., sand or silt) that is placed over the contaminated area using dredge or platform barges.    Caps can be constructed to various sizes or thicknesses and may be augmented after initial construction to increase effectiveness.    For a large site like the Palos Verdes Shelf, a phased approach to capping would likely be desirable in order to maximize cost-effectiveness.    Any cap design would need to consider the engineering characteristics of the cap material and the effluent-affected sediment in order to address potential erosion by currents and waves, mixing of the cap material and underlying sediment by bottom-dwelling organisms or other disturbances.

In situ capping has the potential to isolate the contaminated marine sediments, thereby providing long-term protection for the majority of the mass of contaminants on the Palos Verdes Shelf.    Approximately 25% of the mass of contaminants is on the Palos Verdes slope, which is likely to be too steep for capping.    Over the short term, capping would have some adverse impact on the existing benthic communities in the capped area, although it is expected that they would rapidly recolonize.    If the cap were composed of suitable dredged material generated by local navigation projects (e.g., maintenance dredging), there would be no additional excavation beyond that already required for those projects, and reuse of the material for capping would reduce short-term impacts at traditional disposal sites.    Carefully controlled placement of the cap material would minimize the resuspension of contaminated sediment.

In situ caps have been used successfully at numerous sites, although not as deep as the deeper parts of the Palos Verdes Shelf.    In general, existing caps have stabilized after initial reworking and consolidation of the contaminated sediment.    Capping could be accomplished reasonably quickly, depending on the availability of capping material.

A draft report (September 1998) prepared by the United States Army Corps of Engineers for EPA evaluates “Options for In-Situ Capping of Palos Verdes Shelf Contaminated Sediment”.    The report considers two options:    1) capping an area of approximately 4.9 square kilometers centered over the area with the highest DDT contamination;    2) capping a secondary area of contamination comprising approximately 2.7 square kilometers located northwest of the first area.    Bioturbation, consolidation and cap effectiveness evaluations indicated that a thickness of 15 centimeters would be appropriate for a thin capping approach, designed to isolate contaminated material from shallow burrowing benthic organisms, while a 45 centimeter cap would be adequate for a thick cap design, effectively isolating the contaminated material from benthic organisms.    Capping both areas with a thick cap (45 cm) would result in a reduction of potential exposures to contaminants over the total shelf area on the order of 60-70%, while a thin cap (15 cm) over both area reduces the potential exposures on the order of 60%.    Capping only the most contaminated area (4.9 square kilometers) with a thin cap would reduce potential exposures on the order of 40%.

E)    Cost Estimate to Implement Cleanup Plan

Cost estimates have been developed by the United States Environmental Protection Agency for three capping options (others may be developed):

Option 1 - capping of both areas (4.9 + 2.7 square kilometers) with a thick (45 cm) isolation cap = approximate cost would be $44 million to $67 million.

Option 2 - capping of both areas (4.9 + 2.7 square kilometers) with a thin (15 cm) cap = approximate cost would be $18 million to $30 million.

Option 3 - capping of only the most contaminated area (4.9 square kilometers) with a thin (15 cm) cap    approximate cost would be $13 million to $19 million.

Option 1 would require on the order of 7 million cubic meters of capping material for implementation, while options 2 and 3 would require proportionally less material.

F)    Estimate of Recoverable Costs from Dischargers

The United States National Oceanographic and Atmospheric Administration (NOAA), via its Natural Resource Damage Assessment, and the United States Environmental Protection Agency (EPA), via Superfund, are attempting to recover financial damages from parties responsible for DDT-related damages to the environment on the Palos Verdes Shelf.    EPA estimates that approximately $20-25 million may be recovered from municipalities through settlement agreements.    NOAA and EPA are seeking to recover approximately $100 million from Montrose Chemical Corporation, Westinghouse Electric Corporation and other industrial dischargers.    All of the recovery estimates are approximations, and the actual amount recovered may change.

G)    Two-year Expenditure Schedule

EPA should complete its evaluation of alternatives (including the "no-action" alternative) and issue the EE/CA report during 1999.    At the end of the EE/CA process, EPA will solicit public comment on the EE/CA report, including the recommended removal alternative.    If EPA decides to move ahead, EPA would issue an Action Memorandum formally selecting the response action.

Option 1 would require approximately 5 years to construct with a single hopper dredge.    However, to take advantage of the availability of clean dredged material from the Queensway Bay dredging project for use in the cap, it may be necessary to use three hopper dredges, reducing the time for completion of the project to less than 2 years.    Options 2 and 3 would require proportionally less material and less time for completion.

If $20-25 million becomes available from settlement agreements or other means, Options 2 and 3 potentially could be implemented within two years.    Although Option 1 could be completed with 2 years with the use of multiple hopper dredges, $20-25 million would only allow completion of approximately one-third to one-half of the capping project, unless additional funds are available.

H) Benefits of Remediation

Capping of the DDT and PCB contamination on the Palos Verdes Shelf would isolate this material from the benthic environment and reduce bioaccumulation and movement of contaminants into the food chain.    This would improve the ecological health of the marine environment and could lead to elimination of the health advisory warning against human consumption of fish caught in this area.

I) Environmental Impacts of Remediation

Placement of a cap could release contaminants into the marine environment, but design studies indicate that this should not occur with proper deployment of the capping material.    Depending on the nature of the cap material, placement of the cap could destroy or modify the existing benthic community.    Placement of the cap could cause damage to the ocean outfall and interfere with its operation.    Monitoring will be required to verify the integrity of the final cap and assess environmental impacts from cap placement.

2.    Mugu Lagoon/Calleguas Creek Tidal Prism

Monitoring of Mugu Lagoon and the lower Calleguas Creek watershed has identified the following problems:    1) impaired reproduction in the light-footed clapper rail, a resident endangered species inhabiting the lagoon, due to elevated levels of DDT and PCBs;    2) fish and shellfish tissue levels exceeded National Academy of Sciences guidelines for several pesticides;    3) possible exceedances of U.S. Environmental Protection Agency water quality criteria for the protection of saltwater biota for nickel, copper and zinc at some locations;    4) possible impacts to sediment and water quality, as well as aquatic community health, from operations at the Naval Air Base over many years.    Several pesticides whose use has been discontinued still are found at high concentrations in the sediment and biota;    5) excessive sediment loading.

The Point Mugu Naval Air Base is located in the immediate vicinity of Mugu Lagoon.    The surrounding Oxnard Plain supports a large variety of agricultural crops.    These fields drain into ditches which either enter the lagoon directly or through Calleguas Creek and its tributaries.    The lagoon borders on an Area of Special Biological Significance and supports a great diversity of wildlife, including several endangered birds and one endangered plant species.    Except for the military base, the Oxnard Plain portion of the watershed is relatively undeveloped.

Calleguas Creek and its major tributaries (Revolon Slough, Conejo Creek, Arroyo Conejo, Arroyo Santa Rosa and Arroyo Simi) drain an area of 343 square miles in southern Ventura County and a small portion of western Los Angeles County.    This watershed is about 30 miles long and 14 miles wide.

The Calleguas Creek watershed exhibits some of the most active and severe erosion rates in the country.    Although erosion rates are naturally high in this tectonically active area, land use also is a factor in erosion and sedimentation problems.    Channelization of Calleguas Creek was initiated by local farmers in Somis and downstream areas beginning about 1884, and around Revolon Slough in 1924.    Following complete channelization, eroded sediment generated in the higher reaches of the Calleguas Creek watershed has begun to reach Mugu Lagoon even during minor flood events.    At current rates of erosion, it is estimated that the lagoon habitat could be filled with sediment within 50 years.

Urban developments generally are restricted to the city limits of Simi Valley, Moorpark, Thousand Oaks and Camarillo.    Although some residential development has occurred along the slopes of the watershed, most upland areas still are open space.    Agricultural activities (primarily cultivation of orchard and row crops) are spread out along valleys and on the Oxnard Plain.    The U.S. Navy maintains a Naval Air Base on much of the area around Mugu Lagoon.

The main surface water system drains from the mountains and toward the southwest, where it flows through the flat, expansive Oxnard Plain before emptying into the Pacific Ocean through Mugu Lagoon.    Mugu Lagoon, situated at the mouth of the Calleguas Creek system, is one of the few remaining salt marshes in southern California along the Pacific Flyway.    Threatened and endangered species that are supported by valuable habitats in Mugu Lagoon include the peregrine falcon, least tern, light-footed clapper rail and brown pelican.    In addition to providing one of the last remaining habitats on the mainland for harbor seals to pup, Mugu Lagoon is a nursery ground for many marine fish and mammals.

The Eastern Arm of Mugu Lagoon is somewhat removed from the rest of the lagoon and tends to receive water from and drain directly into the lagoon mouth.    The arm empties and fills rather quickly, leaving a considerable amount of sand near its western end, but moving towards finer sediments further east.    The water tends to be marine in character the majority of the time.

The Main Lagoon and Western Arm are the areas most heavily used by birds (including endangered species).    The Western Arm, with its slight gradient and slow water flow, has the most widespread freshwater influence during dry weather, receiving water from several drains.    The Main Lagoon is affected primarily by Calleguas Creek, which may carry a considerable amount of fresh water during storms, although this flow generally is funneled into a channel which leads to the lagoon mouth.

A) Areal Extent of Toxic Hot Spot

Sediment contamination clearly exists throughout Mugu Lagoon and within the Calleguas Creek Tidal Prism (Figure 10).    Problems appear to be worst in the Western Arm of Mugu Lagoon, particularly near the Rio de Santa Clara, which drains neighboring agricultural lands, and parts of the Eastern Arm.    Although sediment contamination problems occur in the Main Lagoon, it appears that the large volume of this waterbody and good flushing is helping to keep contamination and associated effects at a lower level than might otherwise be expected.    It is estimated that approximately 20% of the Western Arm and approximately 10% of the Eastern Arm of Mugu Lagoon contain contaminated sediments.    The total volume of contaminated sediments is estimated to be approximately 725,000 cubic yards (based on approximately 150 acres with 3-foot depth of contamination).

Twenty-two miles of Calleguas Creek are listed as impaired due to high sediment concentrations of pesticides and accumulation in fish and shellfish.    However, the area with the greatest contamination problem is estimated to cover approximately 3 miles.    The total volume of contaminated sediments is estimated to be approximately 50,000 to 100,000 cubic yards.

In samples collected for the Bay Protection and Toxic Cleanup Program on February 6, 1997, sediment concentrations at stations 48013.0, 48014.0, 48015.0, 48016.0, 48017.0 and 48018.0 exceeded the ERM Thresholds for p,p’-DDE and Total DDT.    Station 44054.0 also exceeded the p,p’-DDE threshold on June 19, 1996.    No sediment chemistry data were collected during sediment toxicity screening surveys conducted on January 12, 1993 and April 14, 1994.

Amphipod toxicity with whole sediment was observed at stations 44016.0, 44050.0, 44051.0, 44052.0, 44053.0 and 44054.0 on January 15, 1993.    Amphipod toxicity was observed at stations 44053.0 and 44054.0 on April 18, 1994, and station 48015.0 on February 10, 1997.    A degraded benthic community was found at all of the stations analyzed (48013.0, 48014.0, 48015.0, 48016.0, 48017.0 and 48018.0) on February 10, 1997.

Fish were collected from Mugu Lagoon for bioaccumulation analyses.    Shiner surfperch exceeded the EPA guidelines for total PCB, but not for total DDT.    Topsmelt did not exceed the EPA screening guidelines for total DDT or total PCB.

Figure 10.    Areal extent of toxic hot spot within Mugu Lagoon/Calleguas Creek Tidal Prism.

Mugu Lagoon

BPTCP Stations With Sediment Chemistry Concentrations Exceeding ERM Threshold

| BPTCP Station | Sampling Date | p,p’- DDE (ppb) | Total DDT (ppb) |
| --- | --- | --- | --- |
| 44054.0 | 6/19/96 | 30.5 |  |
| 48013.0 | 2/6/97 | 44.7 | 64.7 |
| 48014.0 | 2/6/97 | 68.1 | 103.4 |
| 48015.0 | 2/6/97 | 131.0 | 255.1 |
| 48016.0 | 2/6/97 | 112.0 | 166.7 |
| 48017.0 | 2/6/97 | 165.0 | 276.8 |
| 48018.0 | 2/6/97 | 129.0 | 232.6 |

B)    Sources of Pollutants

Pesticides are of concern in Mugu Lagoon at the mouth of the Calleguas Creek watershed.    The primary source of pesticides probably is agricultural runoff, both during dry weather and wet weather.    Water-soluble pesticides currently in use, such as diazinon and chlorpyrifos, may be occurring in sediment porewater at high enough concentrations to be causing observed porewater toxicity.    These pesticides are likely involved with observed upstream ambient toxicity.      Historical discharges of pesticides, such as DDT, PCBs, toxaphene, chlordane and others, probably has contributed to the existing sediment contamination problem.    Erosion from unlined channels in the watershed and from agricultural lands probably contributes to the excessive sediment loading in Mugu Lagoon.    Metals may originate from non-point source runoff during dry and wet weather conditions.

The Regional Board has issued 37 permits for discharges of wastewater from point sources into the Calleguas Creek watershed.    Of the 22 permitted discharges under the NPDES program, 7 are for municipal wastewaters from publicly-owned treatment works, accounting for a combined permitted discharge of 36.7 million gallons per day (98% of the total permitted discharges).    Of the remaining NPDES permits, 11 are for discharges of treated groundwater from hydrocarbon or other contamination, and 5 are general permits for discharges of either well development water or ground water from dewatered aquifers at construction sites.      In addition, 88 releases of stormwater from major municipalities, certain industrial activities and construction projects are now permitted under the Regional Board's NPDES program for storm water.

Only one landfill, the Simi Valley Landfill, is active in the watershed.    Simi Valley Landfill began operating in 1970.    Hazardous wastes were accepted until 1983;    since that time, only Class III wastes (municipal solid waste) have been discharged at this landfill.    Since operations at the landfill predate current regulations for siting waste management units, only a portion of the Simi Valley Landfill is lined in accordance with current regulations.    Leaks from unlined portions of the landfill have contaminated ground water in an underlying sandstone aquifer;    corrective actions are underway by the operator under the direction of the Regional Board.

C)    Actions by Regional Board

The Los Angeles Regional Board’s Water Quality Assessment identifies the following problems in Mugu Lagoon:    aquatic life beneficial use is impaired based on water column exceedances of criteria for copper, mercury, nickel, and zinc, bird reproductivity affected (DDT), tissue accumulation (arsenic, cadmium, silver; chlordane, DDT, endosulfan, dacthal, toxaphene, PCBs);    sediment concentrations (DDT, toxaphene), sediment toxicity and excessive sediment.    Fish consumption beneficial use is impaired based on tissue accumulation of DDT, PCBs and toxaphene.    For Calleguas Creek (Estuary to Arroyo Los Posas), the Water Quality Assessment lists the following problems:    aquatic life beneficial use is impaired based on water column toxicity, sediment contamination (DDT, toxaphene), tissue bioaccumulation (chlordane, toxaphene, PCBs, DDT, ChemA, dacthal, endosulfan) and sediment toxicity.    Fish consumption beneficial use is impaired based on tissue bioaccumulation (DDT, toxaphene, chlordane).

The first large-scale stakeholder effort in the watershed was Mugu Lagoon Task Force, formed in September 1990.    The purpose of the Task Force is to improve communication between agencies with various interests and specific projects in Ventura County that may impact water quality in Mugu Lagoon.    All of the members share a common goal - to preserve and enhance Mugu Lagoon.    The Task Force currently meets infrequently, since many of its members belong to the Calleguas Creek Watershed Management Committee.    Active members of the Mugu Lagoon Task Force include the U.S. Army Corps of Engineers, University of California Cooperative Extension Service Farm Advisor, Ventura County Public Works Agency, Ventura County Planning Department, California Department of Fish and Game, California Coastal Conservancy, U.S. Navy Point Mugu Naval Air Station, Ventura County Resource Conservation District, U.S. Natural Resources Conservation Service and Los Angeles Regional Water Quality Control Board.

The Los Angeles Regional Board's Watershed Management Initiative began in late 1994 with the Calleguas Creek (and Ventura River) watersheds.    Through watershed management, the Regional Board expects to regulate pollutant loads from point sources through permits that better focus on issues relevant to each watershed.    The Regional Board also expects that pollutant loads from nonpoint sources can be better controlled through the participation of the public in the management of their watersheds.

The Los Angeles Regional Board renewed NPDES permits for discharges within the Calleguas Creek Watershed in June 1996.    However, the Regional Board was unable to fully assess cumulative impacts to beneficial uses from all pollutant sources, particularly from nonpoint sources, during the first eighteen months of application of the Watershed Management Initiative.    The Regional Board was able to develop a regional monitoring program for the inland waters of the watershed which is currently being implemented and should provide additional information needed to assess cumulative impacts.

Thanks to the formation of the Calleguas Creek Watershed Management Committee in 1996, stakeholders will have the opportunity to structure and implement measures that will address pollutants from nonpoint sources through the development of a Watershed Management Plan.    The Committee intends to hire a facilitator to help prepare a plan to develop a strategy for the preservation, enhancement and management of the watershed’s resources, including identification and control of sources of pollution.    The Committee has outlined a three-phased plan to accomplish this goal over a 2.5 year period, beginning in January 1998.    The Regional Board plans to reassess cumulative impacts to the beneficial uses of waters in the watershed by fiscal year 2002-2003.    Using this information, the Regional Board is scheduled to revise NPDES permits by June 2003.

The Regional Board is working with the Naval Air Weapons Station at Point Mugu to develop a cleanup plan for contamination at this Department of Defense site.    This effort still is at the stage of characterizing historical sources of pollution and the extent of existing contamination levels.    In the near future, decisions will be made concerning possible remediation and restoration activities in and around Mugu Lagoon.

D)    Preliminary Assessment of Remediation Actions

Effects-based data has established that Mugu Lagoon sediment is more toxic than sediment from other lagoons in the region.    Current agricultural and erosion control practices are likely moving soils heavily polluted with residuals of banned pesticides to drainages and subsequently into Mugu Lagoon.

Under the direction of the California Coastal Conservancy, Ventura County Resource Conservation District and other members of the Mugu Lagoon Task Force, the U.S. Natural Resources Conservation Service completed a report entitled:    "Calleguas Creek Watershed Erosion and Sediment Control Plan for Mugu Lagoon (May 1995)".    The primary focus of this study was to address erosion and sedimentation impacts and solutions for the watershed.    The U.S. Environmental Protection Agency, State Water Resources Control Board and the Los Angeles Regional Water Quality Control Board recently have granted additional 319(h) funds to implement specific erosion control measures for Grimes Canyon, a critical area targeted for remediation in the plan.

Existing contaminated sediments within Mugu Lagoon and the Calleguas Creek Tidal Prism are unlikely to remediate naturally within a reasonable time frame.    Removal of the contaminated sediments (i.e., dredging) or treatment appear to be the most appropriate remediation alternatives, although in situ capping might be the best solution for historical deposits, particularly within the lagoon.

E)    Cost Estimate to Implement Cleanup Plan

Given the sensitive nature of Mugu Lagoon as a habitat for endangered species, the most likely remediation alternatives would be no action or in situ treatment.    The no action alternative would not have a financial cost, but the contaminated sediment could remain in the environment and continue to cause problems for several more decades.    In situ treatment would be very expensive and may pose technical problems for remediation in an estuarine environment.    No reliable cost estimate exists at this time for this treatment method, but it would probably exceed $100 per cubic yard.    The total cost for remediation of Mugu Lagoon would be at least $72.5 million.

Dredging could be used to remove the contaminated sediments from the Calleguas Creek Tidal Prism.    However, identifying a suitable and legal disposal site for contaminated sediments may be difficult.    Application of this technique would cost an estimated $1 million to $5 million, based on a cost estimate of $20-100 per cubic yard (disposal costs are likely to be high, so the cost estimate probably would approach or even exceed the upper limit of the cost estimate range).

F)    Estimate of Recoverable Costs From Dischargers

Contamination of the Mugu Lagoon sediments probably associated with historical use of the now-banned pesticide DDT.    Although the United States Navy could be liable for any remediation activities required as a result of historical discharges of pollutants due to operations at the Naval Air Weapons Station at Point Mugu, there is no evidence that the Navy is responsible for the elevated concentrations of DDT in the sediments.    It is unlikely that costs can be recovered from any other dischargers in this watershed.

G) Two-Year Expenditure Schedule

The Regional Board plans to work with the Calleguas Creek Watershed Management Committee, which already has begun development of a watershed management plan, to select the appropriate remediation alternative for Mugu Lagoon and the Calleguas Creek Tidal Prism.    In addition, watershed management measures may be required to control sources of contaminants and prevent recontamination of these areas.

During Year One, the focus would be on selection of the appropriate remediation alternative for Mugu Lagoon and Calleguas Creek Tidal Prism.    Additional sediment sampling may be required, particularly for Calleguas Creek Tidal Prism, to fully characterize the areal extent of the sediment contamination and prepare a plan for capping, dredging or treatment of the contaminated sediments.    This sampling program probably will require approximately $100,000 - $250,000 for implementation.    A source for this funding has not been determined.

During Year Two, the focus would be on implementation of the remediation alternative(s) selected for Mugu Lagoon and Calleguas Creek Tidal Prism, as well as watershed management measures to control sources of contamination and prevent recontamination of the existing hot spots.    Remediation of the Calleguas Creek Tidal Prism probably could be completed within Year Two, if funding is available.    However, remediation of Mugu Lagoon could require additional time, depending upon the alternative selected.    A monitoring program will be required to measure the success of the remediation plans that are implemented;    although a monitoring program has not yet been designed, the estimated cost would be $50,000 - $100,000 per year, and may be required for at least three to five years following completion of the remediation activities.

H) Benefits of Remediation

Successful remediation of the contamination in Mugu Lagoon and the Calleguas Creek Tidal Prism could eliminate the source of impairment of the beneficial uses of these waters.    However, watershed management efforts to control erosion probably would be required to prevent recontamination of the these areas.

I) Environmental Impacts of Remediation

If in-situ treatment is implemented, it could result in short-term impacts to the benthic infaunal community.    However, this community would be expected to fully recover within 2-3 years.    Any remediation activity within this sensitive watershed, particularly in Mugu Lagoon, potentially could affect endangered species, such as the peregrine falcon, least tern, light-footed clapper rail and brown pelican.    Prior to initiating any remediation plan, the Regional Board will consult with the California Department of Fish and Game and the United States Fish and Wildlife Service concerning potential adverse impacts to endangered species.

With proper management of dredging and disposal of dredged material, this activity would not be expected to result in adverse environmental impacts.

3.    Los Angeles/Long Beach Harbors

The Los Angeles and Long Beach Harbors are located in the southeastern portion of the Los Angeles Basin.    Along the northern portion of San Pedro Bay, there is a natural embayment formed by a westerly extension of the coastline which contains both harbors, with the Palos Verdes Hills as the dominant onshore feature.    Offshore, a generally low topographic ridge is associated with the eastern flank of the Palos Verdes uplift and adjacent Palos Verdes fault zone, and extends northwest across the San Pedro shelf nearly to the breakwater of the Los Angeles Harbor.

The port and harbor areas have been modified over the course of more than one hundred years to include construction of breakwaters, landfills, slips and wharves, along with channelization of drainages, dredging of navigation channels and reclamation of marshland.    The inner harbor includes the Main Channel, the East and West Basins, and the East Channel Basin.    The outer harbor is the basin area located between Terminal Island and the San Pedro and Middle Breakwaters.    Los Angeles and Long Beach Harbor are considered to be a single oceanographic unit, and share a common breakwater across the mouth of San Pedro Bay.    The outer harbor areas reflect the conditions of the coastal marine waters of the Southern California Bight, while the inner harbor areas typically have lower salinities.

In the presence of the strong currents and rocky habitat of the outer harbor, aquatic life communities are similar to those of the nearby coast, while the inner harbor supports biota generally found in bays and estuaries.    The inner harbor has a mostly soft bottom character.

The major surface drainages in the area include the Los Angeles River, which flows in a channel and drains parts of the San Fernando Valley, as well as downtown and south Los Angeles, into eastern San Pedro Bay at Long Beach.    The Dominguez Channel drains the intensely urbanized area west of the Los Angeles River into the Consolidated Slip of the Los Angeles Inner Harbor, carrying with it mostly urban runoff and non-process industrial waste discharges.    A major source of both freshwater and waste in the outer harbor is secondary effluent from the Terminal Island Treatment Plant.    Waste discharges to the inner harbor area of Los Angeles Harbor consist of both contact and non-contact industrial cooling wastewater and stormwater runoff.    Fuel spills and oil spills from marine vessel traffic or docking facilities also contribute pollutants to the inner harbor.

3.1    Los Angeles Outer Harbor/Cabrillo Pier

A)    Areal Extent of Toxic Hot Spot

The site's toxic hot spot status is based on several factors, including a fish advisory warning against human consumption of white croaker, which resulted from an OEHHA study released in 1991 which cited elevated DDT and PCB levels in a number of fish species caught in the area.    Sediment DDT levels in some BPTCP samples collected from the site were elevated above that found elsewhere in the harbor, while sediment PCB levels were comparable to other sites.    Sediment toxicity fluctuated widely.    This is a heavily used sustenance and sportfishing pier (Figure 11).    It is unclear whether fish caught there are contaminated from DDT found locally or from sources outside of but close to the harbor.      It is estimated that 25,000 to 50,000 cubic yards of contaminated sediments exist within the Cabrillo Pier area (based on 1 to 2 foot depth of contaminants).

Based on samples collected for the BPTCP, sediment concentrations exceeded the ERM Threshold for Total DDT at every station (40010.1, 40010.2, 40010.3, 49001.0, 49002.0, 49003.0) on each occasion that sediment chemistry analyses were conducted (August 18, 1992;    September 16, 1992;    August 19, 1993;    May 19, 1994;    February 15, 1994;    May 13, 1997).    Sediment concentrations also exceeded the ERM for copper at station 40010.1 (Replicates 1, 2 and 3) on February 14, 1994.    Amphipod toxicity with whole sediments was observed at station 40010.1 on May 28, 1993, and again at stations 40010.1, 40010.2 and 40010.3 on February 14, 1994.    A degraded benthic community was observed at station 40010.2 (Replicate 2) on August 17-19, 1993.

Figure 11.    Areal extent of toxic hot spot within Los Angeles Outer Harbor/Cabrillo Pier.

Cabrillo Pier Area

BPTCP Stations With Sediment Chemistry Concentrations Exceeding ERM Threshold

| BPTCP Station | Sampling Date | Copper (ppm) | Total DDT (ppb) |
| --- | --- | --- | --- |
| 40010.1 | 8/18/92 |  | 267.5 |
| 40010.2 | 8/18/92 |  | 207.3 |
| 40010.3 | 8/18/92 |  | 203.7 |
| 40010.1 | 9/16/92 |  | 195.6 |
| 40010.1 | 8/19/93 |  | 304.8 |
| 40010.2 | 8/19/93 |  | 204.6 |
| 40010.3 | 8/19/93 |  | 215.0 |
| 40010.1 Rep 1 | 2/15/94 | 247.0 | 175.0 |
| 40010.1 Rep 2 | 2/15/94 | 274.0 | 186.6 |
| 40010.1 Rep 3 | 2/15/94 | 273.0 | 174.4 |
| 40010.2 Rep 1 | 2/15/94 |  | 207.2 |
| 40010.2 Rep 2 | 2/15/94 |  | 168.8 |
| 40010.2 Rep 3 | 2/15/94 |  | 180.4 |
| 40010.3 Rep 1 | 2/15/94 |  | 171.6 |
| 40010.3 Rep 2 | 2/15/94 |  | 212.0 |
| 40010.3 Rep 3 | 2/15/94 |  | 163.2 |
| 49001.0 | 5/13/97 |  | 192.9 |
| 49002.0 | 5/13/97 |  | 100.0 |
| 49003.0 | 5/13/97 |  | 53.5 |

Fish were collected on May 12, 1997, to assess bioaccumulation of DDT and PCB.    Total DDT and total PCB in white croaker muscle tissue samples exceeded EPA screening values at stations 49001.0, 49002.0 and 49003.0.    Total PCB in white surfperch muscle tissue also exceeded the EPA screening value at all three stations, although total DDT concentrations fell below the EPA screening value.    Clams (Macoma) collected at station 49002.0 also exceeded the EPA screening value for total PCB.

B)    Sources of Pollutants

Historical discharges of DDT, PCBs and metals are the probable cause of sediment contamination in the Cabrillo Pier area.    Discharge of wastewater effluent from the Terminal Island Treatment Plant is a potential source of pollutants, especially metals.    Nonpoint sources of pollutants include spills from ships and industrial facilities, as well as stormwater runoff.    Many areas of the port have experienced soil and/or groundwater contamination, which may result in possible transport of pollutants to the harbor’s surface waters.

C)    Actions by Regional Board

The Los Angeles Regional Board’s Water Quality Assessment lists the following problems in the Cabrillo area of Los Angeles Outer Harbor:    aquatic life beneficial use is impaired due to tissue accumulation (DDT), sediment toxicity, sediment contamination (PAHs, DDT, zinc, copper, chromium).

The Los Angeles Regional Board has adopted a watershed management approach, which is expected to regulate pollutant loads from point sources through permits that better focus on issues relevant to each watershed.    The Regional Board also expects that pollutant loads from nonpoint sources can be better controlled through the participation of the public in the management of their watersheds.    During the 2001-02 Fiscal Year, the watershed management approach will be used to renew NPDES permits within the Los Angeles/Long Beach Harbors Watershed. The Los Angeles Regional Board’s Site Cleanup Unit has developed cleanup and remediation plans for many contaminated soil and groundwater sites, including refineries and old oil fields.    The Regional Board has issued waste discharge requirements for some of the boatyards and stormwater runoff sources within the port.

The Los Angeles Regional Board and the California Coastal Commission began work during fiscal year 1997-98 to prepare a long-term management plan for the dredging and disposal of contaminated sediments in the coastal waters adjacent to Los Angeles County.    The goals of this plan will be to develop unified multi-agency policies for the management of contaminated dredged material, promote multi-user disposal facilities and reuse, to the extent practicable, and support efforts to control contaminants at their source using a watershed management approach.

D)    Preliminary Assessment of Remediation Actions

Given the protected nature of the Cabrillo Pier area within the Los Angeles Outer Harbor, in situ capping might be a feasible method for containment of contaminated sediments.    Dredging would be a proven method to remove the contaminated sediments, but identification of a suitable and legal disposal site is often a problem.    Treatment of contaminated sediments may be feasible, but is likely to be expensive and difficult to accomplish with marine sediments.

E)    Cost Estimate to Implement Cleanup Plan

In situ capping would probably be the least expensive remediation option.    However, a stable cap must be designed to prevent reexposure of the contaminated sediments.    Application of this technique to contain contaminated sediments from the Cabrillo Pier area would cost an estimated $0.5 million to $1 million, based on a cost estimate of up to $20 per cubic yard (this is a rough estimate, since the unit cost could be higher).

Dredging could be used to remove the contaminated sediments from the Cabrillo Pier area.    However, identifying a suitable and legal disposal site for a large volume of contaminated sediments can be difficult.    Application of this technique would cost an estimated $0.5 million to $5 million, based on a cost estimate of $20-100 per cubic yard (if a disposal site, such as a confined aquatic disposal or land disposal site, is available within or close to the Los Angeles/Long Beach Harbors complex, the cost estimate probably would approach the lower limit of the cost estimate range).

Treatment of the contaminated sediments is likely to be expensive.    Application of this technique would cost an estimated $2.5 million to $50 million, based on a cost estimate of $100-$1,000 per cubic yard (due to limited experience in treating marine sediments, costs are likely to be in the upper part of the cost estimate range).

F)    Estimate of Recoverable Costs from Dischargers

In July 1996, the U.S. Environmental Protection Agency decided to undertake a Superfund response (under the Comprehensive Environmental Response, Compensation and Liability Act) to address the contaminated sediment problem on the Palos Verdes Shelf.    However, the Los Angeles Harbor area was not included within the scope of the Superfund action.    Since it will be difficult or impossible to prove that the contamination of the harbor is due to stormwater runoff from the Montrose Chemical Corporation’s historical manufacturing site in Torrance, which appears to be a likely source for this contamination, we do not anticipate recovering any remediation costs from dischargers.

G)    Two-year Expenditure Schedule

The Regional Board plans to work with the Los Angeles Basin Contaminated Sediments Task Force to select a remediaton alternative and implement the cleanup plan for the Cabrillo Pier hot spot.    Additional sediment sampling will be required to better define the areal extent of the sediment contamination, prior to selection of an appropriate remediation alternative.    This sampling program could be conducted during Year One, if funding becomes available (estimated cost approximately $250,000 - $500,000).    However, the Regional Board would recommend implementing the cleanup of the Consolidated Slip/Dominguez Channel hot spot prior to initiating any remediation activities at the Cabrillo Pier site, since the Consolidated Slip/Dominguez Channel area may represent a source of contamination to the Cabrillo Pier area.    A monitoring program would be required upon completion of any remediation activities;    it is estimated that monitoring would cost $50,000 to $100,000 per year, and may be required for three to five years.

H)    Benefits of Remediation

Remediation of the contamination would eliminate the immediate source of impairment of beneficial uses of the receiving waters.    However, recontamination from other areas of the harbor could occur.

I)    Environmental Impacts of Remediation

If capping or dredging is implemented, it could result in short-term impacts to the benthic infaunal community.    However, this community would be expected to fully recover within 2-3 years.    There is potential for release of contaminants into the marine environment during dredging, but proper management of this operation should minimize this risk.    Special management practices would be required for disposal of contaminated sediments to contain the material and prevent releases of contaminants to the environment.

3.2    Los Angeles Inner Harbor/Dominguez Channel, Consolidated Slip

A)    Areal Extent of Toxic Hot Spot

A reservoir of polluted sediment in Consolidated Slip (moving down from Dominguez Channel) probably is continuing to contaminate a large part of Los Angeles Inner Harbor (Figure 12).    It is estimated that approximately 30,000 cubic yards of contaminated sediments exist in Consolidated Slip and approximately 20,000 cubic yards in Dominguez Channel (based on 6 miles of channel contaminated to an average depth of 1 foot).

In limited sampling conducted on July 30, 1992, sediment samples from stations 40006.1 and 40006.2 exceeded ERM thresholds for zinc, total chlordane and total PCB;    in addition, station 40006.1 also exceeded the ERM for mercury.    Amphipod toxicity with whole sediments, as well as porewater toxicity with the abalone test, were observed at both stations.    A degraded benthic community was observed at station 40006.1.

In limited sampling conducted on February 3, 1994, sediment samples from station 40006.1 (Replicates 1, 2 and 3) exceeded ERM thresholds for zinc, total chlordane, total PCB and high molecular weight PAH;    in addition, Replicate 3 from this station also exceeded the ERM for mercury.    Amphipod toxicity was observed in Replicates 1 and 2 from station 40006.1.    Benthic samples were not analyzed on this occasion.

A more extensive survey was conducted at several stations on July 22, 1996, including the collection of surface samples and subsurface samples.    Sediment samples from stations 47001.0, 47002.0, 47003.0, 47004.0, 47005.0, 47010.0, 47007.0, 47008.0 and 47009.0 all exceeded at least one ERM threshold, and sometimes exceeded several, including those for cadmium, copper, lead, mercury, zinc, dieldrin, total PCB, low molecular weight PAH, high molecular weight PAH and total PAH.    Amphipod toxicity with whole sediment was observed at stations 47001.0 (surface and depth 2), 47002.0 (surface), 47003.0 (surface and depth 2), 47004.0 (surface and depth 2), 40005.0 (surface and depth 2), 47007.0 (surface), 47008.0, 47009.0 (surface) and 47010.0 (surface).    A degraded benthic community was found at stations 47002.0, 47003.0, 47009.0 and 47010.0.

When average ERM Quotient exceeds 1.00, the probability of amphipod toxicity was found to be 71% (Long et al., 1995).    When average PEL Quotient exceeds 1.00, probability of significant amphipod toxicity was found to be 56% (McDonald, 1996).    Consolidated Slip exceeded both of these effect thresholds at several stations (47004.0, 4006.1, 47002.0, 47009.0, 47003.0, 47008.0, 47001.0, 40006.2, 40007.0). When sediment concentrations were found to exceed 11 or more of the ERM thresholds, 85% of the samples have been found to be significantly toxic to amphipods.    When sediment concentrations exceeded 21 or more of the PEL thresholds, 100% of the samples have been found to be significantly toxic to amphipods.    One of the Consolidated Slip stations exceeded the ERM threshold (47004.0), but not the PEL threshold.

Figure 12.    Areal Extent of toxic hot spot within Los Angeles Inner Harbor/Dominguez Channel, Consolidated Slip

Consolidated Slip/Dominguez Channel

BPTCP Stations With Sediment Chemistry Concentrations Exceeding ERM Threshold

| BPTCP Station | Sampling Date | Compound | Concentration |
| --- | --- | --- | --- |
| 40006.1 | 7/30/92 | Mercury | 0.73 ppm |
| 40006.1 | 7/30/92 | Zinc | 540 ppm |
| 40006.1 | 7/30/92 | Total Chlordane | 50.0 ppb |
| 40006.1 | 7/30/92 | Total PCB | 473.8 ppb |
| 40006.2 | 7/30/92 | Zinc | 570 ppm |
| 40006.2 | 7/30/92 | Total Chlordane | 46.0 ppb |
| 40006.2 | 7/30/92 | Total PCB | 534.5 ppb |
| 40006.1 Rep 1 | 2/3/94 | Zinc | 463 ppm |
| 40006.1 Rep 1 | 2/3/94 | Total Chlordane | 112.8 ppb |
| 40006.1 Rep 1 | 2/3/94 | Total PCB | 513.1 ppb |
| 40006.1 Rep 1 | 2/3/94 | HMW PAH | 12146 ppb |
| 40006.1 Rep 2 | 2/3/94 | Zinc | 606 ppm |
| 40006.1 Rep 2 | 2/3/94 | Total Chlordane | 83.6 ppb |
| 40006.1 Rep 2 | 2/3/94 | Total PCB | 504.5 ppb |
| 40006.1 Rep 2 | 2/3/94 | HMW PAH | 11963 ppb |
| 40006.1 Rep 3 | 2/3/94 | Mercury | 0.74 ppm |
| 40006.1 Rep 3 | 2/3/94 | Zinc | 616 ppm |
| 40006.1 Rep 3 | 2/3/94 | Total Chlordane | 58.1 ppb |
| 40006.1 Rep 3 | 2/3/94 | Total PCB | 578.6 ppb |
| 40006.1 Rep 3 | 2/3/94 | HMW PAH | 12553 ppb |
| 47001.0 Surface | 7/22/96 | Total PCB | 981.8 ppb |
| 47001.0 Depth 2 | 7/22/96 | Total PCB | 646.2 ppb |
| 47002.0 Surface | 7/22/96 | Total PCB | 2118.2 ppb |
| 47002.0 Depth 2 | 7/22/96 | Total PCB | 803.3 ppb |
| 47002.0 Depth 2 | 7/22/96 | HMW PAH | 10374 ppb |
| 47003.0 Surface | 7/22/96 | Dieldrin | 10.1 ppb |
| 47003.0 Surface | 7/22/96 | Total PCB | 1420.6 ppb |
| 47003.0 Depth 2 | 7/22/96 | Lead | 385.0 ppm |
| 47003.0 Depth 2 | 7/22/96 | Mercury | 1.57 ppm |
| 47003.0 Depth 2 | 7/22/96 | Zinc | 568 ppm |
| 47003.0 Depth 2 | 7/22/96 | Total PCB | 893.4 ppb |
| 47004.0 Surface | 7/22/96 | Zinc | 473 ppm |
| 47004.0 Surface | 7/22/96 | HMW PAH | 11721 ppb |

| 47004.0 Depth 2 | 7/22/96 | Mercury | 0.78 ppm |
| --- | --- | --- | --- |
| 47004.0 Depth 2 | 7/22/96 | Zinc | 737 ppm |
| 47003.0 Depth 2 | 7/22/96 | Dieldrin | 33.3 ppb |
| 47004.0 Depth 2 | 7/22/96 | Total PCB | 1341.6 ppb |
| 47004.0 Depth 2 | 7/22/96 | LMW PAH | 9679 ppb |
| 47004.0 Depth 2 | 7/22/96 | HMW PAH | 16467 ppb |
| 47005.0 Surface | 7/22/96 | Copper | 478 ppm |
| 47005.0 Surface | 7/22/96 | Lead | 460 ppm |
| 47005.0 Surface | 7/22/96 | Mercury | 3.28 ppm |
| 47005.0 Surface | 7/22/96 | Zinc | 447 ppm |
| 47005.0 Surface | 7/22/96 | Total PCB | 1599.9 ppb |
| 47005.0 Depth 2 | 7/22/96 | Copper | 1740 ppm |
| 47005.0 Depth 2 | 7/22/96 | Lead | 542 ppm |
| 47005.0 Depth 2 | 7/22/96 | Mercury | 2.94 ppm |
| 47005.0 Depth 2 | 7/22/96 | Zinc | 700 ppm |
| 47005.0 Depth 2 | 7/22/96 | Total PCB | 525.8 ppb |
| 47005.0 Depth 3 | 7/22/96 | Lead | 1590 ppm |
| 47005.0 Depth 3 | 7/22/96 | Mercury | 1.49 ppm |
| 47005.0 Depth 3 | 7/22/96 | Zinc | 1010 ppm |
| 47010.0 Surface | 7/22/96 | Total PCB | 361.5 ppb |
| 47007.0 Surface | 7/22/96 | Total PCB | 246.2 ppb |
| 47008.0 | 7/22/96 | Cadmium | 14.5 ppm |
| 47008.0 | 7/22/96 | Total PCB | 942.4 |

B)    Sources of Pollutants

Historical discharges of DDT, PCBs and metals probably caused much of the existing contamination.    Current point source discharges of process water and other waste streams from refineries located along Dominguez Channel may be contributing to the contamination problem.    Numerous nonpoint sources, such as spills, vessel discharges, leaching of pollutants from boat anti-fouling paints, and storm drains, also are present in the area.

C)    Actions by Regional Board

The Los Angeles Regional Board’s Water Quality Assessment lists the following problems in Dominguez Channel: aquatic life beneficial use is impaired due to sediment contamination (chromium, zinc, DDT, PAHs) and benthic community impairment.    The Water Quality Assessment identifies the following problems in Consolidated Slip:    aquatic life beneficial use is impaired due to tissue accumulation (DDT, chlordane, PCBs, tributyltin, zinc), sediment toxicity, benthic community effects, sediment contamination (PAHs, zinc, chromium, lead, DDT, chlordane, PCBs);    fish consumption advisory.

The Los Angeles Regional Board’s Site Cleanup Unit has developed cleanup and remediation plans for many contaminated soil and groundwater sites, including refineries and old oil fields.    The Regional Board has issued waste discharge requirements for some of the boatyards and stormwater runoff sources within the port.

The Los Angeles Regional Board has adopted a watershed management approach, which is expected to regulate pollutant loads from point sources through permits that better focus on issues relevant to each watershed.    The Regional Board also expects that pollutant loads from nonpoint sources can be better controlled through the participation of the public in the management of their watersheds.    During the 2001-02 Fiscal Year, the watershed management approach will be used to renew NPDES permits within the Los Angeles/Long Beach Harbors Watershed and the Dominguez Channel Watershed.

The Los Angeles Regional Board and the California Coastal Commission began work during fiscal year 1997-98 to prepare a long-term management plan for the dredging and disposal of contaminated sediments in the coastal waters adjacent to Los Angeles County.    The goals of this plan will be to develop unified multi-agency policies for the management of contaminated dredged material, promote multi-user disposal facilities and reuse, to the extent practicable, and support efforts to control contaminants at their source using a watershed management approach.

D)    Preliminary Assessment of Remediation Actions

Dredging would be a proven method to remove the contaminated sediments, but identification of a suitable and legal disposal site often can be a problem.    Treatment of contaminated sediments may be feasible, but is likely to be expensive and difficult to accomplish with marine sediments.    In situ capping is not likely to be chosen as an alternative, due to the high flows that can occur in this area and the potential for reexposure and transport of contaminated material.

E)    Cost Estimate to Implement Cleanup Plan

Dredging could be used to remove the contaminated sediments from the Dominguez Channel/Consolidated Slip area.    However, identifying a suitable and legal disposal site for a large volume of contaminated sediments can be difficult.      Application of this technique would cost an estimated $1 million to $5 million, based on a cost estimate of $20-100 per cubic yard (if a disposal site, such as a confined aquatic disposal or land disposal site, is available within or close to the Los Angeles/Long Beach Harbors complex, the cost estimate probably would approach the lower limit of the cost estimate range).

Treatment of the contaminated sediments is likely to be expensive.    Application of this technique would cost an estimated $5 million to $50 million, based on a cost estimate of $100-$1,000 per cubic yard (due to limited experience in treating marine sediments, costs are likely to be in the upper part of the cost estimate range).

F)    Estimate of Recoverable Costs from Dischargers

No responsible parties have been identified from which costs could be recovered.

G)    Two-year Expenditure Schedule

The Regional Board plans to work with the Los Angeles Basin Contaminated Sediments Task Force to select a remediation alternative and implement the cleanup plan for the Consolidated Slip/Dominguez Channel hot spot.    Additional sediment sampling will be required to precisely define the areal extent of the sediment contamination, prior to selection of an appropriate remediation alternative.    This sampling program could be conducted during Year One, if funding becomes available (estimated cost approximately $250,000 - $500,000). If dredging is selected as the desired remediation method, the Regional Board will work with the Task Force to identify a suitable disposal alternative (e.g., constructed fill site, confined aquatic disposal site). A monitoring program would be required upon completion of any remediation activities;    it is estimated that monitoring would cost $50,000 to $100,000 per year, and may be required for three to five years.

H)    Benefits of Remediation

Remediation of the contamination would eliminate the immediate source of impairment of beneficial uses of the receiving waters.    However, recontamination of the site from other areas is possible.

I)    Environmental Impacts of Remediation

If capping or dredging is implemented, it could result in short-term impacts to the benthic infaunal community.    However, this community would be expected to fully recover within 2-3 years.    There is potential for release of contaminants into the marine environment during dredging, but proper management of this operation should minimize this risk.    Special management practices would be required for disposal of contaminated sediments to contain the material and prevent releases of contaminants to the environment.