- Table A9: Measurement Quality Objectives* – Toxicity Testing (General)

| Negative Controls | Frequency of Analysis | Control Limits |
| --- | --- | --- |
| Laboratory Control Water | Laboratory Control Water consistent with Section 7 of the appropriate EPA method must be tested with each analytical batch. | Laboratory Control Water must meet all test acceptability criteria (Please refer to Section 7 of the EPA manuals) for the species of interest. |
| Conductivity/Salinity Control Water | It is required that a conductivity or salinity control be tested when these parameters are above or below the species tolerance . | Follow EPA guidance on interpreting data and refer to tables below for tolerance ranges. |
| Additional Control Water | Additional method blanks are required whenever manipulations are performed on one or more of the ambient samples within each analytical batch (e.g. pH adjustments, continuous aeration, etc.). | No statistical difference between the laboratory control water and each additional control water within an analytical batch. |
| Sediment Control | Sediment Control consistent with those described in Section 7 of the EPA manual must be tested with each analytical batch of sediment toxicity tests. | Sediment Control must meet all data acceptability criteria (Please refer to Section 7 of the EPA manuals) for the species of interest. |
| Positive Controls | Frequency of Analysis | Control Limits |
| Reference Toxicant Tests | Reference Toxicant Tests must be conducted monthly for species that are raised within a laboratory. Reference Toxicant Test must be conducted per analytical batch for species from commercial supplier settings. Reference Toxicant Tests must be conducted concurrently for test species or brood stocks that are field collected. | Last plotted data point must be within 2 SD of the cumulative mean (n=20). (Reference toxicant tests that fall outside of recommended control chart limits are evaluated to determine the validity of associated tests. An out of control reference toxicant test result does not necessarily invalidate associated test results. More frequent and/or concurrent reference toxicant testing may be advantageous if recent problems have been identified in testing.) |
| Field Quality Control | Frequency of Analysis | Control Limits |
| Sample Duplicate | 5% of total project sample count | Recommended acceptable RPD criterion +/- 20% |
| Field Blanks | Based on project requirements | No statistical difference between the laboratory control water (or sediment control) and the field blank within an analytical batch |
| Bottle Blanks | Based on project requirements | No statistical difference between the Laboratory Control Water and the Equipment Blank within an analytical batch |

*Unless method specifies more stringent requirements.

The measurement quality objectives for water quality parameters (pH, dissolved oxygen, conductivity, temperature, unionized ammonia, salinity, alkalinity and hardness) are detailed in the Field Measurement and Conventional Analytes tables of this Appendix.

In special cases where the criteria listed in the following tables cannot be met, EPA minimum criteria may be followed. The affected data should be qualified accordingly.

Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests should be repeated with the newly collected sample.  If this is not possible, test should be repeated with archived sample and all tests must be properly qualified.

Deviations from the summary of recommended test conditions must be evaluated on a project specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result depending on the degree of the departure and the objective of the test.  The reviewer should consider the degree of the deviation and the potential or observed impact of the deviation on the test result before rejecting or accepting a test result is valid.  For example, if dissolved oxygen is measured below 4.0 mg/L in one test chamber, the reviewer should consider whether any observed mortality in that test chamber corresponded with the drop in dissolved oxygen.

Table A10: Measurement Quality Objectives – 7-Day Survival and Growth Pimephales promelas Toxicity Test

| Method Recommendation |  |
| --- | --- |
| EPA/821/R-02/013 (Test Method 1000.0) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | 80% or greater survival in controls and an average dry weight per surviving organism in control chambers equals or exceeds 0.25 mg |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static renewal |
| Age at Test Initiation | Newly-hatched larvae <24 hours old. If shipped, <48 hours old with a 24-hour age range |
| Replication at Test Initiation | 4 (minimum) |
| Organisms/Replicate | 10 (minimum) |
| Food Source | Newly-hatched Artemia nauplii (<24hoursold) |
| Renewal Frequency | Daily |
| Test Duration | 7 days |
| Endpoints | Survival and growth (biomass) |
| Test Conditions | Recommended** |
| Temperature Range | 25 ± 1.0 °C (+/- 3 C required) |
| Light Intensity | 10 – 20 µE/m2/s or 50 – 100 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | >500 mL or per method specific requirements |
| Replicate Volume | >250 mL or per method specific requirements |
| Feeding Regime | 2 or 3 times per day |
| Laboratory Control Water | Moderately hard water prepared in accordance with EPA protocols |
| Minimum Sample Volume | 7 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, conductivity and temperature measurement per sample and per dilution |
| Initial Unionized Ammonia | One measurement per sample |
| Initial Hardness and Alkalinity | One measurement per sample within 48 h of sample receipt |
| Daily Water Chemistry | One initial DO, one final DO and one final pH measurement per sample |
| Final Water Chemistry | One DO, pH, and temperature measurement and per sample and per dilution |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% saturation |
| Initial pH Range | 6.0 - 9.0 |
| Conductivity Controls | Include appropriate controls when sample conductivities are 0 – 100, or above 1900 µS/cm |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A11: Measurement Quality Objectives – 6-8 Day Survival and Reproduction Ceriodaphnia dubia Toxicity Test

| Method Recommendation |  |
| --- | --- |
| EPA/821/R-02/013 (Test Method 1002.0) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥80% survival in controls and an average of 15 or more young per surviving female. 60% of the surviving control females must produce three broods. |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static renewal |
| Age at Test Initiation | <24 hours old and all released within an 8-h period |
| Replication at Test Initiation | 10 (minimum) |
| Organisms/Replicate | One ( assigned using blocking by known parentage) |
| Food Source | YCT and Selenastrum or comparable food |
| Renewal Frequency | Daily |
| Test Duration | 6-8 days (when 60% surviving females produces 3rd brood |
| Endpoints | Survival and reproduction |
| Test Conditions | Recommended** |
| Temperature Range | 25 ± 1.5 °C (+/- 3 C required) |
| Light Intensity | 10 – 20 µE/m2/s or 50 – 100 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 20 - 40 mL |
| Replicate Volume | >15 mL |
| Feeding Regime | Daily |
| Laboratory Control Water | Moderately hard water prepared in accordance with EPA protocols |
| Minimum Sample Volume | 2 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, conductivity and temperature measurement per sample and per dilution |
| Initial Unionized Ammonia | One measurement per sample |
| Initial Hardness and Alkalinity | One measurement per sample within 48 h of sample receipt |
| Daily Water Chemistry | One initial DO, one final DO and one final pH measurement per sample |
| Final Water Chemistry | One DO, pH and temperature measurement per sample and per dilution |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% saturation |
| Initial pH Range | 6.0 - 9.0 |
| Conductivity Controls | Include appropriate controls when sample conductivities are 0 – 100, or >1900 µS/cm. Substitute with Hyalella azteca if conductivity is >2500. |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A12: Measurement Quality Objectives – 96-Hour Survival Ceriodaphnia dubia Toxicity Test

| Method Recommendation |  |
| --- | --- |
| EPA/821/R-02/012 (Test Method 2002.0) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥90% survival in the controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static renewal |
| Age at Test Initiation | <24hours |
| Replication at Test Initiation | 4 (minimum) |
| Organisms/Replicate | 5 (minimum) |
| Food Source | YCT and Selenastrum or comparable food |
| Test Duration | 96 hours |
| Renewal Frequency | 100% Daily Renewal |
| Feeding Regime | Feed while holding prior to test and 2 hours prior to test solution renewal |
| Endpoints | Survival |
| Test Conditions | Recommended** |
| Temperature Range | 25 ± 1 °C (+/- 3 C required) |
| Light Intensity | 10 – 20 µE/m2/s OR 50 – 100 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 20 - 40 mL |
| Replicate Volume | >15 mL |
| Laboratory Control Water | Moderately hard water prepared in accordance with EPA protocols |
| Minimum Sample Volume | 1 L |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, conductivity and temperature measurement per sample and per dilution |
| Initial Unionized Ammonia | One measurement per sample |
| Initial Hardness and Alkalinity | One measurement per sample within 48 h of sample receipt |
| Daily Water Chemistry | One initial DO, one final DO and one final pH measurement per sample |
| Final Water Chemistry | One DO, pH, and temperature measurement per sample and per dilution |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% saturation |
| Initial pH Range | 6.0 - 9.0 |
| Conductivity Controls | Include appropriate controls when sample conductivities are 0 – 100, or >1900 µS/cm. Substitute with Hyalella azteca if conductivity is >2500. |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A13: Measurement Quality Objectives – 10-Day Survival Hyalella azteca Water Toxicity Test

| Method Recommendation |  |
| --- | --- |
| EPA/821/R-02/012 or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥80% survival in controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static renewal |
| Age at Test Initiation | 7 – 14 days old |
| Replication at Test Initiation | 5 (minimum) |
| Organisms/Replicate | 10 (minimum) |
| Food Source | YCT |
| Renewal Frequency | 80% renewal on Day 5 (or more frequently if desired) |
| Test Duration | 10 days |
| Endpoints | Survival |
| Test Conditions | Recommended** |
| Temperature Range | 23 ± 1.0 °C |
| Light Intensity | 500 - 1000 lux |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 300 mL |
| Replicate Volume | 100 mL water |
| Feeding Regime | 1.5 mL YCT every other day |
| Laboratory Control Water | Moderately hard water prepared in accordance with EPA protocols |
| Minimum Sample Volume | 1L |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, conductivity and temperature measurement per sample and per dilution |
| Initial Unionized Ammonia | One measurement per sample |
| Initial Hardness and Alkalinity | One measurement per sample within 48 h of sample receipt |
| Renewal Water Chemistry | One initial DO, one final DO and one final pH measurement per sample |
| Final Water Chemistry | One DO, pH, and temperature measurement and per sample and per dilution |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 2.5 mg/L - 100% saturation |
| Initial pH Range | 6.0 - 9.0 |
| Conductivity Controls | Include appropriate controls when sample conductivities are 0 – 100, or >10,000 µS/cm |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field; 0 - 6 °C refrigeration in laboratory; dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A14: Measurement Quality Objectives – 96-Hour Survival Hyalella azteca Water Toxicity Test

| Method Recommendation |  |
| --- | --- |
| EPA/821/R-02/012 or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥90% survival in controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static renewal |
| Age at Test Initiation | 7 – 14 days old |
| Replication at Test Initiation | 5 (minimum) |
| Organisms/Replicate | 10 (minimum) |
| Food Source | YCT |
| Renewal Frequency | 80% renewal on Day 2 |
| Test Duration | 10 days |
| Endpoints | Survival |
| Test Conditions | Recommended** |
| Temperature Range | 23 ± 1.0 °C |
| Light Intensity | 500 - 1000 lux |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 300 mL |
| Replicate Volume | 100 mL water |
| Feeding Regime | 1.5 mL YCT every other day |
| Laboratory Control Water | Moderately hard water prepared in accordance with EPA protocols |
| Minimum Sample Volume | 1L |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, conductivity and temperature measurement per sample and per dilution |
| Initial Unionized Ammonia | One measurement per sample |
| Initial Hardness and Alkalinity | One measurement per sample within 48 h of sample receipt |
| Renewal Water Chemistry | One initial DO, one final DO and one final pH measurement per sample |
| Final Water Chemistry | One DO, pH, and temperature measurement and per sample and per dilution |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 2.5 mg/L - 100% saturation |
| Initial pH Range | 6.0 - 9.0 |
| Conductivity Controls | Include appropriate controls when sample conductivities are 0 – 100, or >10,000 µS/cm |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field; 0 - 6 °C refrigeration in laboratory; dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

Table A15: Measurement Quality Objectives – 10-Day Survival Hyalella azteca Sediment Toxicity Test

| Method Recommendation |  |
| --- | --- |
| EPA/600/R-99/064 (Test Method 100.1) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥80% survival and measurable growth in the controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Whole sediment toxicity test with renewal of overlying water |
| Age at Test Initiation | 7 – 14 days old |
| Replication at Test Initiation | 8 (minimum) |
| Organisms/Replicate | 10 |
| Food Source | YCT |
| Renewal Frequency | Twice daily |
| Test Duration | 10 days |
| Endpoints | Survival and growth |
| Test Conditions | Recommended** |
| Temperature Range | 23 ± 1.0 °C |
| Light Intensity | 500 - 1000 lux |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 300 mL |
| Replicate Volume | Sediment volume 100 mL; Overlying water volume 175 mL |
| Feeding Regime | Daily |
| Laboratory Control Water | Moderately hard water prepared in accordance with EPA protocols |
| Sediment Control | Control sediment as listed in method (Control sediment should follow EPA requirements for formulated sediments) |
| Minimum Sample Volume | 6 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, conductivity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Initial Hardness and Alkalinity | One measurement per sample |
| Daily Water Chemistry | One final DO per sample |
| Final Water Chemistry | One DO, pH, and temperature measurement per sample |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 2.5 mg/L - 100% saturation |
| Initial pH Range | 6.0 - 9.0 |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Sediment |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <14 days (recommended) or <8 weeks (required) @ 0 - 6 °C; dark; Do not freeze |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A16: Measurement Quality Objectives – 96-Hour Growth Selenastrum capricornutum  Toxicity Test

| Method Recommendation |  |
| --- | --- |
| EPA/821/R-02/013 (Test Method 1003.0) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | Mean cell density of at least 1 X 106 cells/mL in the controls and variability (CV%) among control replicates less than or equal to 20% (non-EDTA: Mean cell density of at least 2 X 105 cells/mL in the controls; and variability (CV%) among control replicates less than or equal to 20% (required) |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static non-renewal |
| Age at Test Initiation | 4 - 7 days |
| Replication at Test Initiation | 4 (minimum)10,000 cells/mL (recommended) |
| Organisms/Replicate | 10,000 cells/mL (recommended) |
| Food Source | n/a |
| Renewal Frequency | None |
| Test Duration | 96 h |
| Endpoints | Growth |
| Test Conditions | Recommended** |
| Temperature Range | 25 ± 1 °C (+/- 3 C required) |
| Light Intensity | 86 ± 8.6 µE/m2/s OR 400 ± 40 ft-c |
| Photoperiod | Continuous Illumination (“cool white” fluorescent lighting) |
| Test Chamber Size | 125 mL or 250 mL |
| Replicate Volume | 50 mL or 100 mL |
| Feeding Regime | None |
| Nutrient Media | Media prepared in accordance with EPA protocols |
| EDTA Addition | EDTA required per method |
| Laboratory Control Water | Moderately hard water or stock culture medium prepared in accordance with EPA protocols |
| Minimum Sample Volume | 1 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, conductivity and temperature measurement per sample and per dilution |
| Initial Unionized Ammonia | One measurement per sample |
| Initial Hardness and Alkalinity | One measurement per sample within 48 h of sample receipt |
| Daily Water Chemistry | One pH measurement per sample |
| Final Water Chemistry | One DO, pH and temperature measurement and per sample and per dilution |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% saturation |
| Initial pH Range | 6.0 - 9.0 |
| Conductivity Controls | Include appropriate controls when sample conductivities are 0 - 100, or >1500 µS/cm |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A17: Measurement Quality Objectives – 7-Day Survival and Growth Atherinops affinis  Test

| Method Recommendation |  |
| --- | --- |
| EPA/600/R-95/136 (Test Method 1006.0) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥80% survival in controls, 0.85 mg average weight of control larvae (9 days old) |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static renewal |
| Age at Test Initiation | 9 – 15 days post-hatch |
| Replication at Test Initiation | 5 (minimum) |
| Organisms/Replicate | 5 (minimum) |
| Food Source | Newly-hatched Artemia nauplii |
| Renewal Frequency | Daily |
| Test Duration | 7 days |
| Endpoints | Survival and growth (biomass) |
| Test Conditions | Recommended** |
| Temperature Range | 20 ± 1.0 °C |
| Light Intensity | 10 – 20 µE/m2/s OR 50 – 100 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 600 mL |
| Replicate Volume | 200 mL |
| Feeding Regime | Twice daily |
| Laboratory Control Water | Dilution water should be 1-µ filtered natural seawater of hyper-saline brine prepared from uncontaminated natural sweater plus reagent water |
| Minimum Sample Volume | 8 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, Salinity, pH, and temperature measurement per sample and per dilution |
| Initial Unionized Ammonia | One measurement per sample |
| Daily Water Chemistry | One final DO measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement and per sample and per dilution (One DO per renewal) |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% Saturation |
| Initial pH Range | 6.0 - 9.0 |
| Salinity Control | Include appropriate controls if salinity is less than 5 or greater than 36 ppt. |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

Table A18: Measurement Quality Objectives – 10-Day Survival Ampelisca abdita Sediment Toxicity Test

| Method Recommendation≥ |  |
| --- | --- |
| EPA/600/R-94/025 or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥90% survival in the controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Whole sediment toxicity test, static |
| Size at Test Initiation | 3 – 5 mm (no mature males of females) |
| Replication at Test Initiation | 4 (minimum) |
| Organisms/Replicate | 20 (minimum) |
| Food Source | Do not feed |
| Renewal Frequency | None |
| Test Duration | 10 days |
| Endpoints | Survival |
| Test Conditions | Recommended** |
| Temperature Range | 20 ± 1.5 °C |
| Light Intensity | 500 – 1000 lux |
| Photoperiod | Continuous luminance |
| Test Chamber Size | 1 L |
| Replicate Volume | Sediment volume 175 mL; Overlying water volume 800 mL |
| Feeding Regime | Do not feed |
| Laboratory Control Water | Clean, natural seawater diluted to the appropriate salinity with distilled (or similar) water |
| Sediment Control | Control sediment listed in method (Control sediment should follow EPA requirements for formulated sediments) |
| Minimum Sample Volume | 2 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Daily Water Chemistry | One temperature measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 6.45 mg/L - 100% Saturation |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Sediment |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <14 days (recommended) or <8 weeks (required) @ 0 - 6 °C; dark; Do not freeze |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A19: Measurement Quality Objectives – 10-Day Survival Eohaustorius estuarius Sediment Toxicity Test

| Method Recommendation |  |
| --- | --- |
| EPA/600/R-94/025 or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥90% survival in controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Whole sediment toxicity test, static |
| Size at Test Initiation | 3 – 5 mm (no mature males of females) |
| Replication at Test Initiation | 4 (minimum) |
| Organisms/Replicate | 20 (minimum) |
| Food Source | Do not feed |
| Renewal Frequency | None |
| Test Duration | 10 days |
| Endpoints | Survival |
| Test Conditions | Recommended** |
| Temperature Range | 15 ± 1.0 °C |
| Light Intensity | 500 – 1000 lux |
| Photoperiod | Continuous luminance |
| Test Chamber Size | 1 L |
| Replicate Volume | Sediment volume 175 mL; Overlying water volume 800 mL |
| Feeding Regime | Do not feed |
| Laboratory Control Water | Clean, 1-µ filtered natural seawater diluted to the appropriate salinity with distilled (or similar) water |
| Sediment Control | Control sediment listed in method (Control sediment should follow EPA requirements for formulated sediments) |
| Minimum Sample Volume | 2 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Daily Water Chemistry | One temperature measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 6.45 mg/L - 100% Saturation |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Sediment |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <14 days (recommended) or <8 weeks (required) @ 0 - 6 °C; dark; Do not freeze |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A20: Measurement Quality Objectives – 48-Hour Larval Development Haliotis rufescens  Test

| Method Recommendation |  |
| --- | --- |
| EPA/600/R-95/136 (Test Method 995) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥80% normal shell development in the controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static non-renewal |
| Age at Test Initiation | n/a |
| Replication at Test Initiation | 5 (minimum) |
| Organisms/Replicate | 5 – 10 per mL |
| Food Source | Do not feed |
| Renewal Frequency | None |
| Test Duration | 48 h |
| Endpoints | Normal shell development |
| Test Conditions | Recommended** |
| Temperature Range | 15 ± 1.0 °C |
| Light Intensity | 10 µE/m2/s or 50 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 600 mL |
| Replicate Volume | 200 mL or per method |
| Feeding Regime | Do not feed |
| Laboratory Control Water | Dilution water should be 1-µ filtered natural seawater of hyper-saline brine prepared from uncontaminated natural seawater plus reagent water |
| Minimum Sample Volume | 2 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% Saturation |
| Initial pH Range | 6.0 - 9.0 |
| Salinity Control | Include appropriate controls if salinity is less than 32 or greater than 36 ppt |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column, pore water |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A21: Measurement Quality Objectives – 7-DaySurvival and Growth Holmesimysis costata  Test

| Method Recommendation |  |
| --- | --- |
| EPA/600/R-95/136 (Test Method 1007.0) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥75% survival, average dry weight ≥0.40 µg in the controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static renewal |
| Age at Test Initiation | 3 - 4 days post-hatch juveniles |
| Replication at Test Initiation | 5 (minimum) |
| Organisms/Replicate | 5(minimum) |
| Food Source | Newly hatched Artemia nauplii (< 24hours old) |
| Renewal Frequency | 75% renewal at 48hours and 96 h |
| Test Duration | 7 days |
| Endpoints | Survival and growth (biomass) |
| Test Conditions | Recommended** |
| Temperature Range | 15 ± 1.5 °C |
| Light Intensity | 10 – 20 µE/m2/s OR 50 – 100 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 1000 mL |
| Replicate Volume | 200 mL |
| Feeding Regime | Twice per day |
| Laboratory Control Water | Dilution water should be 1-µm filtered natural seawater of hyper-saline brine prepared from uncontaminated natural seawater plus reagent water |
| Minimum Sample Volume | 3 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample and per dilution |
| Initial Unionized Ammonia | One measurement per sample |
| Daily Water Chemistry | One final DO and temperature measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement per sample and per dilution (One DO per renewal) |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% Saturation |
| Initial pH Range | 6.0 - 9.0 |
| Salinity Control | Include appropriate controls if salinity is less than 32 or greater than 36 ppt |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

- **Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A22: Measurement Quality Objectives – 48-hour Embryo-Larval Development Mytilus galloprovinciallis and M. spp. Test

| Method Recommendation |  |
| --- | --- |
| EPA/600/R-95/136 or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥50% survival, ≥90% of those must have normal shell development |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static non-renewal |
| Age at Test Initiation | Within 4hours of fertilization |
| Replication at Test Initiation | 4 (minimum) |
| Organisms/Replicate | 150 – 300 (15-30/mL) |
| Food Source | Do not feed |
| Renewal Frequency | None |
| Test Duration | 48 h |
| Endpoints | Survival of normal live prossidoconch larvae |
| Test Conditions | Recommended** |
| Temperature Range | 15 ± 1.5 °C |
| Light Intensity | 10 – 20 µE/m2/s or 50 – 100 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 20 mL |
| Replicate Volume | 10 mL |
| Feeding Regime | Do not feed |
| Laboratory Control Water | Dilution water should be 1-µm filtered natural seawater of hyper-saline brine prepared from uncontaminated natural seawater plus reagent water |
| Minimum Sample Volume | 1L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Test Parameter | Recommended Criteria |
| Initial DO Range | >4.0 mg/L |
| Initial pH Range | 6.0 - 9.0 |
| Salinity Control | Include appropriate controls if salinity is less than 28 or greater than 36 ppt. |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column, pore water |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

- **Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A23: Measurement Quality Objectives – 72-Hour  Embryo Development Strongylocentrotus purpuratus and Dendraster excentricus Test

| Method Recommendation |  |
| --- | --- |
| EPA/600/R-95/136 or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥80% normal shell development in the controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static non-renewal |
| Age at Test Initiation | Not available |
| Replication at Test Initiation | 4 (minimum) |
| Organisms/Replicate | 250 embryos |
| Food Source | Do not feed |
| Renewal Frequency | None |
| Test Duration | 72 h |
| Endpoints | Normal development; survival can be included |
| Test Conditions | Recommended** |
| Temperature Range | 15 ± 1.0 °C |
| Light Intensity | 10 – 20 µE/m2/s or 50 – 100 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 30 mL |
| Replicate Volume | 10 mL |
| Feeding Regime | Do not feed |
| Laboratory Control Water | Dilution water should be 1-µm filtered natural seawater of hyper-saline brine prepared from uncontaminated natural seawater plus reagent water |
| Minimum Sample Volume | 1 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% Saturation |
| Initial pH Range | 6.0 - 9.0 |
| Salinity Control | Include appropriate controls if salinity is less than 32 or greater than 36 ppt. |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column, pore water |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

- **Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A24: Measurement Quality Objectives – 20-Minute Fertilization Strongylocentrotus purpuratus and Dendraster excentricus Test

| Method Recommenda |  |
| --- | --- |
| EPA/600/R-95/136 or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥70% egg fertilization and appropriate sperm counts in controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static non-renewal |
| Age at Test Initiation | n/a |
| Replication at Test Initiation | 4 (minimum) |
| Organisms/Replicate | ~1,120 eggs from not more than four females and <3,360,000 sperm from not more than four males per test tube |
| Food Source | Do not feed |
| Renewal Frequency | None |
| Test Duration | 40 min (20 min plus 20 min) |
| Endpoints | Fertilization of egg |
| Test Conditions | Recommended** |
| Temperature Range | 12 ± 1.0 °C |
| Light Intensity | 10 – 20 µE/m2/s or 50 – 100 ft-c |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 16 x 100 or 16 x 125 mm |
| Replicate Volume | 5 mL |
| Feeding Regime | Do not feed |
| Laboratory Control Water | Dilution water should be 1-µm filtered natural seawater of hyper-saline brine prepared from uncontaminated natural seawater plus reagent water |
| Minimum Sample Volume | 1 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% Saturation |
| Initial pH Range | 6.0 - 9.0 |
| Salinity Control | Include appropriate controls if salinity is less than 32 or greater than 36 ppt. |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column, pore water |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

- **Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

- Table A25: Measurement Quality Objectives – 48-Hour   Germination and Germ-Tube Length Macrocystis pyrifera Test

| Method Recommendation |  |
| --- | --- |
| EPA/600/R-95/136 (Test Method 1009.0) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥70% germination in the controls, ≥10 µm germ-tube length in the controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static non-renewal |
| Age at Test Initiation | n/a |
| Replication at Test Initiation | 5 |
| Organisms/Replicate | 7500 spores/mL of test solution |
| Food Source | Do not feed |
| Renewal Frequency | None |
| Test Duration | 48 h |
| Endpoints | Germination and germ-tube length |
| Test Conditions | Recommended** |
| Temperature Range | 15 ± 1.0 °C |
| Light Intensity | 50 ± 10 µE/m2/s |
| Photoperiod | 16 hours of ambient laboratory light, 8 hours dark |
| Test Chamber Size | 600 mL |
| Replicate Volume | 200 mL |
| Feeding Regime | Do not feed |
| Laboratory Control Water | Dilution water should be 1-µm filtered natural seawater or hyper-saline brine prepared from uncontaminated natural seawater plus reagent water |
| Minimum Sample Volume | 2 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Test Parameter | Recommended Criteria |
| Initial DO Range | 4.0 mg/L - 100% Saturation |
| Initial pH Range | 6.0 - 9.0 |
| Salinity Control | Include appropriate controls if salinity is less than 32 or greater than 36 ppt. |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | < 48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.

Table A26: Measurement Quality Objectives – 96-Hour Acute Mysidopsis Bahia Survival Test

| Method Recommendation |  |
| --- | --- |
| EPA/821/R-02/012 (Test Method 2007.0) or validated and SWAMP-approved alternative method |  |
| Data Acceptability Requirements |  |
| Parameter | Criteria |
| Test Acceptability Criteria* | ≥90% Mean survival in the controls |
| Data Qualification |  |
| Test Conditions | Required |
| Test Type | Static non-renewal, static-renewal, or flow-through (available options) |
| Age at Test Initiation | 1-5 days; less than or equal to 24-h range in age (required) |
| Replication at Test Initiation | 2 for effluent tests (required minimum) 4 for receiving water tests (required minimum) |
| Organisms/Replicate | 10 for effluent and receiving water tests (required minimum) |
| Food Source | Newly hatched Artemia nauplii (< 24hours old) |
| Renewal Frequency | After 48 h (required minimum) |
| Test Duration | 24, 48, or 96 h (available options) |
| Endpoints | Survival |
| Test Conditions | Recommended** |
| Temperature Range | 20°C ±1°C; or 25°C ±1°C (recommended) Test temperatures must not deviate (i.e., maximum minus minimum temperature) by more than 3°C during the test (required) |
| Light Intensity | 10-20 μE/m2/s (50-100 ft-c) (ambient laboratory levels) (recommended) |
| Photoperiod | 16 h light, 8 h darkness (recommended) |
| Test Chamber Size | 250 mL (recommended minimum) |
| Replicate Volume | 200 mL (recommended minimum) |
| Feeding Regime | Artemia nauplii are made available while holding prior to the test; feed 0.2 mL of concentrated suspension of Artemia nauplii # 24-h old, daily (approximately 100 nauplii per mysid) (recommended) |
| Laboratory Control Water | Dilution water should be 1-µm filtered natural seawater or hyper-saline brine prepared from uncontaminated natural seawater plus reagent water |
| Minimum Sample Volume | 2 L for one-time grab sample |
| Sensitivity | Performance Criteria |
| Reference Toxicant Testing | See Table A9 |
| Water Chemistry |  |
| Test Parameter | Required Frequency |
| Initial Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Initial Unionized Ammonia | One measurement per sample |
| Renewal Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Final Water Chemistry | One DO, pH, Salinity and temperature measurement per sample |
| Test Parameter | Required Frequency |
| Initial DO Range | 4.0 mg/L - 100% Saturation |
| Initial pH Range | 6.0 - 9.0 |
| Conductivity Controls | Include appropriate controls if salinity is less than 32 or greater than 36 ppt. |
| Sample Handling/Collection |  |
| Test Parameter | Recommended Conditions |
| Relevant Media | Water column |
| Sample Container Type | Amber glass |
| Sample Preservation | Wet or blue ice in field, 0 - 6 °C refrigeration in laboratory, dark at all times |
| Sample Receipt Temperature | 0 - 6 °C |
| Holding Time | <48 hours@ 0 - 6 °C; dark |

*Test data are reviewed to verify that the test acceptability criteria (TAC) requirements for a valid test have been met.  Any test not meeting the minimum test acceptability criteria is considered invalid.  All invalid tests must be repeated with the newly collected sample.

**Deviations from the summary of recommended test conditions must be evaluated on a project-specific basis to determine the validity of test results. Deviations from recommended conditions may or may not invalidate a test result, depending on the degree of the departure and the objective of the test.