swamp workplan

California Regional Water Quality Control Board – Region 7

Prepared by

California Regional Water Quality Control Board

Colorado River Basin Region

July 18, 2001

# Introduction

In order to coordinate existing monitoring, the Regional Water Quality Control Board Colorado River Basin Region (Regional Board) will develop a comprehensive workplan that will ultimately cover site-specific monitoring planned for the next five (5) years.  The workplan will be developed in three phases: (1) Listing of the sites or water bodies that are potential reference sites or with suspected problems, (2) specific activities planned for fiscal year 2001-02, and (3) planning for subsequent years.  This workplan focuses on items 1 and 2, and does not currently address item 3.  Furthermore, this workplan is based on projected funding and estimated costs.

# Monitoring Sites

A prioritized list of monitoring locations for fiscal year 2001-02, with either site-specific problems or clean water, is shown in Table 1.

Table 1  Monitoring Sites

|  |  | Potential | Potential |
| --- | --- | --- | --- |
| Sampling Location | Problems | Problems | Reference Site |
| Colorado River at Nevada State Line |  |  | x |
| Colorado River at Parker Dam | B | O, P |  |
| Colorado River at Imperial Dam | B | O, P |  |
| Colorado River above Imperial Dam (USGS station) | B | O, P |  |
| Alamo River Outlet | B, O, P, N, S | P |  |
| Alamo River @ Drop 3 | B, P, N, S |  |  |
| Alamo River @ Drop 6 | B, P, N, S |  |  |
| Alamo R @ Drop 6A | B, P, N, S |  |  |
| Alamo R @ Drop 8 | B, P, N, S |  |  |
| Alamo R @ Drop 10 | B, P, N, S |  |  |
| Alamo River at International Boundary |  | B, O, P, N |  |
| New River Outlet | B, O, P, M, N, S |  |  |
| New River @ Boundary | B, O, P, M, N, S |  |  |
| Salton Sea at County Line | B, P, M, N | O |  |
| 4 Salton Sea Stations | B, P, M, N | O |  |
| Coachella Valley Drain Outlets (Approx 8) | B | P, N |  |
| Palo Verde Outfall Drain | B | P, N |  |
| New River @ Drop 2 | B, O, P, M, N, S |  |  |
| New River @ Evan Hughes Hwy. | B, O, P, M, N, S |  |  |
| Coachella Valley Stormwater Channel Outlet | B | P, N |  |
| Piute Creek |  | B, N |  |
| Lake Havasu |  | B, N |  |
| Taylor Lake |  | B, N |  |
| Ferguson Lake |  | B, N |  |
| Squaw Lake |  | B, N |  |
| Salt Creek |  | B, N |  |
| San Felipe Creek |  | B, N |  |
| Coyote Creek |  | B, N |  |
| Banner Creek |  | B, N |  |
| Borrego Palm Canyon Creek |  | B, N |  |
| Tule Creek |  | B, N |  |
| Carrizo Creek |  | B, N |  |
| Boundary Creek |  | B, N |  |
| Rice Drain | B, P, N, S |  |  |
| Rice Drain #3 | B, P, N, S |  |  |
| Fig Drain | B, P, N, S |  |  |
| Greeson Drain | B, P, N, S |  |  |
| Walker Creek |  | B, N |  |
| Mission Creek |  | B, N |  |
| Big Morongo Creek |  | B, N |  |
| Little Morongo Creek |  | B, N |  |
| Tahquitz Creek |  | B, N |  |
| Azalea Creek |  | B, N |  |
| Twin Pines Creek |  | B, N |  |
| Millard Canyon Creek |  | B, N |  |
| Arrastre Creek |  | B, N |  |
| Crystal Creek |  | B, N |  |
| Antelope Creek |  | B, N |  |
| Copper Basin |  | B, N |  |
| Reservation Main Drain 4 |  | B, N |  |
| Rose Drain | B, P, N, S |  |  |
| Holtville Main Drain | B, P, N, S |  |  |
| Central Drain | B, P, N, S |  |  |
| South Central Drain | B, P, N, S |  |  |
| Verde Drain | B, P, N, S |  |  |
| 2 New River stations (to be determined) | B, P, N, S |  |  |
| All American Canal | B, P, N, S |  |  |
| Whitewater River above MWD outfall |  |  | x |

B = Bacteria, P = Pesticides, O = Organics, M = Metals, N = Nutrients, S = Silt

# Monitoring Objectives

Monitoring objectives have been selected on the basis of those listed in the legislative report and on beneficial uses as designated in the Basin Plan.  These objectives are listed for each site or group of sites that will be monitored in a similar manner.  Adherence to these objectives is based upon funding and sampling costs, which are not available at this time.  These objectives refer to the region as a whole and may include objectives which are not relevant to sites sampled in a given fiscal year.

- Beneficial Use: Water Contact Recreation  “Is the Water Safe to Swim?”

  - At sites influenced by point sources or nonpoint sources of pathogenic contaminants, estimate the concentration of bacteria above screening values, health standards, or adopted water quality objectives.

- Beneficial Use: Municipal and Domestic Water Supply  “Is it Safe to Drink the Water?”

  - At specific locations in lakes, rivers and streams that are sources of drinking water and suspected to be contaminated, estimate the concentration of microbial and chemical contaminants above screening values, drinking water standards, or adopted water quality objectives used to protect drinking water quality.

- Beneficial Use: Sport Fishing  “Is it safe to eat fish and other aquatic resources?”

  - At specific sites influenced by sources of chemical contaminants, estimate the concentration of chemical contaminants in edible aquatic life tissues above advisory levels and critical thresholds of potential human health risk.

  - At frequently fished sites, estimate the concentration of chemical contaminants in commonly consumed fish target species above advisory levels and critical thresholds of potential human health risk (Adapted from USEPA, 1995).

  - At frequently fished sites, verify previous estimates of the concentration of chemical contaminants in commonly consumed fish target species above advisory levels and critical thresholds of potential human health risk (Adapted from USEPA, 1995).

  - Throughout water bodies (streams, rivers, lakes) estimate the concentration of chemical contaminants in fish and aquatic resources from year to year using several critical threshold values of potential human impact (advisory or action levels).

- Beneficial Use: Inland Saline Water Habitats; Preservation of Biological Habitats; Rare, Threatened or Endangered Species; Warm Freshwater Habitat; Wildlife Habitat  “Are aquatic populations, communities, and habitats protected?”

  - At sites influenced by point sources (e.g., storm drains, publicly owned treatment works, etc.) or nonpoint sources of pollutants, identify specific locations of degraded water or sediments in rivers, lakes, streams, and agricultural drains using water quality objective values, water column or benthic community analysis, habitat condition, and chemical concentration.

  - Identify the areal extent of degraded sediment locations in rivers, lakes, streams, and agricultural drains using water quality objective values, water column or benthic community analysis, habitat condition, and chemical concentration.

- Beneficial Use: Reproduction and/or Early Development

  - At sites influenced by point sources (e.g., storm drains, publicly owned treatment works, etc.) or nonpoint sources of pollutants, identify specific locations of degraded water or sediment in rivers, lakes, streams, or agricultural drains using water quality objective values, critical threshold values of early life-stage toxicity, chemical concentration, and physical characteristics.

  - At sites influenced by point sources (e.g., storm drains, publicly owned treatment works, etc.) or nonpoint sources of pollutants, verify previous measurements identifying specific locations of degraded water or sediment in rivers, lakes, streams, or agricultural drains using water quality objective values, critical threshold values of early life-stage toxicity, chemical concentration, and physical characteristics.

- Beneficial Use: Rare, Threatened or Endangered Species; Wildlife Habitat  “Is Water Flow Sufficient to Protect Fisheries?”

  - At specific sites influenced by pollution, estimate the presence of conditions necessary for the survival of aquatic organisms using measures of habitat condition including sedimentation, temperature, and biological communities.

  - At specific sites influenced by pollution, verify previous estimates of the presence of conditions necessary for the survival of aquatic organisms using measurements of habitat condition including sedimentation, temperature, and biological communities.

- Beneficial Use: Non-Contact Water Recreation  “Are Aesthetic Conditions for the Water Protected?”

  - At specific locations in lakes, rivers, and streams, estimate the aesthetic condition above screening values or adopted water quality objectives used to protect non-contact water recreation.

  - At specific locations in lakes, rivers, and streams, verify previous estimates of the aesthetic condition above screening values or adopted water quality objectives used to protect non-contact water recreation.

## General Study Design

### Overview Of General Approach

Monitoring sites have been selected based on investigator pre-selection.  The region will be divided into five watershed areas based on the planning areas listed in the Basin Plan: Lucerne Valley, Hayfield, Coachella Valley, Anza-Burrego/Imperial Valley (Anza-Burrego and Imperial Valley planning areas combined), and the East Colorado River.  The five watershed areas will be monitored for a period of one year each on a rotating basis.  Due to the magnitude of pollutant loading and/or the potential impact of pollutant loading, the major rivers, drains, and lakes (major surface waters) within the region are to be monitored on a regular schedule (i.e. throughout the five year period).

A minimum of quarterly monitoring for the major surface waters will ensure that seasonal variation in both flow and pollutant loading will be accounted.  Likewise, bi-monthly monitoring of the remaining surface waters within each sub-watershed will depict any seasonal variation.  This monitoring schedule allows for the evaluation of variations in magnitude, duration, and frequency of the water quality problem.

Sparse data dictates that spatial characteristics within sub-watersheds will be addressed during each sampling event.  For example, sampling locations for a small stream may vary for each sampling event due to flow conditions.  Sampling locations for major surface waters have been chosen on the basis of land use, location, accessibility, and on the basis of historical sampling sites.

### Water Quality Indicators

Water quality indicators relating to monitoring objectives are listed in Table 2 below.  Indicators corresponding to a particular monitoring objective/beneficial use may not be implemented on every site corresponding to the monitoring objective/beneficial use.  Use of indicators is dependent upon funding, sampling cost, and characteristics of the waterbody.  Monitoring objectives as listed in the Report to the Legislature dated November 30, 2000 are attached to the Appendix.

Table 2.  Water Quality Indicators

| Beneficial Use | Monitoring Objectives | Category | Indicator |
| --- | --- | --- | --- |
| Water Contact | 1 | Contaminant Exposure | Total coliform bacteria |
|  |  |  | Fecal coliform bacteria |
|  |  |  | Enterococcus bacteria |
| Drinking Water | 2 and 3 | Contaminant Exposure | Inorganic water chemistry |
|  |  |  | Nutrients |
|  |  |  | Organic water chemistry |
|  |  |  | Total coliform bacteria |
| Fish Contamination | 3 | Contaminant Exposure | Fish tissue chemistry |
|  |  |  | Fecal coliform bacteria in water |
| Aquatic Life | 4, 5, and 6 | Biological Response | Rapid Bioassessment |
|  |  |  | Chlorophyll |
|  |  |  | Water Toxicity |
|  |  | Pollutant Exposure | Organic & Inorganic Sediment Chemistry |
|  |  |  | Total Organic Carbon |
|  |  |  | Fish tissue chemistry |
|  |  |  | Nutrients |
|  |  |  | Turbidity |
|  |  |  | Inorganic and Organic water chemistry |
|  |  | Habitat | Dissolved oxygen |
|  |  |  | Sediment grain size analysis |
|  |  |  | Sediment organic Carbon |
|  |  |  | Electrical conductivity |
|  |  |  | Salinity |
|  |  |  | Hydrogen Sulfide |
|  |  |  | Ammonia |
| Aesthetic Condition | 7 | Pollutant Exposure | Debris and trash |

# Activities Planned for FY 2001-02

## Sampling Schedule FY 2001-02

A list of water bodies to be sampled in fiscal year 2001-02 is shown in Table 3.  The list is contingent on funding and sampling costs, which are not available at this point in time.

Table 3.  Sampling Locations (FY 2001-02)

Sampling Location

Colorado River at Nevada State Line

Colorado River at Parker Dam

Colorado River at Imperial Dam

Colorado River above Imperial Dam (USGS station)

Lake Havasu

Taylor Lake

Ferguson Lake

Squaw Lake

Palo Verde Outfall Drain

Alamo River Outlet

Alamo River @ Drop 3

Alamo River @ Drop 6

Alamo R @ Drop 6A

Alamo R @ Drop 8

Alamo R @ Drop 10

Alamo River at International Boundary

New River Outlet

New R @ Drop 2

New R @ Evan Hughes Hwy.

New R @ Boundary

Rice Drain

Rice Drain #3

Fig Drain

Greeson Drain

Salton Sea at County Line

CV Drain Outlets

Other Salton Stations

Salt Creek

San Felipe Creek

Coyote Creek

Banner Creek

Borrego Palm Canyon Creek

Tule Creek

Carrizo Creek

Boundary Creek

Walker Creek

Whitewater River above MWD outfall

Coachella Valley Stormwater Channel Outlet

## Review Of Available Information

The Regional Board has historical trend monitoring data over the period of 1983-1993 for various locations on the Alamo River, New River, Colorado River, Salton Sea, Imperial Valley drains, and creeks throughout the region.  Measured parameters include physical parameters, bacteria, some metals, nutrients, and “oxygen” parameters.

The Imperial Irrigation District has some physical and chemical, and flow data for the boundaries and outlets of both the Alamo and New Rivers.

The USGS has historical data on the Colorado, Alamo, and New Rivers.  The parameters measured include flow and various physical and chemical parameters.  The monitoring period varies for each individual station.

Several public agencies have performed various studies on the Salton Sea and its tributaries.  The University of Redlands has compiled a listing of these entities in a report titled “Salton Sea Watershed Water Quality Monitoring Program Inventory and Assessment”.  The report lists the type of data collected and the period in which the data was collected.  A major shortcoming of this report is that it contains no data.  It also indicates that the data is also not readily available.  A further obstacle is that much of the data has no QC plan associated with it, or the data collection and/or analysis methods differ, rendering the data incompatible.

## Specific Sampling Design/Sample Collection

The field crew will collect the samples at sites where the latitude and longitude (and GPS coordinates) were previously recorded during past fieldwork at these stations.  If a new station is being collected, the latitude and longitude, as well as BPS coordinates and cross-referenced photographs shall be provided for the site for future reference.  If there is confusion about locating a site, it shall be resolved in consultation with the RWQCB staff member present in the field or via phone contact.

Sufficient volume of sediment, tissue, or water shall be collected in order to perform the analyses to be conducted at each station, as well as to allow for archiving of samples for future analysis, according to Sampling and analyses schedule for 2001-02 (Table A.1) located in the appendix.  Sample collection and subsequent processing and testing will be performed according to the most recent version of SWAMP QAPP and SOPs (pending Regional Board review).

Sampling schedule, analyses, and costs pertaining to fiscal year 2001-02 are listed in Table A.1, located in the appendix.  Because this table is based on both estimated funding and costs, it should only be considered an approximation of the planned activities for the fiscal year.

## Laboratory Analysis

Specific laboratory analyses to be performed on samples are listed in Table A.1 located in the appendix.  Except for bacteria analyses, all lab work is currently scheduled to be accomplished through DFG.  Bacteria analyses will be performed by a local lab due to holding time constraints.  For all analyses, detection limits, QA/QC criteria, and analytical specific information are included in the QAPP.

As previously stated, Table A.1 is based on estimated funding and costs, and is consequently subject to change.

## Data Quality Evaluation And Data Reporting

QA/QC evaluation reports and verification that data met QA criteria set forth in the QAPP must be provided with hardcopy data report.  In addition, a QA/QC evaluation should be included in the final data report, containing information of how the data complied with the actual QA/QC parameters.

## Deliverable Products

Following is a list of deliverable products:

- Data reports for each sampling event.

- Task orders as needed.

- Final data report.

## Milestones

Scheduled milestones are listed in Table 4.  These milestones are based on estimated costs and a sample throughput of two-weeks.

Table 4.  Scheduled Milestones

|  | Scheduled Completion |
| --- | --- |
| Milestone | Date |
| Alamo River & New River Sampling Event | 7/30/2001 |
| Colorado River & Lakes Sampling Event | 8/15/2001 |
| Alamo Rive & New River Data Report | 8/15/2001 |
| Colorado River & Lakes Data Report | 8/30/2001 |
| Alamo River & New River Sampling Event | 8/30/2001 |
| Alamo River & New River Data Report | 9/15/2001 |
| Alamo River & New River Sampling Event | 9/30/2001 |
| Salton Sea Sampling Event | 9/30/2001 |
| Alamo River & New River Data Report | 10/15/2001 |
| Salton Sea Data Report | 10/15/2001 |
| Alamo River & New River Sampling Event | 10/30/2001 |
| Alamo River & New River Data Report | 11/15/2001 |
| Alamo River & New River Sampling Event | 11/30/2001 |
| Alamo River & New River Data Report | 12/15/2001 |
| Alamo River & New River Sampling Event | 12/30/2001 |
| Colorado River & Lakes Sampling Event | 12/30/2001 |
| Alamo River & New River Data Report | 1/15/2002 |
| Colorado River & Lakes Data Report | 1/15/2002 |
| Alamo River & New River Sampling Event | 1/30/2002 |
| Alamo River & New River Data Report | 2/15/2002 |
| Alamo River & New River Sampling Event | 2/28/2002 |
| Alamo River & New River Data Report | 3/15/2002 |
| Alamo River & New River Sampling Event | 3/30/2002 |
| Salton Sea Sampling Event | 3/30/2002 |
| Colorado River & Lakes Sampling Event | 3/30/2002 |
| Alamo River & New River Data Report | 4/15/2002 |
| Salton Sea Data Report | 4/15/2002 |
| Colorado River & Lakes Data Report | 4/15/2002 |
| Alamo River & New River Sampling Event | 4/30/2002 |
| Anza-Burrego/Imperial Valley Watershed Sampling Event | 4/30/2002 |
| Alamo River & New River Data Report | 5/15/2002 |
| Anza-Burrego/Imperial Valley Watershed Data Report | 5/15/2002 |
| Alamo River & New River Sampling Event | 5/30/2002 |
| Alamo River & New River Data Report | 6/15/2002 |
| Alamo River & New River Sampling Event | 6/30/2002 |
| Colorado River & Lakes Sampling Event | 6/30/2002 |
| Anza-Burrego/Imperial Valley Watershed Sampling Event | 6/30/2002 |
| Alamo River & New River Data Report | 7/15/2002 |
| Colorado River & Lakes Data Report | 7/15/2002 |
| Anza-Burrego/Imperial Valley Watershed Data Report | 7/15/2002 |
| Final Report | 7/30/2002 |

## Sample Throughput Schedule

Sample analysis turn-around time, from time of receipt of sample to time of submission of analytical data, should be no more than two weeks (14 days).

## Budget

With regards to this work plan, the authorized ceiling of expenses is $590,000.  Detailed site-by-site costs associated with services to be conducted are shown in Table A.1 located in the appendix.  Site costs do not include the costs for QC samples, since it is assumed that multiple stations will be collected during the same day, which eliminates the need for QC samples at every station.

Furthermore, these site costs are based on estimated expenses and are therefore subject to change.

# Working Relationships

Table 5. Working Relationships

| Task | Responsible Organization |  |  |
| --- | --- | --- | --- |
|  | SWRCB | RWQCB | DFG |
| Develop contract(s) for monitoring services. |  |  |  |
| Identify water bodies or sites of concern and clean sites to be monitored. |  |  |  |
| Identify site-specific locations with potential beneficial use impacts or unimpacted conditions that will be monitored. |  |  |  |
| Decide if concern is related to objectives focused on location or trends of impacts. |  |  |  |
| Select monitoring objective(s) based on potential beneficial use impact(s) or need to identify baseline conditions. |  |  |  |
| Identify already-completed monitoring and research efforts focused on potential problem, monitoring objective, or clean conditions. |  |  |  |

# Appendix - Other INformation and List Of Attachments

Table A.1 contains the SWAMP sampling locations and cost schedule.  Sampling locations are denoted across the top row, and analytes and prices are shown in the first four columns.  The number of sampling events is located in the body of the table.  The columns to the far right of the table list the number of samples, the number of QC samples, and the total dollar amount for each analysis.  These columns are located in the lower half of the table, as the spreadsheet has been cut in half due to limited space.  Site totals are listed in the bottom row of the table.

Table A.1 Sampling Locations and Cost Schedule

| $620,000.00 |  |  |  | Colorado River Watershed |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Alamo R Watershed |  |  |  |  |  |  | New R Watershed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Colorado River at Nevada State Line | Colorado River at Parker Dam | Colorado River at Imperial Dam | Colorado River above Imperial Dam (USGS station) | Lake Havasu | Taylor Lake | Ferguson Lake | Squaw Lake | Reservation Main Drain 4 | Palo Verde Lagoon (LG1) | Palo Verde Lagoon (PVCO3) | Palo Verde Lagoon (LG3) | Palo Verde Outfall Drain (PVOD1) | Palo Verde Outfall Drain (PVOD2) | Palo Verde Outfall Drain (PVD1) | Alamo River Outlet | Alamo River @ Drop 3 | Alamo River @ Drop 6 | Alamo R @ Drop 6A | Alamo R @ Drop 8 | Alamo R @ Drop 10 | Alamo River at International Boundary | New River Outlet | New R @ Drop 2 | New R @ Evan Hughes Hwy. | New R @ Boundary | Rice Drain | Rice Drain #3 | Fig Drain | Gresson Drain |
| FIELD SAMPLES | Cost |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEDIMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sample collection charge | $570 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| CONVENTIONAL WATER CHEMISTRY | Sediment | Water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nitrates & Nitrite |  | $27 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| TKN |  | 28 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Ammonia |  | $25 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Total Phosphorous |  | $28 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Ortho Phosphates |  | $27 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Organic Phosphorous |  | $28 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Sulfates |  | $27 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Chlorophyll |  | $40 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Alkalinity |  | $15 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| TSS |  | $15 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| BOD |  | $40 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| COD |  | $38 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| TOC |  | $65 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| DOC |  | $65 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| TOC (Sediment) | $65 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  | 1 |  |  |  |  |
| Grain size – full analysis (phl) | $125 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  | 1 |  |  |  |  |
| Bacteria Suite |  | 120 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| VOC's |  | 150 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| TRACE ORGANIC CHEMISTRY (per lab replicate) | Sediment | Tissue | Water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full Scan + PAH (NIST list) | $1,423 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  | 1 | 1 |  | 1 | 1 |  |  |  |  |
|  |  | $1,423 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | $798 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 | 4 |  | 4 | 4 |  |  |  |  |
| Total Petroleum Hydrocarbons (TPH) | $384 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  | 1 | 1 |  |  | 1 |  |  |  |  |
|  |  | $384 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | $210 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  |  |  | 4 | 4 |  |  | 4 |  |  |  |  |
| TRACE METAL CHEMISTRY[1] (per lab replicate) | Sediment | Tissue | Water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: 3 Lab replicates are required for bagged bivalve metal chemistry if end result is to integrate with SMW data (to be comparable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ICP scan: first element (other than Se in the case of sediments) | $245 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  | 1 | 1 |  | 1 | 1 |  |  |  |  |
|  |  | $245 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | $219 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 |  | 1 |  | 1 |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  |  |  |  |
| ICP scan: additional elements beyond first element, per element | $11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | $11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | $11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Se (not done w/ICP for sediments; separate analysis if desired) | $91 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 |  |  |  |  |
| Mercury | $91 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 |  |  |  |  |
|  |  | $91 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | $91 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  | 2 | 2 |  |  |  |  |
| ANALYSIS TYPE | Sediment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAPID BIOASSESSMENT MONITORING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site collection, sorting, taxonomy, QA, and data report (includes 3 replicate samples) | $1,221 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 |  |  |  |  |
|  | ($407 per sample) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sample taxonomy only (for one sample only) for those providing sample to Jim Harrington | $353 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ($353 per sample) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TOXICITY TESTING OF SALT WATER-ORIGIN SAMPLES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TOXICITY TESTING OF FRESHWATER-ORIGIN SAMPLES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water (Fresh Water) Toxicity Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ceriodaphnia 96-h Survival in Fresh Water | $368 |  |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sediment (Fresh Water) Toxicity Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Amphipod 10-d Survival in Fresh Water Sediments (Hyalella) | $840 |  |  | 2 | 2 | 2 | 2 | 2 | 1 | 1 | 1 |  |  |  |  |  | 1 |  | 1 |  |  |  | 1 |  |  | 1 |  |  | 1 |  |  |  |  |
| OTHER TOXICITY TESTING SERVICES[2]/ [3] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OTHER SERVICES | Cost |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MISCELLANEOUS SERVICES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pass-through sub-contract fee payable to Contractor (DF&G) | 15.3% on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1st $50,000[4] | $ 7,650.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cruise Reports (per unique sampling event) | $525 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data Reports (per analytical lab sector/contractor data batch) | $1,575 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Technical Reports, Interpretive Reports, GIS Mapping, etc. | Will be negotiated on case by case basis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LUMP SUM TASK FUNDING FOR STATEWIDE PLANNING ASSISTANCE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Provide statewide planning assistance: develop QAPP, organize scientific/technical workshops, and other assistance as appropriate to SWRCB. | $100,000 | $ 8,600.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Totals |  |  |  | $ 12,219.00 | $ 12,219.00 | $ 12,219.00 | $ 12,219.00 | $ 12,219.00 | $ 11,379.00 | $ 11,379.00 | $ 11,379.00 | $ 6,566.00 | $ 7,687.00 | $ 6,037.00 | $ 6,466.00 | $ 6,037.00 | $ 9,263.00 | $ 7,258.00 | $ 10,433.00 | $ 5,735.00 | $ 5,735.00 | $ 5,735.00 | $ 6,575.00 | $ 5,735.00 | $ 11,819.00 | $ 12,659.00 | $ 3,960.00 | $ 10,405.00 | $ 12,659.00 | $ 3,522.00 | $ 3,522.00 | $ 3,522.00 | $ 3,522.00 |

| 620000 |  |  |  | Salton Sea Watershed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Whitewater River Watershed |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Salton Sea USGS3 | Salton Sea USGS5 | Salton Sea USGS10 | Salton Sea USGS2 | Salton Sea USGS7 | Salton Sea USGS9 | Salton Sea Drain NE1 | Salton Sea Drain NE2 | Salton Sea Drain NW1 | Salton Sea Drain NW2 | Salton Sea Drain SW1 | Salton Sea Drain SW2 | Salton Sea Drain S1 | Salton Sea Drain S2 | Salton Sea Drain S3 | other Salton Station | Salt Creek | San Felipe Creek | Coyote Creek | Banner Creek | Borrego Palm Canyon Creek | Tule Creek | Carrizo Creek | Boundary Creek | Walker Creek | Whitewater River above MWD outfall | Coachella Valley Stormwater Channel Outlet | Mission Creek | Total units | Add 20% for QA/QC | Total Cost (Total Units x cost/unit) |
| FIELD SAMPLES | Cost |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEDIMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sample collection charge | 570 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 59 | 0 | $ 33,630.00 |
| CONVENTIONAL WATER CHEMISTRY | Sediment | Water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nitrates & Nitrite |  | 27 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 6,696.00 |
| TKN |  | 28 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 6,944.00 |
| Ammonia |  | 25 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 6,200.00 |
| Total Phosphorous |  | 28 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 6,944.00 |
| Ortho Phosphates |  | 27 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 6,696.00 |
| Organic Phosphorous |  | 28 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 6,944.00 |
| Sulfates |  | 27 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 6,696.00 |
| Chlorophyll |  | 40 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 9,920.00 |
| Alkalinity |  | 15 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 3,720.00 |
| TSS |  | 15 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 3,720.00 |
| BOD |  | 40 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 9,920.00 |
| COD |  | 38 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 9,424.00 |
| TOC |  | 65 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 16,120.00 |
| DOC |  | 65 |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 206 | 248 | $ 16,120.00 |
| TOC (Sediment) | 65 |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 49 | 59 | $ 3,835.00 |
| Grain size – full analysis (phl) | 125 |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 49 | 59 | $ 7,375.00 |
| Bacteria Suite |  | 120 |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 176 | 212 | $ 21,120.00 |
| VOC's |  | 150 |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 |  |  |  |  |  |  |  |  |  | 2 | 145 | 174 | $ 21,750.00 |
| TRACE ORGANIC CHEMISTRY (per lab replicate) | Sediment | Tissue | Water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full Scan + PAH (NIST list) | 1423 |  |  | 2 | 2 | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 40 | 48 | $ 68,304.00 |
|  |  | 1423 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | $ - |
|  |  |  | 798 | 2 | 2 | 2 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 114 | 137 | $ 109,326.00 |
| Total Petroleum Hydrocarbons (TPH) | 384 |  |  | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  | 1 | 26 | 32 | $ 12,288.00 |
|  |  | 384 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | $ - |
|  |  |  | 210 | 2 | 2 | 2 | 2 | 2 | 2 |  |  |  |  |  |  |  |  |  | 2 | 2 | 2 |  |  |  |  |  |  |  |  |  | 2 | 52 | 63 | $ 13,230.00 |
| TRACE METAL CHEMISTRY [1] (per lab replicate) | Sediment | Tissue | Water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: 3 Lab replicates are required for bagged bivalve metal chemistry if end result is to integrate with SMW data (to be comparable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ICP scan: first element (other than Se in the case of sediments) | 245 |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 39 | 47 | $ 11,515.00 |
|  |  | 245 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | $ - |
|  |  |  | 219 | 2 | 2 | 2 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 123 | 148 | $ 32,412.00 |
| ICP scan: additional elements beyond first element, per element | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | $ - |
|  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | $ - |
|  |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | $ - |
| Se (not done w/ICP for sediments; separate analysis if desired) | 91 |  |  | 2 | 2 | 2 |  |  |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 81 | 98 | $ 8,918.00 |
| Mercury | 91 |  |  | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  | 1 | 32 | 39 | $ 3,549.00 |
|  |  | 91 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | $ - |
|  |  |  | 91 | 4 | 4 | 4 | 4 | 4 | 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 103 | 124 | $ 11,284.00 |
| ANALYSIS TYPE | Sediment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAPID BIOASSESSMENT MONITORING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site collection, sorting, taxonomy, QA, and data report (includes 3 replicate samples) | 1221 |  |  | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 39 | 0 | $ 47,619.00 |
|  | ($407 per sample) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sample taxonomy only (for one sample only) for those providing sample to Jim Harrington | 353 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | $ - |
|  | ($353 per sample) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TOXICITY TESTING OF SALT WATER-ORIGIN SAMPLES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TOXICITY TESTING OF FRESHWATER-ORIGIN SAMPLES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water (Fresh Water) Toxicity Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ceriodaphnia 96-h Survival in Fresh Water | 368 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 35 | 0 | $ 12,880.00 |
| Sediment (Fresh Water) Toxicity Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Amphipod 10-d Survival in Fresh Water Sediments (Hyalella) | 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 27 | 0 | $ 22,680.00 |
| OTHER TOXICITY TESTING SERVICES [2]/ [3] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OTHER SERVICES | Cost |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MISCELLANEOUS SERVICES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pass-through sub-contract fee payable to Contractor (DF&G) | 15.3% on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1st $50,000[4] | 7650 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | $ 7,650.00 |
| Cruise Reports (per unique sampling event) | 525 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | $ 525.00 |
| Data Reports (per analytical lab sector/contractor data batch) | 1575 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | $ 1,575.00 |
| Technical Reports, Interpretive Reports, GIS Mapping, etc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LUMP SUM TASK FUNDING FOR STATEWIDE PLANNING ASSISTANCE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Provide statewide planning assistance: develop QAPP, organize scientific/technical workshops, and other assistance as appropriate to SWRCB. | 100000 | 8600 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | $ 8,600.00 |
| Site Totals |  |  |  | $ 10,959.00 | $ 10,959.00 | $ 10,619.00 | $ 10,573.00 | $ 9,150.00 | $ 9,150.00 | $ 7,355.00 | $ 7,355.00 | $ 7,355.00 | $ 7,355.00 | $ 7,355.00 | $ 7,355.00 | $ 7,355.00 | $ 7,355.00 | $ 7,355.00 | $ 9,063.00 | $ 9,063.00 | $ 9,253.00 | $ 8,530.00 | $ 8,530.00 | $ 8,530.00 | $ 8,530.00 | $ 8,530.00 | $ 8,530.00 | $ 8,530.00 | $ 8,530.00 | $ 7,400.00 | $ 10,577.00 |  |  | $ 576,129.00 |

Attachment 1.   List of Indicators (Report to Legislature, November 30, 2000)

(To be attached)

List of Indicators

Monitoring programs sponsored by the SWRCB and the RWQCBs have used a variety of environmental indicators.  Indicators that have been used in ambient monitoring efforts and meet the requirements of the general criteria are presented in Table 3.  These indicators are considered a starting point for the indicators which should be used in the State’s ambient monitoring efforts.

Table 3:  List of indicators for Site-Specific and Regional Monitoring

| Beneficial Use | Monitoring Objectives |  | Category | Indicator |
| --- | --- | --- | --- | --- |
|  | Site-Specific | Regional |  |  |
| Water Contact | 1 | 1, 2, and 3 | Contaminant exposure | Total coliform bacteria Fecal coliform bacteria Enterococcus bacteria Enteric viruses |
| Drinking Water | 2 and 3 | 4 and 5 | Contaminant exposure | Inorganic water chemistry Nutrients Organic water chemistry Total coliform bacteria Cryptosporidum Giardia |
| Fish and Shellfish Contamin-ation | 4, 5, 6, 7, and 8 | 6, 7, 8, 9 and 10 | Contaminant exposure | Fish tissue chemistry Shellfish tissue chemistry Coliform bacteria in shellfish Fecal coliform bacteria in water |
| Aquatic Life | 9, 10, 11, 12, and 13 | 11, 12, 13, 14, 15, 16, and 17 | Biological response2 | Phytoplankton Chlorophyll-a Benthic infauna (Animals that live in sediment.) Fish assemblage Fish pathology Recruitment of sensitive life stages Interstitial water toxicity Macroinvertebrate assemblage Periphyton Sediment toxicity Water toxicity |
|  |  |  | Pollutant exposure | Acid volatile sulfides/simultaneously extracted metals Debris Interstitial water metal chemistry Reporter Gene System (RGS 450) Organic and inorganic sediment chemistry Total organic carbon Shellfish or fish tissue chemistry Nutrients Turbidity Inorganic and organic water chemistry |
|  |  |  | Habitat | Dissolved oxygen Sediment grain size Sediment organic carbon Water flow Water temperature Channel morphology Residual pool volume Instream structure Substrate composition Wetland vegetation Riparian vegetation Electrical conductivity Salinity Hydrogen sulfide Ammonia |
| Sufficient Flow | 14 and 15 | 18 and 19 | Habitat | Water flow Suspended solids Channel morphology Water temperature |
|  |  |  | Biological response | Fish assemblage and populations Macroinvertebrate assemblage and populations Periphyton Wetland habitat Riparian habitat |
| Agricultural Supply | 16 and 17 | 20 and 21 | Pollutant Exposure | Organic and inorganic chemistry |
| Industrial Supply | 18 and 19 | 22 and 23 | Pollutant Exposure | Organic and inorganic chemistry Total organic carbon Temperature Electrical conductivity |
| Aesthetic Condition | 20 and 21 | 24 and 25 | Pollutant Exposure | Taste and odor Debris and trash |

Adapted from:  SWRCB, 1993; SPARC, 1997; SCCWRP, 1998; Stephenson et al., 1994; CalEPA, 1998; CABW, 1998; CDFG, 1998; Noble et al., 1999; AB 982 Scientific Advisory Group, personal communication, August, 2000.

The number refers to the monitoring objective discussed previously under site-specific and regional monitoring approaches.

2 While the assessment of invasive species is not a focus of SWAMP, these organisms will very likely be identified when biological community measurements are made.