North Coast Regional Water Quality

Control Board

##### Surface Water Ambient

Monitoring Program

###### Workplan for FY 2001-02

October, 2001

Table of Contents

I.	Introduction										1

II.	Sites to be Monitored									4

III.	Site Identification, Monitoring Activities, and Objectives				5

Smith River Hydrologic Unit (103)						6

Klamath River Hydrologic Unit (105)						7

Trinity River Hydrologic Unit (106)						11

Redwood Creek Hydrologic Unit (107)						14

Trinidad Hydrologic Unit (108)							15

Mad River Hydrologic Unit (109)						16

Eureka Plain Hydrologic Unit (110)						17

Eel River Hydrologic Unit (111)							20

Mendocino Coast Hydrologic Unit (113)						24

Russian River Hydrologic Unit (114)						25

- Specific Sample Design and Sample Collection

Wadeable Stream and River Sites						29

Lake Sampling for MTBE							29

Vitellogenin Screening								30

Stream Gaging									30

Laboratory Analysis								31

Data Quality Evaluation and Data Reporting					31

Deliverable Products								31

Desired Milestone Schedule							32

Desired Sample Throughput Schedule						32

Budget										32

- Working Relationships/Decision Matrix						33

- Other Information and Attachments							33

# I  Introduction

The Porter-Cologne Water Quality Control Act and the federal Clean Water Act (CWA) direct the water quality programs to implement efforts intended to protect and restore the integrity of waters of the State. California Assembly Bill (AB) 982 (Water Code Section 13192; Statutes of 1999) requires the State Water Resources Control Board (SWRCB) to assess and report on the State monitoring programs and to prepare a proposal for a comprehensive surface water quality monitoring program.  Ambient monitoring is independent of the water quality programs and serves as a measure of (1) the overall quality of water resources and (2) the overall effectiveness of Regional Water Quality Control Boards’ (RWQCBs’) prevention, regulatory, and remedial actions.

Pursuant to this directive, the SWRCB has developed the Surface Water Ambient Monitoring Program (SWAMP).  SWAMP is a new and comprehensive program which will (1) integrate the existing water quality monitoring of the SWRCB and RWQCBs and (2) coordinate with monitoring programs of other agencies, dischargers, and citizens groups.

## Overview of the Surface Water Ambient Monitoring Program (SWAMP)

## SWAMP Program Goals

SWAMP is intended to meet four goals as follows:

- Create an ambient monitoring program that addresses all hydrologic units of the State using consistent and objective monitoring, sampling and analytical methods; consistent data quality assurance protocols; and centralized data management. This will be an umbrella program that monitors and interprets those data for each hydrologic unit at least one time every five years.

- Document ambient water quality conditions in potentially clean and polluted areas. The scale for these assessments ranges from the site-specific to statewide.

- Identify specific water quality problems preventing the SWRCB, RWQCBs, and the public from realizing beneficial uses of water in targeted watersheds.

- Provide the data to evaluate the overall effectiveness of water quality regulatory programs in protecting beneficial uses of waters of the State.

The Surface Water Ambient Monitoring Program (SWAMP) is a combination of (1) regional monitoring to provide a picture of the status and trends in water quality and (2) site-specific monitoring to better characterize problem and clean locations. This approach balances these two important monitoring needs of the SWRCB and serves as a unifying framework for the monitoring activities being conducted by the SWRCB and RWQCBs. The coordinated SWRCB and RWQCB involvement in study design and sampling is critical to providing a comprehensive, effective monitoring

Site-Specific Monitoring

The overall goal of this portion of SWAMP is to develop site-specific information on sites that are (1) known or suspected to have water quality problems and (2) known or suspected to be clean. It is intended that this portion of SWAMP will be targeted at specific locations in each region, and focus on collecting information from sites in water bodies of the State to support remedial actions as well as the potential listing or delisting under Clean Water Act Section 303(d).

In addition to the SWAMP effort, there is another closely coordinated assessment effort in place in the North Coast Region called The North Coast Watershed Assessment Program (NCWAP) that will provide valuable assessment information to resource managers and to the Regional Board.

The North Coast Watershed Assessment Program (NCWAP) is a multi-agency effort led by the California Resources Agency. The principal goal of NCWAP is to compile and develop baseline scientific information about existing biophysical conditions in north coast watersheds. The Resources Agency initiated this program in response to requests from landowners, industry and environmental groups, watershed groups, a Science Review Panel on Forest Practices, and other stakeholders that the state take a leadership role in conducting interdisciplinary watershed assessments which could be used for guiding decision making. The goals of this program are:

- Provide a baseline of data for evaluating the effectiveness of various resource protection programs over time;

- Guide watershed restoration programs, e.g., targeting grant dollars to those projects that most efficiently and effectively recover salmonid populations, and assisting local watershed groups, counties, etc., to develop successful projects;

- Guide cooperative interagency, nonprofit and private sector approaches to “protect the best” through stewardship, easement and other incentive programs;

- Help landowners and agencies implement laws that require specific assessments such as the State Forest Practice Act, Clean Water Act and State Lake and Streambed Alteration Act.

The NCWAP is on a schedule to assess approximately one million acres a year for a 7-year period.  Being largely dependent on existing information, NCWAP is closely integrated with SWAMP in both timing and site location.

The SWAMP uses a two-component approach to address regional and site-specific monitoring: 1) long-term monitoring sites for trend analysis, and 2) rotating intensive basin surveys. The rotation schedule is closely coordinated with the NCWAP assessment schedule, to provide additional and current information on water quality parameters to the NCWAP assessment.

II Sites to be Monitored

## North Coast Region – Description and Water Quality Issues

The North Coast Region (NCR), comprises all basins draining into the Pacific Ocean from the California -Oregon state line (including Lower Klamath Lake and Lost River Basins) southerly to the southern boundary of the watershed of the Estero de San Antonio and Stemple Creek in Marin and Sonoma Counties.  The North Coast Region covers all of Del Norte, Humboldt, Trinity, and Mendocino Counties, major portions of Siskiyou and Sonoma Counties, and small portions of Glenn, Lake, and Marin Counties. The North Coast Region encompasses a total area of approximately 19,390 square miles, including 340 miles of scenic coastline and remote wilderness areas, as well as urbanized and agricultural areas.

Distinct temperature zones characterize the North Coast Region. Along the coast, the climate is moderate and foggy and the temperature variation is not great. For example, at Eureka, the seasonal variation in temperature has not exceeded 63 F for the period of record. Inland, however, seasonal temperature ranges in excess of 100 F have been recorded.

Precipitation over the North Coast Region is higher than for any other part of California, and damaging floods are a fairly frequent hazard. Particularly devastating floods occurred in the North Coast area in December of 1955, in December of 1964, and in February of 1986. Ample precipitation in combination with the mild climate found over most of the North Coast Region has provided a wealth of fish, wildlife, and scenic resources.

The mountainous nature of the Region, with its dense coniferous forests interspersed with grassy or chaparral covered slopes, provides shelter and food for deer, elk, bear, mountain lion, furbearers and many upland bird and mammal species. The numerous streams and rivers of the Region contain anadromous fish, and the reservoirs, although few in number, support both coldwater and warmwater fish.

Tidelands, and marshes too, are extremely important to many species of waterfowl and shore birds, both for feeding and nesting.  Cultivated land and pasturelands also provide supplemental food for many birds, including small pheasant populations.  Tideland areas along the north coast provide important habitat for marine invertebrates and nursery areas for forage fish, game fish, and crustaceans.  Offshore coastal rocks are used by many species of seabirds as nesting areas.

Major components of the economy are tourism and recreation, logging and timber milling, aggregate mining, commercial and sport fisheries, sheep, beef and dairy production, and vineyards and wineries.

Regional Water Quality Issues

The North Coast Region faces several water quality issues. The highest priority water quality problems include contamination of surface water due to nonpoint source pollution from storm water runoff, erosion and sedimentation (roads, vineyards, grazing and timber harvest), channel modification, gravel mining and dairies, and MtBE and dioxin contamination. Ground water contamination from leaking underground tanks and health and safety issues from contaminated areas that are open to the public are also priority issues. Flow in rivers and streams is becoming an issue, as is the use of agricultural chemicals in vineyards.  High priority water quality problems due to point sources include chronic violations by POTWs and lack of permit compliance.  Lack of funding for water quality monitoring and watershed assessment compounds the difficulty of addressing these issues.

The highest priority activities to address those problems include:

protect and restore water quality and beneficial uses

maintaining the core regulatory program for regulated dischargers

increasing emphasis on storm water runoff issues

increasing monitoring and assessment activities

increasing emphasis on nonpoint source issues (including forestry and agriculture), especially as they affect salmonid resources

developing and implementing Total Maximum Daily Load strategies (mostly sediment and temperature associated with salmonid resource declines)

improving outreach and community involvement in decisions

fostering watershed groups and volunteer monitoring

ensuring prompt and appropriate enforcement

With the exception of the Russian River watershed and the Klamath River Basin in California, we have very little data on surface water quality in most of our watersheds.  The general driving force for our SWAMP monitoring program for this and other fiscal years will be to collect water quality assessment data at permanent stations established throughout our region and to gather more intensive information using the rotating basin approach.

## III Site Identification ,Monitoring Activities and Objectives

This section of the workplan will address site identification organized by Basin Plan Hydrologic Unit Code (HUC) and further referenced to Watershed Management Areas as identified in the North Coast Regional Water Quality Control Board Watershed Planning Chapter.

For each HUC a table is presented detailing the beneficial uses to be protected and the monitoring objectives for that particular site or station.  Monitoring frequency and water quality indicators used for that site are also enumerated.

For a complete description of the indicators used, please refer to the Report to the Legislature November 30, 2000, Attachment A.

Smith River Hydrologic Unit (103).

The Smith River is within the larger North Coast Rivers Watershed Management Area (WMA) within the Watershed Planning Chapter.

Under the Federal Endangered Species Act (ESA), the Smith River watershed is wholly contained in the Northern California Coast Evolutionary Significant Unit (ESU) for Coho salmon.  As such, it is designated as critical habitat for Coho salmon, listed as “threatened” under the ESA in 1997.

The Smith River Plains area has not been comprehensively sampled for pesticides for nearly a decade and several new chemicals are in use. Expansion of agricultural practices such as lily fields and floral greenhouses has occurred at a rapid rate over the past several years and investigation of the impacts of pesticide and fertilizer use on surface and ground water is warranted. There is also a concern about dairies and the impact of manure on the lower Smith River.

Surface Water Ambient Monitoring Program Monitoring Stations for the Smith River Hydrologic Unit (103)

| Smith River Hydrologic Unit (103) – FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category | Indicator(s) (4) |
| SMHFIS (P) 103.11 (Smith River – d/s Dr. Fine Bridge) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,5,9,10,11,12,13 | 3 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| SMHSFK (P) 103.20 (Smith River – South Fork above Hiouchi) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,5,9,10,11,12,13 | 3 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| SMHMAN (P) 103.20 (Smith River – u/s South Fork) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,5,9,10,11,12,13 | 3 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Klamath River Hydrologic Unit (105).

Klamath River Hydrologic Unit as addressed in Region One’s WMI Chapter as The Klamath River Watershed Management Area (most of that portion of the overall Klamath River Basin which is within the State of California), has been divided into three sub-basins: Lower Klamath, Middle Klamath and Upper Klamath.  This division helps us recognize that the size of the overall basin, and its diversity in climatic and geologic facets and land uses affect water quality in different ways in different sub-areas of the basin. In addition to this for-convenience segmentation of the watershed area within California, we recognize that roughly half of the watershed is north (and mostly upstream) of the California -Oregon state border. This “segment” of the basin in Oregon has profound effects on the quality and quantity of the Klamath River in California.  Each sub-basin is described below:

The Lower Klamath sub-basin encompasses that portion of the Klamath River and its tributary watershed downstream from the Scott River to the Pacific Ocean (excluding the Trinity River), and is 2,564 square miles in area. Included in the watershed are the Salmon River, Blue Creek, numerous smaller perennial streams, and the Klamath River delta/estuary. The area is largely rugged, steep forest land with highly erodable soils. The population of the area is small and scattered. Water quality issues have arisen as a result of unauthorized discharges or inadequately treated residential sewage. Current water quality issues in the sub-basin are related to the salmonid-habitat qualities of the mainstem river and the effects of silvicultural activities on both federal and private lands to the tributaries. These issues include high summertime temperatures, sedimentation, erosion, mass wasting and stream modifications which affect salmonid habitats, and forest land herbicide applications which threaten domestic water supplies.

Under the Federal Endangered Species Act (ESA), the Lower Klamath sub-basin is wholly contained in the Southern Oregon/Northern California Coast Evolutionary Significant Unit (ESU) for Coho salmon.  As such, it is designated as critical habitat for Coho salmon, listed as “threatened” under the ESA in 1997.

The Middle Klamath sub-basin is 2,850 square miles in area and encompasses that portion of the Klamath River and tributaries between the confluence of the Klamath and Scott Rivers and Iron Gate Dam. Included in the watershed are the mainstem Klamath, the Shasta and Scott River watersheds and lesser tributaries. The two major tributaries, the Shasta and Scott Rivers, receive localized precipitation as well as snow and glacial melt from nearby mountain ranges. The quality of water from Iron Gate reservoir (which is the sum total of the effects of reservoir limnology, up-river irrigation development and hydropower hydrology), agriculture in the Shasta and Scott Valleys and silvicultural activities in the remainder of the drainage are the major issues. Other water quality issues are related to surface water and ground water contamination from toxic chemical discharges in the Weed and the Yreka areas.

Under the Federal Endangered Species Act (ESA), the Middle Klamath sub-basin is wholly contained in the Southern Oregon/Northern California Coast Evolutionary Significant Unit (ESU) for Coho salmon.  As such, it is designated as critical habitat for Coho salmon, listed as “threatened” under the ESA in 1997.

The Upper Klamath sub-basin includes watershed areas in California that are upstream of Iron Gate Dam. Many natural and human-altered watershed elements above Iron Gate and across the California -Oregon border affect the quality and quantity of water which exits Iron Gate Dam, supplies the mainstem flow, and affects (both supports and jeopardizes) the beneficial uses of the River within California. The complexity of this sub-basin is magnified by jurisdictional issues associated with water-delivery/utilization infrastructures (including the Federal Klamath Project irrigation), hydropower, endangered species, tribal rights, lake-level-management demands for Upper Klamath Lake, the waters criss-crossing the California -Oregon border, and minimum flow requirements in the Klamath below Iron Gate Dam.

Most of the Upper Klamath watershed area is in Oregon. The primary sub-watershed in California is the Lost River watershed, which is 1,689 square miles in area. That sub-watershed, which is about half-and-half in California and Oregon, encompasses Clear Lake Reservoir and most of its tributaries in California, the agricultural and contributing areas in Oregon, and back in California, the agricultural and wildlife-refuge areas which were once the bottom of Tule Lake and the Lower Klamath Lake. The Lost River basin was, until Euroamerican settlement and development including farmland “reclamation” and construction of the railroad, periodically connected to the Klamath River via the marshes which occurred south of what is now the community of Klamath Falls, Oregon. Further south, the marsh-river systems dead-ended in Tule Lake that was a closed part of the basin with no natural outlet. The lower end of this basin has been modified to support agricultural crop production, and consequently an artificial outlet has been provided for Lost River water to be pumped into Lower Klamath Lake. Lower Klamath Lake was originally a backwater of the Klamath River, but has been extensively modified for agriculture and a wildlife refuge. Water leaving that system is discharged northward, back into Oregon to the Klamath River, via the Klamath Straits Drain. Much of the former wetlands in the basin are now intensively managed for wildlife as part of the Klamath Basin National Wildlife Refuges, with mingled and overlapping cropping and wildlife uses.

Primary beneficial uses in the upper Klamath basin are domestic, agricultural and industrial water supply, cold and warm water fisheries, and recreation. The shortnosed sucker (Chamistes brevirostris) and Lost River sucker (Deltistes luxatus), native to the watershed, are listed as endangered under the federal Endangered Species Act of 1973.

Monitoring priorities and needs detail for the Klamath WMA

The Surface Water Monitoring Program (SWAMP) will rotate intensive surveys into the Klamath WMA in FY 2002-03. The intensive survey will focus on overall assessment of water quality in the WMA, and address assessment of known problem areas.

Additional assessment by Regional Water Board staff is needed to test hypotheses about support of beneficial uses MUN, REC1, COLD, RARE, or provide assessment information essential for program implementation.

- Nutrient and Eutrophication Studies

An intensive nutrient, temperature and dissolved oxygen monitoring and assessment program was funded for two years on the upper and middle Klamath River. The effort continues with some 205(j) funds, and by other agencies and entities in the upper and middle Klamath River without significant involvement by Regional Water Board staff. We should be collecting data specific to our needs for TMDL development and implementation of nonpoint source controls. We have established four permanent stations in the upper and middle Klamath in FY 2000-01.  The intensive survey planned for FY 2002-03 will focus more intensively on problem areas.

- Sedimentation –

The Scott River watershed is 303(d) listed for sediment impacts. Assessment of sediment sources and impacts is needed to assist in developing a TMDL sedimentation reduction strategy for the watershed. A Section 205(j) project with the Siskiyou RCD is evaluating sediment sources in Moffett Creek. Additional assessment is needed in the lower Klamath River tributaries (Terwer, Blue, High Prairie, Hunter)  For FY 01-02 we have established one permanent and four rotating stations on the Scott River.

- Lake Shastina Toxics

While cleanup activities continue on Beaughton and Boles creeks to eliminate metals, dioxins, and MtBE contamination, new sources have been identified. Additional assessment is needed to determine the extent of the problem in the tributaries and Lake Shastina.  For FY01-02 we have established three stations in the Shasta River, at the Highway 3 crossing, at the Highway 263 crossing and near Big Springs, downstream from Parks Creek to address toxics as well as nutrient issues (#1 above).

- Chemicals in POTWs

Petroleum products, including solvents, MTBE, and gasoline, as well as pesticides should be sampled in the influent and effluent of POTWs.  The SWAMP rotation in FY 2002-03 will address this need.

- Pesticides in Tulelake

The US Geological Survey assessed pesticides in the basin some years ago. Additional assessment keyed to current agricultural chemical use should be performed. The SWAMP rotation in FY 2002-03 may be able to partially address this need.

- Yreka Creek Petroleum

While groundwater contamination from solvents and other petroleum products are documented and being addressed to varying degrees, contamination of Yreka Creek from contiguous ground waters is a concern. To date no significant problems have been identified, however it remains a concern.

The SWAMP has addressed some monitoring issues in the WMA in FY 2000-01, and and will continue to do so for FY 2002-03.  A more intensive investigation is planned for the rotation into the WMA in FY 2002-03. Listed below are the planned and proposed monitoring activities under the program:

Long-term monitoring stations: Seven long-term stations will be maintained in the WMA as initiated in FY 2000-01: Klamath River at Klamathon, near Empire Creek, near Horse Creek, at Seiad Valley, at Weitchpec; Shasta River at the mouth; Scott River at the mouth. Other long-term stations in the WMA will be proposed if appropriate from the rotation in FY 2003-04.

Surface Water Ambient Monitoring Program Monitoring Stations for the Klamath River Watershed Management Area – HUC 105

| Klamath River Hydrologic Unit (105) - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| KLAEVC (P) 105.00 (Klamath River below Horse Creek) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| KLARMP (P) 105.00 (Klamath River at Gottville River Access) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| KLAMCO (P) 105.00 (Klamath River below Iron Gate) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| KLAMWP (P) 105.12 (Klamath River at Weitchpec) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| KLAMSI (P) 105.33 (Klamath River at Seiad Valley) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| SCOTSH (P) 105.40 (Scott River at Steel Head) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| SCOTFJ (R) 105.42 (Scott River at Fort Jones) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| SCOTET (R) 105.42 (Scott River u/s of Etna Creek) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| SCOTCA (R) 105.42 (Scott River at Callahan) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| SHASYR (P) 105.50 (Shasta River at Highway 263) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| SHAHW3 (R) 105.50 (Shasta River at Highway 3) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| SHASBS (R) 105.50 (Shasta River nr Big Springs, d/s Parks Creek) | MUN,REC1,REC2,RARE, COLD,SPWN,MIGR,WILD | 1,2,3,9,10,11,12,13 | 5 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Trinity River Hydrologic Unit (106).

The Trinity River comprises the Trinity WMA as described in the Watershed Planning Chapter.

The Trinity River, a wild and scenic river located in northwestern California, is the largest tributary to the Klamath River. Its basin drains an area of about 2900 square miles of mountainous terrain, with its headwater streams originating in the Klamath and Coast Ranges. From its headwaters, the river flows 172 miles south and west through Trinity County, then north through Humboldt County and the Hoopa Valley and Yurok Indian reservations. The confluence with Klamath River at Weitchpec is about 43 miles upstream from the Pacific Ocean. In the early 1950’s two major water-development features: Lewiston Dam and its reservoir and related facilities and Trinity Dam and its reservoir, known as Trinity Lake, which are jointly known as the Trinity River Division of the Bureau of Reclamation’s Central Valley Project (CVP) were installed above River-Mile 112 and the community of Lewiston. Water stored and released from the Trinity Dam reservoir is used for power generation and diverted to out-of-Basin multiple uses throughout the Central Valley of California.

Since the installation of the Trinity River Division (TRD) works, the Lewiston Dam is the uppermost limit of natural salmon and steelhead fish-migration. A fish hatchery and rearing facilities were constructed and operate as part of the TRD to mitigate for the loss of upstream habitat. Trinity Lake has been stocked with a variety of nonnative fish, including Smallmouth and Largemouth bass and Kokanee (landlocked Sockeye salmon). Trinity River downstream of TRD is habitat for not only the anadromous salmonids and other native species, but also has populations of brown trout.

Under the Federal Endangered Species Act (ESA), the Trinity River downstream of the Lewiston Dam is wholly contained in the Southern Oregon/Northern California Coast Evolutionary Significant Unit (ESU) for Coho salmon.  As such, it is designated as critical habitat for Coho salmon, listed as “threatened” under the ESA in 1997.

The public lands that adjoin the TRD facilities are managed for multiple uses as part of the Whiskeytown-Shasta-Trinity National Recreation Area; those in upper portions of the basin are managed as components of the US Shasta -Trinity and Six Rivers National Forests. Private timberlands, ranches and residential properties are mostly near the Highway 3-Highway 299 corridors in the southeastern part of the basin. The Hoopa Valley Reservation occupies about 170 square miles on both sides of the lowest 15 miles of the river.

This WMA is mostly rural with human population centered near Trinity Center, Weaverville,  Lewiston, Hayfork and Hyampom. The only large-scale agriculture is cattle grazing. Timber harvest continues but at a much reduced level than in the past on Federal lands. However, the intensity and scope of logging appears to be increasing in private lands. Toxicity concerns center around acid mine drainage from abandoned mines and past mining activities, sediment release from subdivision development and eroded roads in areas with unstable soil and decomposed granite, septic tank use, aboveground and underground tanks, and lumber mills. The U.S. Forest Service and the Bureau of Land Management federally manage approximately 80 percent of the land in the Trinity WMA. Of the remaining 20 percent of the basin, which is privately owned, approximately half are industrial timberlands. Old existing access roads that are not maintained or properly decommissioned are a continual source of sedimentation into the Trinity River and its tributaries. Tourism including boating and rafting, is q significant part of the economy of this area.

Water Quality Goals and Actions

The broad goals for the Trinity River WMA include improving the anadromous fishery through sediment reductions and habitat enhancements and maintaining the other high beneficial uses of both surface and ground water.

Three specific goals for the Trinity River were identified in the Watershed Planning Chapter and are related through the beneficial uses they address:

GOAL 1: Protect and enhance salmonid resources (COLD, MIGR, SPWN, RARE)

GOAL 2: Protect and enhance ground water resources and attendant beneficial uses

GOAL 3: Protect all other surface water uses

The protection of cold water fisheries (Goal 1) requires the protection of surface water (Goal 3)

and ground water (Goal 2) along with additional concerns for siltation, habitat loss, temperature

and low tributary flows. Actions for protecting the beneficial uses for Goal 1 (COLD) largely

serve to protect all other uses, except MUN.

Surface Water Monitoring Program Monitoring Stations for the Trinity River Watershed Management Area - HUC 106

| Trinity River Hydrologic Unit (106) - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| TRINHP (R) 106.11 (Trinity River at Hoopa) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| TRINWP (P) 106.11 (Trinity River at Weitchpec) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,5,9,10, 11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| TRINSL (R) 106.13 (Trinity River at Salyer) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| TRNNFH (R) 106.15 (Trinity River – North Fork at Helena) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| TRINSF (R) 106.21 (Trinity River – South Fork near Salyer) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| TRINDC (R) 106.31 (Trinity River at Douglas City) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| TRINPB (R) 106.31 (Trinity River at Poker Bar – d/s Grass Valley Creek) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| TRHTCH (P) 106.32 (Trinity River at Lewiston) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,5,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Redwood Creek Hydrologic Unit (107).

Redwood Creek comprises a portion of the Humboldt Bay Watershed Management Area and is largely National Park land in the lower section of the watershed and along the main stem. However, private industrial timberland comprises a significant portion of the upper watershed and tributary areas. Redwood Creek supports production of anadromous salmonids, including steelhead and cutthroat trout, and coho and chinook salmon. It appears that sedimentation has moved into the lower part of the watershed from past activities in the upper watershed.  The National Park staff are conducting assessments of documented problem areas and follow-up coordination for implementing controls is being conducted. A Section 303(d) Water Quality Attainment Strategy (“TMDL”) will build upon the existing efforts to coordinate activities in the watershed to benefit enhancement of the salmonid resources. Redwood Creek is also a NCWAP assessment watershed for the calendar year 2001.

Under the Federal Endangered Species Act (ESA), the Redwood Creek is wholly contained in the Northern California Evolutionary Significant Unit (ESU) for Steelhead, listed as “threatened” under the ESA in 2000.  The National Marine Fisheries Service (NMFS) is currently developing Steelhead critical habitat status and description for this ESU.

Redwood Creek is also wholly contained in the California Coastal Evolutionary Significant Unit (ESU) for Chinook salmon. As such, it is designated as critical habitat for Chinook salmon, listed as “threatened” under the ESA in 1999.

Surface Water Monitoring Program Monitoring Stations for the Redwood Creek Hydrologic Unit (107).

| Redwood Creek Hydrologic Unit (107) - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| RDWDOR (P) 107.10 (Redwood Creek at Orick) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Trinidad Hydrologic Unit (108)

The Trinidad Hydrologic Unit comprises a portion of the Humboldt Bay Watershed Management Area as identified in the Watershed Planning Chapter.

# Little River

Under the Federal Endangered Species Act (ESA), the Little River is wholly contained in the Northern California Evolutionary Significant Unit (ESU) for Steelhead, listed as “threatened” under the ESA in 2000.  The National Marine Fisheries Service (NMFS) is currently developing Steelhead critical habitat status and description for this ESU.

Little River is also wholly contained in the California Coastal Evolutionary Significant Unit (ESU) for Chinook salmon. As such, it is designated as critical habitat for Chinook salmon, listed as “threatened” under the ESA in 1999.

| Trinidad Hydrologic Unit (108) - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| LITCRN (R) 108.20 (Little River at Crannel) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Mad River Hydrologic Unit (109)

The Mad River Hydrologic Unit comprises a portion of the Humboldt Bay Watershed Management Area as identified in the Watershed Planning Chapter.

The Mad River watershed is mixed private and Forest Service timberland with a long history of timber harvest. Adding to the mix is gravel mining in the lower portions of the watershed.  The Mad River is Section 303(d) listed for turbidity and temperature impacts. The primary issues for the watershed are forestry-related, with urbanization and associated industrial and public point sources. For the Mad River and its tributaries, discharge of waste is allowed only under NPDES permit during the period of October 1 through May 14 and at 1% of the flow of the receiving water. The McKinleyville Community Services District discharges municipal effluent to the Mad River in compliance with those restrictions. The City of Blue Lake does not discharge directly, disposing of effluent in percolation/evaporation ponds.

MtBE was detected in Ruth Lake on the Mad River, upstream of public and private water supplies.  Additional sampling is needed to define the extent of the problem.

Under the Federal Endangered Species Act (ESA), the Mad River is wholly contained in the Northern California Evolutionary Significant Unit (ESU) for Steelhead, listed as “threatened” under the ESA in 2000.  The National Marine Fisheries Service (NMFS) is currently developing Steelhead critical habitat status and description for this ESU.

The Mad River is also wholly contained in the California Coastal Evolutionary Significant Unit (ESU) for Chinook salmon. As such, it is designated as critical habitat for Chinook salmon, listed as “threatened” under the ESA in 1999.

Surface Water Monitoring Program Monitoring Stations for the Mad River Hydrologic Unit (109).

One permanent station and five rotating stations have been established for this HUC.  Two stations have been specifically established in Ruth Lake to monitor the extent of MtBE and other fuel by-products including benzene, toluene, ethylbenzene and xylene (BTEX).

The SWAMP has addressed some monitoring issues in the WMA in FY 2000-01 and will intensify monitoring in the rotation into the WMA in FY 2001-02. Listed below are the planned and proposed monitoring activities under that program:

Long-term monitoring stations:

One long-term station was established during FY 2000-01: Mad River at Blue Lake. Other long-term stations in the watershed will be proposed if appropriate from the rotation in FY 2001-02.

Other SWAMP monitoring:

We initiated investigation of MtBE concentrations in Ruth Lake in FY 2000-01 and will continue our efforts as part of the intensive survey in FY 2001-02.

| Mad River Hydrologic Unit (109). - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| MADBLU (P) 109.10 (Mad River at Blue Lake) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| MADBUT (P) 109.30 (Mad River at Butler Valley, d/s Boulder Creek) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12, 14, 15 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature Channel Morphology, Macroinvertebrate Assembledge |
| MADRUT (R) 109.40 (Mad River at Ruth) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| MADOUT (R) 109.40 (Mad River at Ruth Lake Outlet Works) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,9,11,12 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, MtBE, BTEX |
| RL01 (R) 109.40 (Ruth Lake Station #1) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 2, 3, 9 | 4 | Pollutant Exposure, Habitat | MtBE, BTEX, Dissolved Oxygen, Water Temperature |
| RLO2 (R) 109.40 (Ruth Lake Station #2) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 2, 3, 9 | 4 | Pollutant Exposure, Habitat | MtBE, BTEX, Dissolved Oxygen, Water Temperature |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Eureka Plain Hydrologic Unit (110)

The Eureka Plain Hydrologic comprises a portion of the Humboldt Bay Watershed Management Area as identified in the Watershed Planning Chapter.

Humboldt Bay tributaries have experienced problems from urbanization and agricultural uses in addition to timber harvest issues. There is a concern about the impact of manure from dairies on Humboldt Bay and Elk River. Additionally, they flow into Humboldt Bay and can impact uses there. Historical land uses, which may still be affecting water quality, include industrial lumber mill sites. The major population area in this WMA is the Humboldt Bay Area and the cities of Eureka and Arcata. Suburban growth is occurring in the unincorporated community of McKinleyville, north  of Arcata. Flat land areas around the bay are predominantly pastureland with some limited cultivation, primarily lily bulb farms. Humboldt Bay is an important commercial and recreational shellfish growing area, as well as deep-water port.

Local concerns include sedimentation of Freshwater Creek and Elk River and subsequent flooding and domestic water supply degradation. Some industrial timberland owners are developing Sustained Yield Plans that will address sensitive watershed issues to some degree.

Historically, wastewater discharges to the Bay impacted the shellfish uses. However, considerable emphasis on improved treatment and reliability and the consolidation and relocation of the Eureka wastewater plants has significantly reduced the problem. Discharge of treated wastewater to Humboldt Bay is permitted from the Arcata treatment plant and marsh complex in Arcata Bay (north Humboldt Bay) and the Elk River plant which serves the greater Eureka area. The Arcata plant discharges through a constructed marsh/pond complex prior to discharge to Arcata Bay. The Elk River plant times its discharges to out-going tidal flow so that effluent promptly exits the bay. The College of the Redwoods operates a small sewage treatment plant that discharges indirectly to south Humboldt Bay. Contamination from collection system overflows of raw sewage during high intensity rainfall events is still a threat to the commercial and recreational uses of the Bay.

Storm water runoff from the whole watershed conveys indicators of bacterial contamination that impacts shellfish harvest. Seasonal and rainfall-based shellfish harvesting closures are in effect to mitigate the effects of nonpoint source runoff. A shellfish Technical Advisory Committee was established in November 1995 to address nonpoint source runoff issues.

Under the Federal Endangered Species Act (ESA), the streams and rivers within the Eureka Plain Hydrologic unit are wholly contained in the Northern California Evolutionary Significant Unit (ESU) for Steelhead, listed as “threatened” under the ESA in 2000.  The National Marine Fisheries Service (NMFS) is currently developing Steelhead critical habitat status and description for this ESU.

The streams and rivers within the Eureka Plain Hydrologic unit are also wholly contained in the California Coastal Evolutionary Significant Unit (ESU) for Chinook salmon. As such, they are designated as critical habitat for Chinook salmon, listed as “threatened” under the ESA in 1999.

Surface Water Monitoring Program Monitoring Stations for the Eureka Plain Hydrologic Unit (110).

A total of nine rotating stations have been established in this HUC  They include two in Jacoby Creek, two in Freshwater Creek, three in Elk River, and one in Salmon Creek.

| Eureka Plain Hydrologic Unit (110) - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| JACBAY (R) 110.00 (Jacoby Creek near Bayside) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,4,9,10,11,12,13, 14, 15 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Macroinvertebrate Assemblage, Channel Morphology |
| JACUP (R) 110.00 (Jacoby Creek - Upper Station) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,4,9,10,11,12,13, 14, 15 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Macroinvertebrate Assemblage, Channel Morphology |
| FRESHW (R) 110.00 (Freshwater Creek near Freshwater Corners) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,4,9,10,11,12,13, 14, 15 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Macroinvertebrate Assemblage, Channel Morphology |
| FRESUP (R) 110.00 (Freshwater Creek - Upper Station) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,4,9,10,11,12,13, 14, 15 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Channel Morphology |
| ELKRIV (R) 110.00 (Elk River near Elk River) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,4,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| ELKNFK (R) 110.00 (Elk River - North Fork u/s Jones) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,4,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| ELKSFK (R) 110.00 (Elk River - South Fork u/s Jones) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,4,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| SALHY1 (R) 110.00 (Salmon Creek at Highway 1) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,4,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Eel River Hydrologic Unit (111).

The Eel River comprises the Eel River WMA as described in the Watershed Planning Chapter. The Eel River Watershed encompasses roughly 3,684 square miles in highly erodable soils in the steep coastal mountains of the NCR, supporting a variety of water uses including municipal and agricultural supply systems, salmonid fisheries, and recreation. Surface water in many areas is intimately connected with the ground water along the nearby alluvial valleys, thereby having a profound effect on local groundwater supplies. The Eel River Watershed is also a prime recreational area boasting numerous state and private campgrounds along its length with both contact and noncontact uses such as boating and swimming. The Eel River is the third largest producer of salmon and steelhead in the State of California and supports a large recreational fishing industry. The erodable soils, steep terrain, and timber production evoke a high level of concern for the anadromous fishery resource. Coho salmon were listed as endangered under the federal Endangered Species Act in 1997. It is heavily forested and as such, heavily utilized for timber production. Numerous activities occur within the watershed that may result in potential adverse effects to the beneficial uses of the Eel River Watershed. Municipal, agricultural, and recreational uses may be impaired through discharges to surface water bodies from chemical, biological, and sedimentary materials entering the surface water system. A few of the many activities threatening surface water beneficial uses include: illegal waste disposal, vehicle and railroad maintenance yard operations, herbicide application, gravel extraction, timber harvesting, road building, dairy operations, automotive wrecking yard activities, historical mill operations, wood treatment facilities, publicly owned treatment works, and failing septic systems.

Under the Federal Endangered Species Act (ESA), the Eel River system is wholly contained in the Northern California Evolutionary Significant Unit (ESU) for Steelhead, listed as “threatened” under the ESA in 2000.  The National Marine Fisheries Service (NMFS) is currently developing Steelhead critical habitat status and description for this ESU.

The Eel River system is also wholly contained in the California Coastal Evolutionary Significant Unit (ESU) for Chinook salmon. As such, it is designated as critical habitat for Chinook salmon, listed as “threatened” under the ESA in 1999.

In general, the primary issues associated with water quality in the Eel River WMA are focused on

the beneficial uses for drinking water supply, recreation, and the salmonid fishery.

Four water quality goals for the Eel River WMA have been identified are related through the beneficial uses they address:

Goal 1: Protect and enhance the salmonid resources (COLD)

Goal 2: Protect other surface water uses (MUN, AGR, REC 1, REC-2)

Goal 3: Protect ground water uses (MUN, IND, AGR, REC-1, REC-2)

Goal 4. Protect warmwater fishery resources

Protection of surface water (Goal 2) for the primary beneficial uses MUN, AGR, REC-1 and REC-2 will in most cases protect all other beneficial uses. The MUN (municipal and domestic supply) beneficial use designation is for uses of water for community, or individual water supply systems including, but not limited to, drinking water supply. It demands, therefore, the highest quality of water. The REC-1 (water contact recreation) beneficial use designation is for uses of water for recreational activities involving body contact with water, where ingestion is reasonably possible. This beneficial use also demands a high degree of water quality. If MUN and REC-1 beneficial uses are protected then it follows that agricultural and industrial supplies are also protected which relates Goal 2 to Goal 3. The protection of cold and warm water fisheries (Goals 1 and 4) requires the protection of surface and ground waters (Goals 2 and 3) along with additional concerns for siltation, habitat loss, low tributary flows and water temperature. Therefore, by protecting the beneficial uses that demand the highest quality waters most components supporting the other beneficial uses also will be protected.

Surface Water Ambient Monitoring Program Monitoring Stations

The SWAMP has addressed some monitoring issues in the WMA in FY 2000-01, and will investigate more intensively in the rotation into the WMA in FY 2001-02.

Listed below are the planned and proposed monitoring activities under that program:

Long-term monitoring stations:

Five long-term stations were for setup in spring of 2001: South Fork at confluence, Bull Creek, and near Branscomb Creek; Eel River at Dos Rios; Middle Fork at Dos Rios. Other long-term stations in the WMA will be proposed if appropriate from the rotation in FY 2001-02.

The intensive survey will provide sampling sites in waterbodies in the WMA. Anticipated parameters are general water chemistry, nutrients, metals, organic chemicals, and sediment related parameters.  We will address temperature and bacterial issues in the WMA during the intensive survey.

| Eel River Hydrologic Unit (111) - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| VAN101 (R) 111.11 (Van Duzen River at Highway 101) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| EELHOL (P) 111.12 (Eel River at Holmes) | MUN,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 3 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| YAGCAR (R) 111.12 (Yager Creek at Carlotta) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,,9,10,1112,13, 14, 15 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Channel Morphology |
| VANBRG (R) 111.22 (Van Duzen River at Bridgeville) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| VANBRG (R) 111.22 (Van Duzen River near Dinsmore) | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| EELSFK (R) 111.30 (Eel River – South Fork d/s of Bull Creek) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, PCP/TCP,Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |
| EELBEN (R) 111.32 (Eel River – South Fork near Benbow) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| EELBRN (P) 111.33 (Eel River – South Fork near Branscomb) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 3 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Channel Morphology, Vitellogenin |
| ELDRCR (R) 111.33 (Elder Creek at Eel River) | MUN,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| EELMDV (P) 111.41 (Eel River above Dyerville) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 3 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, PCP/TCP,Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| EELMAN (P) 111.41 (Eel River above Dos Rios) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13, | 3 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| EELALD (R) 111.42 (Eel River near Alder Point) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| NFELMI (R) 111.50 (Eel River – North Fork near Mina) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, |
| EELHST (R) 111.62 (Eel River near Hearst) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, PCP/TCP, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| LP01 (R) 111.63 Lake Pillsbury, Station #1 | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 2, 3, 9 | 4 | Pollutant Exposure, Habitat | MtBE, BTEX, Dissolved Oxygen, Water Temperature |
| LP02 (R) 111.63 Lake Pillsbury, Station #2 | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 2, 3, 9 | 4 | Pollutant Exposure, Habitat | MtBE, BTEX, Dissolved Oxygen, Water Temperature |
| LPOUT (R) 109.40 | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, MtBE, BTEX |
| EELVAN (R) 111.63 (Eel River at Van Arsdale Reservoir) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 5 C 3 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| MFKEEL (P) 111.70 (Eel River – Middle Fork at Dos Rios) | MUN,AGR,REC1,REC2, COLD, WARM, SPWN, MIGR, WILD, RARE | 1,2,3,9,10,11,12,13 | 3 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Mendocino Coast Hydrologic Unit (113)

Gualala Watershed Management Area (113.82)

The Gualala River watershed in Sonoma and Mendocino counties is about 300 square miles in area.  The Gualala River runs in a north-south direction flowing into the ocean at the town of Gualala.  The watershed is in mountainous terrain and contains relatively erodible soils. The tributaries flow through steep valleys with narrow bottom lands and elevations range from sea level to over 2,650 feet. The steep slopes are forested mainly with Douglas fir and redwood interspersed with madrone and tan oak. Rainfall averages 38 inches per year at the coast and up to 100 inches per year on the inland peaks. Primary land use is forest production and grazing.  Hillside vineyard development is becoming an increasing threat to water quality as more and more steep land is converted to vineyards.

The Gualala River is listed on California’s 303(d) list as a water quality limited water requiring the establishment of a Total Maximum Daily Load (TMDL), for sediment. The key stakeholder concern is the decline of the once healthy coho salmon and steelhead trout fisheries thought to be associated with excess sediment load and elevated water temperatures. A Consent Decree entered in settlement of a lawsuit against the USEPA assigned the date of December 31, 2001, for completion of TMDL allocations for the Gualala River.

Under the Federal Endangered Species Act (ESA), the Gualala River is wholly contained in the Northern California Evolutionary Significant Unit (ESU) for Steelhead, listed as “threatened” under the ESA in 2000.  The National Marine Fisheries Service (NMFS) is currently developing Steelhead critical habitat status and description for this ESU.

The Gualala River is also wholly contained in the California Coastal Evolutionary Significant Unit (ESU) for Chinook salmon. As such, it is designated as critical habitat for Chinook salmon, listed as “threatened” under the ESA in 1999.

In addition, the Gualala River is also wholly contained in the Central California Coast Evolutionary Significant Unit (ESU) for Coho salmon. As such, it is designated as critical habitat for Coho salmon, listed as “threatened” under the ESA in 1996.

The primary water quality goals for the Gualala River center around protection of the beneficial uses associated with aquatic life and drinking water supplies. The development of the TMDL waste reduction strategy for sediment is the highest priority for action in the watershed.

Surface Water Monitoring Program Monitoring Sites

The SWAMP and NCWAP addressed water quality and some channel geometry monitoring issues in the WMA in FY 2000-01 at five rotating sites:

North Fork near Gualala

South Fork at Twin Bridges

Wheatfield Fork at Twin Bridges

South Fork near Plantation

Wheatfield Fork above House Creek

A permanent station has been established at Gualala Regional Park and will be included in the FY 01-02 monitoring effort.  Anticipated parameters are general water chemistry, nutrients, metals, and channel geometry and stream bed characteristics.

| Mendocino Coast Hydrologic Unit (113) - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| GUAGRP (P) 113.62 | MUN,REC1,REC2, COLD, SPWN, MIGR, WILD, | 1,2,3, 9,10,11,12,13, 14,15 | 5 C | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Channel Morphology |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

Russian River Hydrologic Unit (114)

The Russian river comprises a major part of the Russian/Bodega WMA as described in the Watershed Planning Chapter.  The Russian River hydrologic unit encompasses 1485 square miles in Mendocino and Sonoma counties, bounded by the Coast Ranges on both the east and west. The mainstem is about 110 miles long, flowing southward from Redwood and Potter valleys (north of Ukiah) to its confluence with Mark West Creek, where it turns west to cut through the coast range and empties into the Pacific Ocean at Jenner.  The principal tributaries from the headwaters down are the East Fork Russian River, Feliz Creek, Pieta Creek, Big Sulfur Creek, Dry Creek, Mark West Creek (including the Laguna de Santa Rosa), Green Valley Creek, Willow Creek, Fife Creek,Austin Creek, and other small streams. Elevations range from sea level at the estuary near Jenner to 4,343 feet at the summit of Mt. St. Helena in the Mayacamas Mountains.

The Russian River ground water basins are generally broad, alluviated valleys in the Coast Range physiographic province of California. The province is dominated by northwest trending mountain ranges and intervening valleys. Cotati Valley is the southern portion of the larger Santa Rosa Valley which is bounded on the east by the Sonoma Mountains and the west by the mountains of the Coast Range Franciscan Complex. The geologic materials in the unsaturated zone between the ground surface and the top of ground water generally consists of interbedded clays, silts and sands. Sediments underlying the valley are predominantly young, unconsoliated fluvial deposits of Quaternary age derived from the nearby Sonoma Mountains and are characterized by variable grain sizes, composed mainly of interbedded gravel, sand, silt, and silty clay. Sedimentary rocks of the Petaluma Formation and volcanic rocks of the Sonoma Group underlie the valley alluvium and are exposed in the hills to the east.

Two reservoirs provide flood protection and water supply storage: 1) Coyote Dam and Lake Mendocino on the East Fork Russian River near Ukiah, and 2) Warm Springs Dam and Lake Sonoma on Dry Creek west of Healdsburg. A diversion from the Eel River through the Potter Valley powerhouse flows into the East Fork and Lake Mendocino. The Russian River hydrologic unit supplies drinking water, including ground water supply to over 500,000 people and an unknown amount of water for agricultural purposes. The State Division of Water Rights has declared the Russian River tributaries fully appropriated from April 1 through December 14. The Water Rights Division is in the process of developing a strategy to deal with additional diversions in the mainstem and tributaries outside of the fully appropriated period. The majority of flow in the Russian River is during the winter season, when rainfall ranges from 30-80 inches, depending on locale. The summer climate is moist and cool near the coast with temperatures increasing in the upper valley areas that are more isolated from the coastal influence. Ground water is found in shallow to deep aquifers. The water table aquifer is particularly vulnerable to contamination from nitrates and toxic chemicals, and drinking water wells often withdraw from this shallow aquifer, and may connect the shallow layers with deeper ground water bearing zones.

Land Uses in the Russian River Hydrologic Unit

The watershed is agriculturally based, with urban and industrial uses concentrated around the incorporated municipalities. The most notable are Ukiah, Cloverdale, Healdsburg, Guerneville, Windsor, Rohnert Park, Cotati, Sebastopol, and Santa Rosa. The largest concentration of urban and industrial use is in the Santa Rosa Plain, with Ukiah and Windsor second and third. Industrial uses include electronics manufacturing industries, petroleum distribution plants, light manufacturing, wrecking and salvage yards, wineries, wood products, and industries related to the construction industry. Santa Rosa is the commercial distribution center for the North Coast.

In the Potter Valley area north of Ukiah, irrigated agriculture and pasturing are common land uses. Rangeland and mixed coniferous forests (with minimal timber harvesting) are prevalent in the hills away from the farmed alluvial plains. Around Ukiah, irrigated orchard and vineyard are common land uses with light industry, several large wood products facilities associated with the timber industry, and gravel mining. Water quality issues in this part of the watershed are primarily associated with industrial areas, historical waste disposal practices, wastewater treatment plants, water use, erosion and sedimentation in the tributaries, destruction of riparian areas, and agricultural chemical uses in the alluvial areas.

Moving down the watershed, the Hopland area is predominantly vineyard with rangeland grazing in the areas away from the mainstem. The river then cuts through a small canyon with rangeland grazing as the primary land use before reaching Cloverdale and more vineyards. Vineyards dominate the valley areas down to the Santa Rosa Plains. Vineyard development in the hillside areas adjacent to the alluvial terrace is an increasing concern from the standpoint of erosion and sedimentation. Gravel terrace pits are another feature interspersed in the alluvial plain. In addition to the water quality issues upstream, bank erosion, health of riparian areas, construction activities, and more industrial, commercial, household, and agricultural chemical uses rank high as concerns for this area.

The Santa Rosa Plain and Healdsburg hydrogeologic areas contain large ground water basins, supplying water for municipal, domestic, industrial and agricultural uses. The Santa Rosa Plain and tributary uplands include a number of animal facility operations. There are currently 24 active dairies in the Mark West Creek (Laguna de Santa Rosa) watershed. Conversion of rangeland, pasture, and orchards to vineyard has increased in the last decade. The availability of reclaimed wastewater produced by the City of Santa Rosa operated sub-regional municipal wastewater treatment facility has resulted in conversion of about 6,500 acres of rangeland to irrigated pasture, cultivated fodder crops, and other uses. The Santa Rosa Plain is the most populated area in the North Coast Region with six incorporated communities and over 200,000 residents (1990 US Census). A number of large river terrace pit-type gravel mines are located downstream of Healdsburg.

The trend appears to be towards continued conversion of range, pasture and forest lands to vineyards, and continued growth of the urban areas of Ukiah, Cloverdale, Guerneville, Healdsburg, Windsor, Santa Rosa, and Rohnert Park. In the future there will be a major expansion in the electronic industry. Associated with that growth are active construction sites and an increase in light industrial operations. A concerted effort is being made in the Santa Rosa Plains to retain the reclaimed wastewater irrigated crop and pastureland type of agriculture and maintain the viability of the dairy industry. However, significant conversion of rangeland and pasture to vineyards continues to occur . The market for premium North Coast wine grapes far outstrips supply. Therefore, the pressure for land conversion to vineyards probably will not diminish, and there could be a water supply issue in the future.

The Laguna de Santa Rosa watershed drains the southern two-thirds of the Santa Rosa Plains. The Laguna de Santa Rosa, which is a major tributary of Mark West Creek, is listed for nutrient and dissolved oxygen impairment on the C W A section 303(d) list. Nutrient and dissolved oxygen impairments result from both point and nonpoint source discharges and the hydrology of the watershed. An active waste reduction strategy is underway per section 303(d) requirements, including the development of waste loading limitations.

The Russian River turns to the west and cuts through the Coastal Range downstream from the confluence of the Laguna de Santa Rosa and Mark West Creek tributary area. This downstream physical structure of the river has a lower gradient and the summer base flow occupies most of the low flow channel. The lower Russian River hillsides are steep and forested with mixed conifers, redwoods being the major component. Residential areas are located periodically along the river with a number of them on the narrow flood plain itself. Land uses are consistent with the semi-rural setting with vineyards and pastures located on the flood plain benches. Industrial activity is associated primarily with timber (harvesting and lumber) and the construction trade. Tourism associated with summer recreational use of the river is a major economic base. Growth has been sporadic. The 1990 census lists five unincorporated communities with less than 10,000 residents. Water quality concerns include effects from upstream land use activities in both urban and rural areas and include individual on-site septic system problems and erosion and sedimentation problems from tributary streams. As the river flood plain flattens to meet the ocean, the river widens into a relatively narrow estuary in the Jenner area. Land use is predominantly rangeland grazing and timber production.

Under the Federal Endangered Species Act (ESA), the Russian River is wholly contained in the Central California Coast Evolutionary Significant Unit (ESU) for Steelhead, listed as “threatened” under the ESA in 2000.  The National Marine Fisheries Service (NMFS) is currently developing Steelhead critical habitat status and description for this ESU.

The Russian River is also wholly contained in the California Coastal Evolutionary Significant Unit (ESU) for Chinook salmon. As such, it is designated as critical habitat for Chinook salmon, listed as “threatened” under the ESA in 1999.

In addition, the Russian River is also wholly contained in the Central California Coast Evolutionary Significant Unit (ESU) for Coho salmon. As such, it is designated as critical habitat for Coho salmon, listed as “threatened” under the ESA in 1996.

Monitoring:

Water quality monitoring efforts will be focused on maintaining four long-term monitoring stations in the Russian River watershed. This includes TMDL confirmation monitoring in the Laguna de Santa Rosa, and expanding the temperature monitoring consortium for the watershed to include other water quality parameters. Those activities will be funded through the Surface Water Monitoring Program (SWAMP). Activities also include ground water quality assessment and public participation. Additional needs in the smaller watersheds in the Bodega Unit including monitoring in the Stemple Creek watershed, and monitoring and assessment in the Americano Creek, Cheney Gulch, and Salmon Creek watersheds. These watersheds will be addressed in the SWAMP rotation in FY 2004-05. Additional options we will consider for improved and enhanced monitoring include the establishment of long-term photo point monitoring records, fostering voluntary monitoring by individuals and watershed groups; reviewing the USEPA Rapid Bioassessment Protocol, providing spatial analysis of surface and ground water data, and increased coordination with local universities and the UC Extension Service for education and outreach. In addition, domestic well sampling in the McMinn Superfund area for the next five years has been funded by the Sonoma County Water Agency as part of the Roseland Action Plan

| Russian River Hydrologic Unit (114) - FY 2001-02 Monitoring Activities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station (Type) (1) HUC | Beneficial Use(s) | Monitoring Objectives (2) | Freq (3) | Category(s) | Indicator(s) (4) |
| RRGRNV (P) 114.11 (Russian River at Guerneville) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, | 1,2,3,4,9,10,11,12,13 | 5 C 5 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| RRHMB (P) 114.24 (Russian River at Healdsburg Memorial Beach) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, | 1,2,3,4,9,10,11,12,13 | 5 C 5 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| RRCLO (P) 114.26 (Russian River Cloverdale) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, | 1,2,3,4,9,10,11,12,13 | 5 C 5 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |
| RRUKTL (P) 114.31 (Russian River at Tamadge - Ukiah) | MUN,REC1,REC2, WARM, COLD, SPWN, MIGR, WILD, | 1,2,3,4,9,10,11,12,13 | 5 C 5 O | Contaminant Exposure, Biological Response, Pollutant Exposure, Habitat | Inorganic and Organic Water Chemistry, Chl-a, Nutrients, Total Organic Carbon, Dissolved Oxygen, Water Temperature, Vitellogenin |

Notes:	1. Type: P = Permanent, R = Rotating

2. Monitoring Objectives: From the November 30, 2000 Report to the Legislature, Section VI, Pages 22-25 (Attachment A)

3. Frequency: 	N = number of samples per FY, C= Conventional Water Chemistry

O = Organic Water Chemistry

4. Indicator: From the November 30, 2000 Report to the Legislature, Section VII, Table 3, Pages 33-35 (Attachment A)

### IV. Specific Sample Design and Sample Collection

### Wadeable Stream and River Sites

The field crew will collect the samples at sites where the geo-coordinates were previously recorded on the site reconnaissance form during past field work at these stations. If sampling work is being performed at a new station, the geo-coordinates and cross-referenced photographs, and other pertinent information shall be recorded on the field form for future reference.  If there is confusion about locating a site, it shall be resolved in consultation with RWQCB staff member present in the field or via phone contact with the regional SWAMP program coordinator.  Sufficient volume of sediment or tissue or water shall be collected in order to perform the analyses to be conducted at each station, as well as to allow for archiving of samples for future analysis, as shown on Attachment B “Services to be performed at each station/Budget” table.

Sample collection and subsequent processing and testing will be performed according to protocols specified in the most recent version of the SWAMP Quality Assurance Project Plan (QAPP) (currently in development) and region-specific QAPP’s/SOP’s.”

### Lake sampling for MTBE

Sampling will be conducted from a boat powered by a 4-cycle outboard motor using MTBE-free fuel.  In order to avoid contamination of the site by the sampling effort, when the boat is within 300 yards of the sampling station, the motor is turned off, raised out of the water, and securely wrapped in a 4 mil plastic bag.  The fuel tank vent is secured, and the station is approached using an electric trolling motor.

Lake sampling stations have been selected in deep, open-water locations that represent areas of general use by all types of watercraft.  In order not to bias the sampling results, sampling near marina areas is avoided.  In order to determine if any contamination is escaping the reservoir, samples will also be collected at each of the reservoir’s outlet works.

Prior to sampling at each lake station, a vertical profile of temperature, dissolved oxygen, pH and specific conductvity will obtained using a multiparameter datasonde and the depth and thickness of the metalimnion, or thermocline, will be determined.  Sample collections will then made at the surface, mid-epilimnion, approximately one meter below the thermocline and 6 meters below the thermocline.

Lake surface samples are collected using a pre-cleaned stainless steel beaker and carefully decanted into VOA containers preserved with 1:1 HCL.

Sub-surface samples are collected with a pre-cleaned 1.1L stainless steel Kemmerer bottle with poly-urathane end seals.

For quality control purposes, one field replicate and one wash-water blank will be collected at each station.

All sample collection and handling will accomplished pursuant to protocols published by the U. S Geological Survey (USGS) National Water-Quality Assessment Program, NAWQA (Shelton 1997).

Vitellogenin Screening

Vitellogenin (VTG) is a blood serum phospholipoglycoprotein precursor to egg yolk normally found in female fish.  Although male fish also carry the gene for VTG synthesis, it is normally not expressed as male fish do not produce eggs.  Exposure to endocrine disrupting compounds, however, can induce the synthesis of vitellogenin in male fish making it ideal biomarker for indicating an organisms exposure to these compounds in their environment.

Fish and blood serum collection protocols are currently under development with California Department of Fish and Game in cooperation with the Core Biomarkers Facility at the University of Florida at Gainsville.  This facility has developed considerable expertise using Enzyme-linked Immunospecific Assay (ELISA) procedures to identify VTG in blood serum and will be the initial sub-contractor responsible for VTG analysis for at least the first year of the screening effort.

The Regional Board, with the assistance of CFG, will collect the fish and prepare the blood samples in the field.  The blood will then be transported to UFLA for subsequent VTG analysis.

### Stream Gaging

Due to funding cutbacks and priority changes, a number of stream gages previously installed and maintained by the United States Geological Service (USGS) and the California Department of Water Resources (DWR) in North Coast watersheds are no longer operational.  Some gages have been decommissioned and completely removed while others may be functional but are not maintained.

Many of the rivers in the North Coast region are listed on the CWA 303(d) list of impaired water bodies as sediment impaired.  River flow and sediment transport data necessary to support TMDL development is lacking in many of these watersheds.

A master contract has been developed between the SWRCB and the United States Geological Service (USGS) and is, in part, funded by SWAMP dollars.  Among other things, this contract provides a mechanism to construct and maintain the stream gages necessary to aquire the data needed for TMDL development.  FY 2000-01 SWAMP funds are being used to install three gages in the Eel River system.  The gages will measure stage height, discharge (flow) and water temperature and will have satellite radio telemetry links.

The gages are currently under construction on the North Fork Eel near Mina, the Middle Fork upstream of Black Butte River and on Black Butte River at its confluence with the Middle Fork.  All are expected to be fully operational by December, 2001.

FY2001-02 funds will be used to maintain these gages while installing a new gage on the Van Duzen River.

Laboratory Analysis

The Regional Board staff shall oversee surface water sample collection.  All serum vitellogenin sample collection shall be performed by Regional Board and/or CDFG staff pursuant to protocols developed through collaboration with the University of Florida Core Biomarkers Facility.

Actual analytical services that will be performed on each sample are detailed in the attached table: “Services to be performed at Each Station/Budget

A statewide SWAMP QAPP is in the process of development by CDFG.  The QAPP will address issues of analytical methodology, detection limits and QA/QC criteria.  The provisions of this plan will be adopted and consistantly applied by all Regions.

### Data Quality Evaluation and Data Reporting

Analytical results shall be reported in tabular format and in a timely manner by the laboratory.  Appropriate QA/QC documentation should accompany each report pursuant to the SWAMP QAPP (in development).  In addition, the data shall be provided electronically in EXCEL format.

The California Stream Bioassessment protocol developed by CDFG will be followed until a SWAMP specific bioassessment protocol is developed.

Stream gage data shall be provided in a manner consistent with the USGS QAPP.

### Deliverable Products

CDFG Master Contract

Site reconnaissance, sample collection, and chemical, physical and biological analysis as specified per contract task order.

Laboratory analysis data reports per contract task order.

The data report will include the following items, where applicable, but shall not necessarily be limited to the following items:

All station data including CDFG station name, station number, IDORG number, leg number, sample collection date, sample station longitude and latitude, sample GPS coordinates, sample station water depth, sample location characteristics, toxicity test endpoint mean and standard deviation, and all detection limits.   In addition to the above data, the following will also be reported for all stations indicated on the attached “Services to be performed at each station/cost” spreadsheet for bioassessment: raw data and computed biological indices.  Additionally, data from the bioaccumulation tests will be reported as tissue chemistry data for the specific chemical constituents shown on the attached “Services to be performed at each station/cost” spreadsheet.  A map (and GIS shape file) should be included showing the locations of each sampling station and an indication of the overall integrity of that site as excellent, good, marginal, or poor.  Another map (and GIS shape file) should further indicate the integrity of each site for biological, chemical, and toxicity data results expressed as a triad for each site.

USGS Master Contract

Installation and maintenance of stream gages per contract task order.

Reports as specified in contract task order.

### Desired Milestone Schedule

We plan five sampling episodes for most stations in this FY for the collection of conventional chemistry and metals.  Three of these episodes will include sampling for water column organic chemicals.

Currently, we plan to initiate sample collection in November, 2001, with subsequent sampling episodes in January, March, April and May of 2002.

MTBE sampling is planned for the end of October, 2001, with subsequent events planned for April, May and June of 2002.

### Desired Sample Throughput Schedule

Ideally, provisional sample results for each sampling episode should be provided to the Regional Board prior to the subsequent sampling episode for the same set of stations.

Actual schedule will be provided by CDFG.

### Budget

Please see Attachment B: “Services to be Preformed at Each Station/Budget”

### V. Working Relationships – Decision Matrix

| Task | #### Responsible Organization |  |  |  |
| --- | --- | --- | --- | --- |
|  | SWRCB | RWQCB | Contractors |  |
|  |  |  | DFG | USGS |
| Develop contract(s) for monitoring services. |  |  |  |  |
| Identify water bodies or sites of concern and clean sites to be monitored. |  |  |  |  |
| Identify site-specific locations with potential beneficial use impacts or unimpacted conditions that will be monitored. |  |  |  |  |
| Decide if concern is related to objectives focused on location or trends of impacts. |  |  |  |  |
| Select monitoring objective(s) based on potential beneficial use impact(s) or need to identify baseline conditions. |  |  |  |  |
| Identify already-completed monitoring and research efforts focused on potential problem, monitoring objective, or clean conditions. |  |  |  |  |
| Make decision on adequacy of available information. |  |  |  |  |
| Prepare site-specific study design based on monitoring objectives, the assessment of available information, sampling design, and indicators. | (Work Plan Review Role) |  |  |  |
| Implement study design. (Collect and analyze samples.) |  |  |  |  |
| Track study progress. Review quality assurance information and make assessments on data quality. Adapt study as needed. | (Review Role) |  |  |  |
| Report data through SWRCB web site |  | (Coordination Role) |  |  |
| Prepare written report of data. |  |  |  |  |

VI. Other Information and Attachments

Statewide Monitoring Efforts

The Toxic Substances Monitoring Program

The Toxic Substances Monitoring Program (TSMP) was initiated in 1976 by the SWRCB.  The TSMP was organized to provide a uniform statewide approach to the detection and evaluation of the occurrence of toxic substances in fresh, estuarine, and marine waters of the State through the analysis of fish and other aquatic life.  The TSMP primarily targets water bodies with known or suspected impaired water quality and is not intended to give an overall water quality assessment.  The California Department of Fish and Game (CDFG) carries out the statewide TSMP for the SWRCB by collecting and analyzing samples.  The SWRCB provides funding for the program under an ongoing interagency agreement with the CDFG.  Each year CDFG in conjunction with Regional Board staff select sites and collect fish or other aquatic organisms from these sites for analysis

During this fiscal year, collections of fish for mercury and organic chemical analysis are planned for Lake Sonoma, Lake Mendocino, Lake Pillsbury and Van Arsdale reservoir.

California State Mussle Watch Program

The California State Mussel Watch Program (SMWP), initiated in 1977 by the State Water Resources Control Board (SWRCB), was organized to provide a uniform statewide approach to the detection and evaluation of the occurrence of toxic substances in the waters of California's bays, harbors, and estuaries.  This is accomplished through the analysis of transplanted and resident mussels and clams.  The SMWP primarily targets areas with known or suspected impaired water quality and is not intended to give an overall water quality assessment.  The CDFG carries out the statewide SMWP for the SWRCB by collecting and analyzing samples.  The SWRCB provides funding under an ongoing interagency agreement with the CDFG.  Sampling stations are selected by the six coastal Regional Water Quality Control Boards.

Collection sites for FY 01-02 are presently being selected by the Regional Board staff and will likely include locations in Crescent City Harbor and Humboldt Bay.

Attachmments:

Attachment A:	“Report to the Legislature, November 30, 2000”

Attachment B:	“Services to be Preformed at Each Station/Budget”