List of Indicators

Monitoring programs sponsored by the SWRCB and the RWQCBs have used a variety of environmental indicators.  Indicators that have been used in ambient monitoring efforts and meet the requirements of the general criteria are presented in Table 3.  These indicators are considered a starting point for the indicators which should be used in the State’s ambient monitoring efforts.

Table 3:  List of indicators for Site-Specific and Regional Monitoring

| Beneficial Use | Monitoring Objectives |  | Category | Indicator |
| --- | --- | --- | --- | --- |
|  | Site-Specific | Regional |  |  |
| Water Contact | 1 | 1, 2, and 3 | Contaminant exposure | Total coliform bacteria Fecal coliform bacteria Enterococcus bacteria Enteric viruses |
| Drinking Water | 2 and 3 | 4 and 5 | Contaminant exposure | Inorganic water chemistry Nutrients Organic water chemistry Total coliform bacteria Cryptosporidum Giardia |
| Fish and Shellfish Contamin-ation | 4, 5, 6, 7, and 8 | 6, 7, 8, 9 and 10 | Contaminant exposure | Fish tissue chemistry Shellfish tissue chemistry Coliform bacteria in shellfish Fecal coliform bacteria in water |
| Aquatic Life | 9, 10, 11, 12, and 13 | 11, 12, 13, 14, 15, 16, and 17 | Biological response2 | Phytoplankton Chlorophyll-a Benthic infauna (Animals that live in sediment.) Fish assemblage Fish pathology Recruitment of sensitive life stages Interstitial water toxicity Macroinvertebrate assemblage Periphyton Sediment toxicity Water toxicity |
|  |  |  | Pollutant exposure | Acid volatile sulfides/simultaneously extracted metals Debris Interstitial water metal chemistry Reporter Gene System (RGS 450) Organic and inorganic sediment chemistry Total organic carbon Shellfish or fish tissue chemistry Nutrients Turbidity Inorganic and organic water chemistry |
|  |  |  | Habitat | Dissolved oxygen Sediment grain size Sediment organic carbon Water flow Water temperature Channel morphology Residual pool volume Instream structure Substrate composition Wetland vegetation Riparian vegetation Electrical conductivity Salinity Hydrogen sulfide Ammonia |
| Sufficient Flow | 14 and 15 | 18 and 19 | Habitat | Water flow Suspended solids Channel morphology Water temperature |
|  |  |  | Biological response | Fish assemblage and populations Macroinvertebrate assemblage and populations Periphyton Wetland habitat Riparian habitat |
| Agricultural Supply | 16 and 17 | 20 and 21 | Pollutant Exposure | Organic and inorganic chemistry |
| Industrial Supply | 18 and 19 | 22 and 23 | Pollutant Exposure | Organic and inorganic chemistry Total organic carbon Temperature Electrical conductivity |
| Aesthetic Condition | 20 and 21 | 24 and 25 | Pollutant Exposure | Taste and odor Debris and trash |

Adapted from:  SWRCB, 1993; SPARC, 1997; SCCWRP, 1998; Stephenson et al., 1994; CalEPA, 1998; CABW, 1998; CDFG, 1998; Noble et al., 1999; AB 982 Scientific Advisory Group, personal communication, August, 2000.

The number refers to the monitoring objective discussed previously under site-specific and regional monitoring approaches.

2 While the assessment of invasive species is not a focus of SWAMP, these organisms will very likely be identified when biological community measurements are made.