# Minimum Health and Safety Guidelines

for SWAMP Field Activities

This information is provided to assist field personnel in the safe performance of water,sediment and tissue quality data collection.  Field work requires an awareness of potential hazards and a knowledge of basic safety procedures.  Field personnel routinely come in direct and indirect contact with waterborne pathogens, chemicals, and potentially hazardous plants and animals.  Safety is nothing more than using common sense and being aware of your surroundings.  Advanced planning can eliminate many safety hazards or at least reduce them.

## Basic Safety Preparation

Basic preparations should become routine before every sampling activity.  At a minimum, a trip plan should be completed for each field trip and left at a designated location in the office.  The trip plan should include the following information:

- Field trip participants including guests and observers with emergency contact information

- Departure and return time(s)/date(s)

- Hotel information and contact phone numbers (for overnight trips)

- Basic itinerary including where and when sampling will occur

- Phone numbers for cellular phones or radio frequencies

Field work should be done in pairs. Always carry a cellular telephone or other communication device.

Carry basic safety equipment; first aid kit, flashlight, boots, rain gear, and antibacterial soap or hand cleaner.

Be aware of changing weather conditions and the potential for flash floods, storms and/or tornados. Include reliable weather reports in the trip planning.

Be aware of potential hazards at a monitoring site.

Site access and potential hazards must be judged critically (i.e. steep or slippery banks or drops, unsafe roads or bridges, any health hazards). Only access the site, if this can be done safely.

Never enter an access-impaired site, if a safe return can not be guaranteed.

It is highly recommended to exit the last site of a day before nightfall.

If health or safety concerns arise which cannot be solved ina proper way, abort the sampling process at the site in question immediately.

Make it a habit to carry a packet of general safety information in each vehicle or boat:

- Material Safety Data Sheets (MSDS) for preservatives

- Basic first aid protocols

- Emergency phone numbers

- Locations of emergency facilities (hospitals, police and fire departments, U.S. Coast Guard)

Use the following checklist to ensure all appropriate safety equipment is available during a sampling trip.

| Basic Safety Equipment Check List |  |
| --- | --- |
| Check List | Safety Items |
|  | Waders/hip boots/rubber knee boots |
|  | Personal floatation device (PFD) |
|  | First aid kit |
|  | Fire extinguisher |
|  | Flashlight and spare batteries |
|  | Cellular phone/Marine radio (including the appropriate chargers) |
|  | Maps and GPS for position determination |
|  | Ropes (for tie downs to secure loads etc.) |
|  | Rain gear |
|  | Hat/sun screen/sunglasses/insect repellent |
|  | Drinking water/sports drinks |
|  | Safety cones/orange safety vest (working on bridges) |
|  | Tool box with basic tools |
|  | Antibacterial soap or hand cleaner/ baby wipes |
|  | Spill kits (for preservatives) |
|  | Material Safety Data Sheets (MSDS) for preservatives |
|  | Hand held eye wash unit/ gallon jugs with water for thorough flushing |
|  | Protective goggles |
|  | Container to carry preservatives |
|  | List of emergency phone numbers/office contacts |
|  | Tyvek full body suits (disposable) |
|  | Decontamination solution (10% bleach) in sprayer |
|  | Nitrile gloves |
|  | Face masks |
|  | Work gloves (i.e.to lift branches and brushwork) |
|  | Shoulder length gloves |

Personal Flotation Devices (PFDs)

Approximately 90% of all boating fatalities are drownings. Virtually all drowning victims are not wearing PFDs or are wearing inadequate ones.  All boats must be equipped with U. S. Coast Guard approved life jackets or PFDs.  The quantity and type depend on the length of the boat and the number of people aboard.  PFDs must be in good condition.  Regularly test the buoyancy in shallow water or a swimming pool.  Inspect the PFDs for weakened material or insecure snaps or zippers.  Replace spent cartridges in inflatable PFDs or tag used cartridges as out of service so they are not used accidentally.  Ensure all PFDs are the proper size for the intended wearer. Read the label to ensure it is the right size for a persons weight and chest size. Keep all PFDs readily accessible.   All sampling personnel should wear PFDs when in boats and when wading.  For boats 16 feet or longer, keep an extra Type IV PFD, in addition to those required for passengers, immediately available.

Select PFDs that are appropriate for the area being sampled.

| Type of PFD | Conditions of Use | Positives | Negatives |
| --- | --- | --- | --- |
| Type I | Offshore work or remote areas where rescure may take awhile | Excellent for flotation and will turn most unconscious persons face up in the water. | bulky |
| Type II | Near-shore vests | Good for calm waters and fast rescues. | Lack the capacity to turn wearers face up. |
| Type III * | Vests or floatation aids | Good for calm waters and fast rescues. | They will not turn an unconscious person face up and should not be used in rough waters. |
| Type IV | Throwable devices - cushions or buoy rings | Designed to be thrown to someone in trouble. | Not good for long hours in the water, rough water, non-swimmers or the unconscious. |
| Type V * | Type V or special use devices are designed for specific activities. They are only appropriate for use in accordance with the specific instructions on the label of the device. |  |  |
| Note: | * Some Type III and Type V PFDs are designed to inflate when the wearer enters the water. This type must be worn when underway to be acceptable. |  |  |

## Transporting Chemicals

Ensure that Material Safety Data Sheets (MSDS) are available for all chemicals used on a trip.  MSDS provide information on signs and symptoms of exposure, first aid procedures, and spill clean up information. Protect field staff by securing all chemicals using a container that will resist and contain the material in event of an accident.  Use an overpack container/secondary containment (e.g., ice chest) to protect against breakage and spills. Fill these with enough formiculite or cat litter to absorb all liquid. The most common and dangerous chemicals carried during sampling trips are sulfuric and nitric acids.  Carry a spill kit containing neutralizing agents in case of a spill.  Label all chemical containers clearly and have MSD Sheets available.  Include a hand held eye wash (including enough water to flush for 10 – 15 minutes!) in a chemical safety kit whenever chemicals are part of the sampling plan.  Consider using ampules of sulfuric and nitric acid in the field instead of transporting large containers of acid.  Although there is always a chance of a spill, the risk is significantly reduced by using ampules.  Do not pipette by mouth.  Always use mechanical pipetters or pipette bulbs.

If extra containers of gasoline are used, ensure that they are transported and stored in approved containers.  When filling gas cans on the bed of a metal truck, be sure to ground the container before filling with fuel. When filling portable tanks on a boat, remove them from the boat first.  Touch the fuel pipe or tank with the spout to prevent build up of static electricity.  Do not fill the tank completely full; leave room for the gasoline to expand.  Cap tanks tightly to prevent vapors from escaping.  Clean up spills immediately and air used rags before storing them. Store containers in a well ventilated area away from the engine.

A note on latex gloves: Try to avoid latex. Allergic reactions can lead to anaphylactic shock.

## Wading (USGS, 1997)

Do not attempt wading in a stream where the depth multiplied by the velocity is equal to or greater than 10 ft2/s; (Stream Depth) x (Velocity)  10 ft2/s.  For example, a stream only 2 feet deep with velocities of 5 ft2/s or more can be dangerous (USGS, 1997).

Always wear a Coast Guard approved PFD while wading.  Although the stream may not appear deep, depressions, holes, or loose footing may cause a fall.

Wear hip boots or chest waders.  Boots and waders provide protection against cold, contaminants and under water objects.  Be aware of the possibility of slipping and going underwater (feet up, head down) while wearing them. Keep in mind that waders and hip boots can contribute substantially to overheating in warm weather.

Avoid hip boots with tight ankles and waders with tight fitting tops.  They are difficult to remove in an emergency.  Wear a Coast Guard approved PFD whenever chest waders are used.

Be aware of surrounding conditions. Watch for floating debris, areas of quick sand, steep and slippery slopes and deep pools.  Watch the stream stage, especially if there is a chance it could rise rapidly.

## Working from Bridges

A significant amount of sample collection is done from bridges.  This is very dangerous so take steps  to minimize the risk.  Some basic safety equipment for bridge sampling should include reflective vests, orange safety cones, and a revolving amber light. Check with the California Highway Patrol/Dept. of Transportation, as an activated flashing or revolving light on vehicles may be required (and is advised) for short-term, short-duration (60 minutes or less) work on the road shoulder. For vehicle parking on the shoulder for longer than 60 minutes, orange safety cones should be used in addition to the flashing amber light.  The cones should begin from 40 feet before the vehicle where the posted speed limit is 30 miles per hour (mph) or less, up to 250 feet before the vehicle where the speed limit is 70 mph.

If the field vehicle is parked on the bridge, never stand in front of it while sampling.  Field staff can not see traffic and drivers can not see field staff. Set up sampling away from the vehicle where traffic from both directions can be observed.

Be aware of any boat traffic.

Wear a Coast Guard approved life jacket  when working on bridges over large rivers.  This is particularly important at sites where USGS gauges, positioned over the water, require climbing over the sides of bridges and down a ladder.

## Working from Boats

Everyone operating a boat must have completed the boating safety course of the California Department of Boating and Waterways and must be checked by the local boating Safety Officer.

Know the boats capacity.  Look for a capacity plate near the operators position or on the transom.  This plate indicates the maximum weight capacity or the maximum number of people the boat can carry safely.  Maximum weight includes the combined weight of passengers and gear.

On outboard powerboats, the capacity plate will also show the maximum horsepower rating of the boat.  Do not exceed this rating.

Use caution when refueling a boat.  Check the entire fuel system for leaks, and tighten connections frequently.  Turn off the engine and all electrical equipment before adding fuel to the tanks.  If the boat is equipped with a power ventilation system, turn it on for at least four minutes before starting the engine to clear gasoline vapors from the bilge.  Never smoke or strike a match while fueling or near a fueling dock.

Leave a  trip itinerary for each boat trip at a designated location in the office.  The plan should include :

- Date and purpose of the trip

- Name of operator(s) and any guests or observers with emergency contact information

- Destination and route to be taken

- Time of departure and estimated time of return

- Cell phone number or radio frequency

- Type of boat including color, length, identification number and any other unique features

Make sure the boat is in good operating condition and full of gas before taking it out on the water.  Use the following check list to ensure the boat is ready for use.

Check weather conditions before departure.  If a storm comes up while on the water, head for shore.  Always carry a marine radio or cellular telephone. Boating should always be done in pairs.

Do not wear waders and hipboots in a boat. They could be a safety hazard if the boat should tip or a person is thrown out.  Always wear a Coast Guard approved life jacket.

Operators of vessels involved in any collision, accident or other casualty that results in death or injury to any person or property must file a complete report of the accident.  Keep in mind that vessel operators involved in a boating accident must stop and render whatever assistance is necessary unless such actions would endanger their own vessel, crew, or passengers.  Operators must give their name, address, and vessel identification number in writing to any injured person and to the owner of any damaged property.

| Boating Safety Checklist |  |  |  |
| --- | --- | --- | --- |
| TRAILER | YES | NO | Comments: |
| Size of coupler and ball hitch match |  |  |  |
| Tire pressures are at the maximum noted on the rim |  |  |  |
| Tires treads are at least 3/32" |  |  |  |
| Tires are in good condition |  |  | Show no bulging, cracking, or tread separation |
| Brake lights and turn signals function |  |  |  |
| Safety chains are attached in an x under the coupling |  |  |  |
| All boat straps are tight |  |  |  |
| License plate is present and firmly attached |  |  |  |
| Trailer stand is secure |  |  |  |
| BOAT | YES | NO | Comments: |
| Boat plugs are present |  |  |  |
| Battery is charged |  |  |  |
| Gas tank is full |  |  |  |
| Anchor and rope are aboard |  |  |  |
| Navigation lights operational |  |  | Lights are appropriate for boat size, no other lights that may be mistaken for navigation lights are exhibited |
| Emergency paddles are aboard |  |  |  |
| First aid kit available |  |  |  |
| First extinguisher is charged and accessible |  |  |  |
| Flashlight with working batteries is available |  |  |  |
| An air horn or whistle is aboard |  |  |  |
| Rain gear is aboard |  |  |  |
| Personal floatation devices are available for every person on board |  |  |  |
| The emergency kill switch for the boat motor is functioning |  |  |  |
| Radio or cell phone is available and functioning |  |  |  |

## Electrofishing

Electrofishing is hazardous work. The batteries and generators used provide more than enough current to electrocute a person. Use extreme caution. Never electrofish alone. Ensure that everyone associated with the electrofishing efforts is aware of the hazards and safety requirements before beginning the project. Remember to notify California Department of Fish and Game of the electrofishing effort and obtain a Scientific Collection Permit before the trip, if you do not have a permit already.

Be familiar with the equipment and inspect it before each use. Correct any equipment problems immediately.If equipment must wait to be repaired, tag it Out of Service so it wont be used accidentally. Evaluate the equipment annually during a preventative maintenance inspection. Do not allow wiring splices.  If connections are necessary ensure that the rating of the connector is the same or greater than the wire. Ensure all junction boxes are weatherproof or rain tight depending on the use. Junction boxes with switching equipment must be weatherproof. Ensure that batteries used on backpack electrofishing units are a gel type that will not leak when tipped or overturned. Check hip and shoulder straps to make sure they are a quick release type, are not damaged, and are sufficient in length for the person who will use them. Ensure that the backpack unit is equipped with a tip switch that breaks the circuit if the user falls. This switch must be the type that is manually reset before reestablishing the circuit.

Ensure at least one member of the crew is trained in CPR.

All personnel must wear Coast Guard approved PFDs, waders and rubber gloves rated for a voltage above that used by the electrofishing unit when in electrofishing from the boat, with no exceptions.  Rubber knee boots are also recommended for boat electrofishing.

Wear waders and rubber gloves rated for a voltage above that used by the electrofishing unit when backpack electrofishing.

Inspect all nets to ensure they are made of non-conductive material and that they are long enough to keep the users hands out of the water.

## Potentially Contaminated Water

Always consider the possibility that the water being sampled may be contaminated with pathogens or hazardous chemicals.  Use caution and extra protection when working in or around water with known or suspected contamination.

- Frequent routine hand washing (oral route of exposure) is one of the most important safeguard in preventing infection by biological agents however remotely possibly that may be present in the sediment and water samples taken in the field and analyzed in the laboratory.  Another equally important safeguard is to control the generation of aerosolized droplets (inhalation route of exposure) created by mechanical devices such as pumps, pipettes, and centrifuges.

- To maintain proper hand hygiene soap and water should be used for hand washing. If soap and water are unavailable or if the situation is not practical, baby wipes can be used instead.  One small zip bag full of disinfectant type baby wipes should be kept by all samplers. Open the bag take out a few wipes, wipe down the outside of the bag, close the bag, then wiped down the potentially contaminated skin surface. Prior to eating, drinking, smoking, applying makeup, or using the bathroom hands must be washed. Baby wipes should also be used in the field anytime accidental splashes of contaminated soil or water happen to make contact with the skin.  During a significant splash episode sampling should cease and the area of the skin contacted should be thoroughly decontaminated.  Insignificant splashes should be controlled by using baby wipes containing disinfectants (quaternary ammonium compounds).

- See table C at the end of this document for a list of common disinfectants and their properties.

- Special protective clothing should be worn when working with samples potentially contaminated with chemical and/or biological agents. Tyvek or other suits (I.e. Polypropylene suits, biological/chemical resistant gloves, face shield or goggles, and possible voluntary use of filtering face piece (N-100 dust mask in high risk areas where potential aerosols are possibly present) are recommended. Neoprene boots might be useful for for employee comfort.

- Decontaminate all gear and clothing (with 10% bleach solution for example) after sampling at a potentially contaminated site. Disposable personal protective equipment (PPE) like suits and gloves sould be sprayed with disinfectant solution and then bagged for disposal.  At t the end of a shift recyclable PPE (face shields, safety eyewear, boots) will be sprayed with a disinfectant solution. This solution should be allowed to stay on the surfaces for some time (10-30 minutes when using a 3% chlorine base solution like bleach [bleach using contains approximately 12.5% sodium hypochlorite but is diluted down to a 1:10 dilution to control bacteria and viruses).  Never should potentially contaminated clothes be worn outside your work area.  Prior to leaving the specific sampling site you must decontaminated yourself and clothing to prevent the possible spread of chemicals and biological agents adhering to your clothing.

- Soil should be removed from field sampler’s shoes prior to entering an enclosed vehicle.  Vehicle mats should be cleaned regularly to remove dirt.  Failure to do so can result in soil sediments drying on the mats, and when the air condition or heater is turned on this dust becomes airborne, making it possible to be inhaled.

- Use sample tags to indicate the level of contamination so the laboratory can handle the sample appropriately.

- Communicate known or suspected contamination to all personnel who could come in contact with a contaminated sample.

Waterborne, disease causing organisms (pathogens) are found in nearly all surface water systems.  Pathogens enter surface water through untreated sewage discharges and bypasses, storm and agricultural runoff, and direct contact (see table A at the end of this document for a list of organisms which are occasionally present in wastewater and sewage sludge). Bacteria, viruses and other pathogens can occur in the most pristine environments.

- Never drink sample water, no matter how pristine the environment appears. Consider making antibacterial soap, baby wipes or hand cleaner a routine item carried on all field trips.

Blood and other bodily fluids do enter the sewage waste stream and hepatitis B and the HIV virus can be present but only in very dilute concentrations (HIV concentration in sewage is several orders of magnitude less than that of hepatitis B).   But we must keep in mind that hepatitis B and HIV are both blood borne pathogen and cannot be transmitted by the oral-fecal route.  The risk of contracting hepatitis B from sewage if virtually non-existent and the risk of HIV is even less.

As a further precaution, a medical monitoring program is recommended for all sampling personnel.

- This program should include an annual health history and physical, respiratory monitoring, hepatitis A vaccines, and a tetanus vaccine with a booster every 10 years, at a minimum.  (Note: MLML does not do this regular checkup.) Note: The Division of Communicable Disease Control in the California Department of Health Services recommends only tetanus-diphtheria (Td) [basic series of three immunizations] for all sewage workers. This immunization should be updated with a booster every ten years.  The Division does not recommend any other immunization at present.  There are presently no state immunization requirements mandated for sewage workers.

See table B at the end of this document for a list of bio agents, their effects and treatments.

A note on Hand Washing:

Hand washing is the best way to ensure that infectious agents are not transferred. When hand washing is done right its one of the most effective way to control disease. Hand washing is the act of vigorously rubbing together all surfaces of the lathered hands for at least 10-20 seconds, which is then followed by thorough rinsing under a stream of warm water. Rinse well and remember to use a paper towel to turn off the faucets to prevent contamination from reoccurring.

Washing hands with plain soaps or detergent (non-antimicrobial) suspends microorganisms and allows them to be rinsed off; this type of hand washing is described as mechanical removal of microbes.  Antimicrobial soaps are defined as chemical removal of microorganism where microbes are killed or inhibited from growing.

The skin is an effective barrier for both infectious and noninfectious bacteria.  On the skin of our hands we have both resident and transient microorganisms.  The resident microorganism are usually not highly virulent, nor implicated in infections.  Most resident microbes can be found in the upper superficial skin layers, but about 10-20% can inhabit the deeper epidermal layers. Hand washing with plain soaps or detergents is very effective at removing most resident microorganisms except for those resident microbial flora that reside deep in the epidermal layers. These resident microorganisms can be killed or inhibited by using antimicrobial soaps or detergent. Transient bacteria are usually passed through skin contact and sometimes contain pathogenic bacteria. Transient microbes hitchhike on the skin but are easily removed through mechanical action. Most transient organisms are washed off during hand washing using plain soaps or detergent.

The use of antiseptic soaps provides no additional benefit over ordinary soaps.  It is well document in the literature that hand washing with plain soap and water significantly reduces the number of potential pathogens on the hands and reduces the chance of infection.

Recent studies suggests that washing hands repeatedly with antimicrobial soaps or detergents can make the microbial flora of the hands resistant to antimicrobial agents (i.e. triclosan) and thus reduce their ability to kill or inhibit bacteria. Antimicrobial soaps and detergents should be used selectively to reduce the chance of bacterial resistance.

If you wear gloves, its still important to wash your hands because gloves may become abraded with pinhole or micro tears during use and thus allowing bacteria to grow rapidly inside the glove. The warm and moist environment inside the glove creates a perfect place for bacteria to thrive. Wearing gloves is not an alternative to washing your hands.  Hands must be washed every time gloves are removed.

Skin conditioning is very important after hand washing. The use of hand lotions may be required, especially if frequent hand washing occurs throughout the day. Repeated hand washing can result in dry skin and alter the normal pH of the skin, which in turn can affect the ability of the skin to act as an effective barrier against microbial intrusion. The use of aqueous based lotions can help restore, maintain, and improve the skin’s integrity. Hand towelettes and cleansing foams are not recommended because they lack the required dilution and detergent action.

If bar soaps are used, they need to be placed on racks so the water can drain away from the bar of soap.  Some liquid soap containers have a disadvantage in that they may serve as a reservoir of microbes. If reusable soap containers are used, they must be cleaned when they are empty and refilled with fresh soap.

## Weather

Weather can change rapidly and create unexpected situations for sampling personnel whether in a boat or in isolated sampling areas.  Check local weather forecasts frequently.  Be alert to visual weather cues such as developing clouds, wind shifts, and greying skies.

When in a boat head for shore immediately.  Head the bow into the waves at a 45 degree angle.  Reduce speed, but keep enough power to maintain headway.  Seat passengers on the bottom of the boat, as close to the centerline as possible.

Leave small creeks and rivers to avoid flash floods.  Dont cross low water crossings as the integrity of the underlying roadway is uncertain.  Floating debris may damage the vehicle or even push it from the road way.

### Temperature Exposure

Extremes of air temperature occur in all parts of the country.  The ideal comfort range for humans is between 10 to 32C (60 to 90F).  Hypothermia (cold) and hyperthermia (heat) normally occur outside this range.

### Cold Emergencies

Hypothermia is a condition of reduced body temperature caused by exposure to cold, and aggravated by wet clothes, wind, hunger and exhaustion.  Hypothermia can occur with air temperatures above 16C (60F) under wet and/or windy conditions.

| Warning Signs | Uncontrollable fits of shivering, incoherence, listlessness, fumbling hands, frequent stumbling, drowsiness, and the inability to get up after resting. |
| --- | --- |
| Treatment | Remove person from cold and get to a dry warm place. Replace wet clothes with dry. Warm body slowly. Give warm non-alcoholic drinks. These are temporary measures until medical help is available. |
| Prevention | The best way to prevent hypothermia is to stay warm and dry. Put on rain gear before it rains. Dress in layers and add more before getting cold. Find shelter before conditions become severe. During colder weather carry a complete change of dry clothes. |

### Heat Emergencies

Hyperthermia is caused by increasing body temperature due to exposure to extreme heat.   Heat emergencies can be brought about by a combination of factors; physical exertion, clothing (waders), humidity, no breeze, air temperature and the rate of fluid intake.  Working in the extreme summer heat creates a very real threat of suffering from some form of heat related stress.

| Warning Signs | Chilling, headache, unsteadiness, dizziness, nausea, dry skin (either hot and red [heat stroke] or cool and pale [heat exhaustion]), rapid pulse and muscle pain/spasms. |
| --- | --- |
| Treatment | General treatment for heat emergencies is cooling down and drinking plenty of fluids. Do not take salt tablets. A common symptom of dehydration is a headache. Heat stroke requires immediate medical attention and can cause death. Cool down victims of heat stroke quickly and watch for signs of shock. Call 911 or, if in an isolated area, transport the victim to a medical facility immediately. |
| Prevention | Hydrate well before working outdoors. Drink water in moderate amounts every fifteen minutes. Do not rely on thirst to indicate dehydration. |
|  | Avoid alcohol, caffeine, and soda. These liquids are not water substitutes and can increase the rate of dehydration. |
|  | Wear lightweight, light colored clothing and a wide-brimmed hat. |
|  | Schedule activities that require the most exertion during early morning or late afternoon hours. |
|  | Find some shade and take breaks during the day. |

## Plants and Animals

Insects, reptiles and certain plants are always potential hazards for field personnel.  The following is a summary of general information on the most common plant and animal hazards encountered by field staff (American Red Cross 1988; USGS, 1997; Milne and Milne, 1980; and Behler and King, 1979).

| Animals | Descriptions/Characteristics | Procedure (ARC 1988; USGS 1997) |
| --- | --- | --- |
| SPIDERS, SCORPIONS and OTHER INSECTS (Milne and Milne, 1980; USGS, 1997) |  |  |
| Black widow spiders | Female (only one that bites) is black with almost spherical abdomen, usually with red hour glass mark below or with 2 transverse red marks, separated by black. Inhabit fallen branches and under objects. Red and brown widow spiders are less common but inhabit the western U.S. (Size: up to 1 ½” long) | Take care when reaching into small dark spaces. If bitten by a black widow or brown recluse, seek medical attention as soon as possible. Scorpion stings may not require medical attention. |
| Brown recluse spiders | Orange-yellow thorax with dark violin pattern. Bases of legs orange-yellow, rest of legs grayish to dark brown. Abdomen grayish to dark brown with no obvious pattern. Frequent areas of human habitation and prefers dark spaces. Found outdoors in sheltered corners, among loose debris; indoors on the floor and behind furniture. (Size: ¼ to ¾”) |  |
| SPIDERS, SCORPIONS and OTHER INSECTS (Milne and Milne, 1980; USGS, 1997) |  |  |
| Scorpions | Lobster-like pincers with a long up curved tail that ends in a poisonous stinger. Nocturnal, sensitive to vibrations. Frequent the desert. Not easily seen in the wild. Field boots are a favorite hiding place. Most scorpions are not dangerous and do not attack. Poison of most North American species is not lethal to humans but they do inflict a painful sting. |  |
| Ticks | Small, less than 3 mm (< 1/8 inch). Clamps to host using dart-like anchor located just below the mouth. Wear long pants and tuck pants legs into socks and use a repellent containing the compound DEET (N-diethyl-meta-toluamide). |  Check for ticks during and after field work.  Remove with tweezers within 24-hours.  Wash and disinfect the bite. |
| Bees | Bees vary in size from 2 mm (0.08 inches) to 4 cm (1.6 inches) long. Locations vary from ground nests to trees and manmade structures. |  Avoid bee hives and wasp nests.  Scrape off the stinger with a knife or other flat object (i.e., credit card).  Wash well with soap and water.  A cold pack may be used to reduce swelling.  Use an over the counter sting ointment or solution of water and baking soda. |
| Wasps | Wasps vary in size from minute to 5 cm (2 inches) long. Adults have a narrow waist between the first and second abdominal segments. Locations vary from ground nests to trees and manmade structures. |  If a member of a field team is allergic to insect bites or stings this should be made known to the rest of team. That person should carry a sting kit for use in emergencies.  Symptoms of an allergic reaction include: pain, swelling of the throat, redness or discoloration in the area of the sting, itching, hives, decreased consciousness, or difficult or noisy breathing. |
| SNAKES and ALLIGATORS (Behler and King, 1979; USGS, 1997) |  |  |
| Copperhead | Description: Stout body; copper, orange, or pink tinged with bold chestnut or reddish brown crossbands narrowing in the middle of the back. Top of head unmarked. Habitat: Wooded hill sides with rock outcrops above streams or ponds, edges of swamps and periodically flooded coastal plains; near canyon springs and dense river cane stands along the Rio Grande. Seen basking during fall and winter months but more nocturnal during warm weather. Favorite warm weather habitats include stonewalls, piles of debris, rotting logs, and large flat stones near streams. Hazard: Copperhead bites are painful but rarely life threatening. | Best defense is to avoid them. Most snakes will go the other way unless unusually agitated or disturbed. Take care when electrofishing and seining near log jams, fallen trees and undercut banks. |
| Cottonmouth (water moccasin) | Description: A dark, heavy-bodied water snake. Broad-based head noticeably wider than neck. Olive, brown or black above; patternless or with jagged-edged dark crossbands. Top of head is flat; eyes not visible from directly above as in other harmless water snakes. Unlike other water snakes, it swims with head well out of water. Never far from water. Most active at night although may be seen sunning during the day. Habitat: Lowland swamps, lakes, rivers, bayheads, irrigation ditches, canals, rice fields, to small clear rocky mountain streams. Do not disturb. Hazard: Bite more serious than that of a copperhead and can be fatal. When disturbed, tend to stand its ground exposing the light cotton lining of its mouth. | If bitten: Do  Reassure the victim  Treat for shock. Keep victim lying down; elevate feet 10 to 12 inches.  Seek medical attention as soon as possible. Do Not  Cut and suck bite area.  Apply ice or a tourniquet  Leave victim unattended. |
| Rattlesnakes | Description: Heavy-bodied with heads distinctly wider than the neck. Most have blotches or crossband patterns on the back. (The diamondback has black bordered diamond patterns or hexagonal blotches. Two diagonal lines on side of face. Tail encircled by broad black and white rings.) They have recurved, retractable, hollow fangs near the front of the upper jaw. Rattlesnakes have a distinctive rattle on the tail. Habitat: Arid and semiarid area from plains to mountains; brushy desert, rocky canyons, bluffs along rivers, sparsely vegetated rocky foothills. Hazard: Western diamondback rattle snake is capable of delivering a fatal bite. When disturbed it normally stands its ground, lifting its head well above the coils. The warning is a buzzing sound. |  |
| Coral snakes | Description: Distinctly colored with wide red and black bands separated by a narrow, bright yellow or white band. Red and black bands never touch. Several harmless snakes have similar color patterns (scarlet kingsnake and scarlet snake). The head is black and the snout is blunt. Habitat: Moist densely vegetated upland areas near ponds or streams in hardwood forests; rocky hillsides and canyons. Usually seen under rotting logs or leaves. May be seen moving on the surface in early morning or late afternoon. Hazard: Unlike the vipers and pit vipers, enlarged grooved fangs are fixed in position on the front part of the upper jaw. Coral snakes must chew prey. The venom is a strong neurotoxin and bites can be fatal. | If bitten: Do  Reassure the victim  Treat for shock. Keep victim lying down; elevate feet 10 to 12 inches.  Seek medical attention as soon as possible. Do Not  Cut and suck bite area.  Apply ice or a tourniquet  Leave victim unattended. |
| Alligators | The American alligator is found in SWAMPS, rivers, and lakes mainly in the east and southeastern portions of CALIFORNIA...DID YOU READ THIS? I'LL KNOW NOW...GO GATORS!!!! There you have your justification for our new SWAMP logo! | Treat alligators with extreme caution. Never approach an alligator on land or in water. During breeding/nesting season alligators can be very aggressive (April and May). Alligators can outrun humans for short distances. If sampling involves fish collection, get the specimens out and away from the water as soon as possible. |
| PLANTS |  |  |
| Poison Oak and Poison Ivy | Climbing poison ivy has alternate, trifoliate leaves with aerial roots that grow straight and are fuzzy. Found in most environments. Non-climbing poison ivy lacks aerial roots. The leaves are the same shape as the climbing poison ivy but are larger and broader. Poison oak is prolific in California, and like Poison ivy, has alternate, trifoliate leaves. These leaves also turn orangish-red in the fall. Poison oak plants are bushy and twiggy, and are often found along trails, along fence lines; they can also be found intertwined around large trees. Vines or twigs/branches without leaves can still cause a case of poison oak or poison ivy. If a piece of vine or twig/branch is used as fire wood, the oily resins can be released into the air. The resin can also remain on unwashed clothing and equipment. | Do Flood the affected area with lots of cold water as soon as possible. Since the oily resin is only slightly soluble in water, a little water will only spread the poison. Use anti-itch cream. People allergic to poison ivy or poison oak may require medical attention. Consider using pre-exposure lotion which creates a barrier against poison ivy, oak and sumac oils. Poison oak and ivy cleansers are available that can be used up to 8 hours after exposure. Wear Tyvek suits, if contact might occour. Carry treatment lotion (Tecnu) and apply as son as possible after contact. Do Not Use hot water or soap. These help increase the affects of poison ivy and poison oak. |

Tables

#### Table A: Potential Pathogens found in Wastewater and Sewage Sludge

Enteric Bacteria

| ORGANISM | DISEASE | SYMPTOMS |
| --- | --- | --- |
| Campylobacter jejuni | Gastroenteritis | Diarrhea, abdominal pain, malaise, fever, nausea, and vomiting |
| Escherichia coli | Gastroenteritis | Same as above |
| Salmonella sp. | Salmonellosis | Food poisoning to typhoid fever |
| Shigella sp. | Bacillary dysentery | Diarrhea, fever, nausea, vomiting, cramps, convulsions |
| Vibrio cholera | Cholera | Watery stools, vomiting, rapid dehydration, acidosis, circulatory collapse |
| Yersinia sp. | Acute gastroenteritis | Diarrhea, abdominal pain, vomiting, headaches, anorexia, arthritis |

Enteric Viruses

| ORGANISM | DISEASE | SYMPTOMS |
| --- | --- | --- |
| Autoviruses | Epidemic gastroenteritis | Diarrhea, fever, nausea, vomiting, headache, malaise |
| Caliciviruses | Epidemic gastroenteritis | Same as above. |
| Entroviruses-Coxsackieviruses | Meningitis, hepatitis, herpes simplex | Sudden onset, fever, sore throat, lesions of the tonsils and palate |
| Entroviruses- Echoviruses | Meningitis, Cat-Scratch, Pink-eye, Myalgia | Diarrhea, fever, paralysis |
| Entroviruses- Polioviruses | Poliomyelitis | Fever, malaise, headache, nausea, vomiting, excruciating muscle pain and spasms, stiffness of the neck and back, eventual paralysis |
| Hepatitis A virus | Infectious hepatitis | Fever, malaise, anorexia, nausea, abdominal pain, jaundice |
| Norwalk viruses | Epidemic gastroenteritis | Nausea, vomiting, diarrhea, abdominal pain, myalgia, headache, malaise |
| Reovirus | Rotaviral enteritis | Diarrhea, vomiting, severe dehydration |

##### Enteric Protozoa

| ORGANISM | DISEASE | SYMPTOMS |
| --- | --- | --- |
| Balantidium coli | Balantidiosis | Diarrhea, dysentery, nausea, vomiting |
| Cryptosporidium | Gastroenteritis | Vomiting, anorexia, mucoid diarrhea, vomiting, abdominal pain, cough, pulmonary infections |
| Entamoeba histolytica | Amebiasis | Fever, chills, bloody or mucous diarrhea, abdominal discomfort |
| Giardia lamblia | Giardiasis | Diarrhea, abdominal pain, cramps, weight loss |
| Toxiplasma gondii | Toxoplasmosis | Fever, pneumonia, rash |

##### Enteric Helminth Worms

| ORGANISM | DISEASE | SYMPTOMS |
| --- | --- | --- |
| Ascaris lumbricoides | Ascariasis (roundworm infection) | Abdominal pain, digestive and nutritional disturbances, restlessness, vomiting |
| Ascaris suum | Ascaridiasis | Fever, irregular and asthamatic breathing, spasmodic coughing |
| Hymenolepsis nana | Hymenolepidosis (dwarf tapeworm infection) | Enteritis, with or without diarrhea, abdominal pain, pallor, loss of weight, and weakness |
| Necator americanus | Necatoriasis (hookworm disease) | Anemia, headache, fatigue, epigastric discomfort |
| Taenia saginata | Taeniasis (beef tapeworm disease) | Nervousness, insomnia, anorexia, loss of weight, abdominal pain, digestive disturbances |
| Taenia solium | Taeniasis (pork tapeworm disease) | Same as above |
| Toxocara canis | Toxocariasis (larva migrans visceralis) | Fever, fits of coughing, nausea, vomiting, dyspnea |
| Trichuris trichiura | Trichocephaliasis (whipworm disease) | Abdominal pain, distension, diarrhea |

###### Table B: Bio Agents, Effects and Treatments

| BIO-AGENT Scientific Name (common reference) | SIGNS & SYMPTOMS | ROUTE OF EXPOSURE | TOXICITY | ONSET OF DISEASE/PERSISTENCE | DECON | TREATMENT |
| --- | --- | --- | --- | --- | --- | --- |
| Clostridium botulinum (botulism) [bacteria] | Nausea, vomiting, difficulty speaking, seeing, and swallowing; drooping eyelids; muscle weakness in the trunk and limbs; eventual muscle paralysis and difficulty breathing | Inhalation, Ingestion, Doesn’t penetrate the skin. Not contagious. | LD50 = 0.00001 mg/kg; 1 gram of toxin dispersed evenly and inhaled would kill one million people | 2-12 hours after exposure Anaerobic, natural habitat is the soil; extreme temperatures, humidity, and acidic conditions will degrade the toxin. | Toxin deactivates at 850C in 5 mins; toxin is neutralized by chlorination; bleach 0.1% solution | No vaccine for the general population; antitoxin available |
| Francisella tularensis (tularemia) [bacteria] | Fevers, chills, headaches, | Inhalation of contaminated dust. Ingestion - of contam- inated meat Injection- insect bites by flies, ticks, and mosquitoes. Direct contact of skin or mucous membranes with infectious materials | Infectious dose 5-10 organisms by inhalation route. Infectious dose 1M-100M organisms by ingestion. | 1-14 days Human infection is rare. Survives outside the host for 133 days on carcasses and organs, 136 days on grain and dust, and 90 days in water. | 1% bleach, 70% ethanol, glutaraldehyde, formaldehyde; moist heat 1210C at 15 mins; 1700C at 1 hour. | Antibiotic therapy within 24 hours may prevent disease. No vaccine for general public. Vaccine available for high risk workers. |
| Yersinia pestis (bubonic plague) [bacteria] | Fevers, chills, headaches, weakness, nausea, vomiting, abdominal pain, extreme lymph node pain; chest pain coughing, bloody or watery sputum, and septic shock. | Inhalation of respiratory droplets. Injection by infected fleas or rodent bite. Contagious by respirable aerosols. Not usually transmitted human to human unless pneumonia or contact with pus. | Infectious dose is a few virons. 50 kg (aerosol) over 5M pop- 150,000 will contract the disease and only 36,000 will die. | 1-6 days after exposure. Fleas remain infective for months under situable conditions of temperature and humidity. Bacteria may survive in soil from some time. | Insecticide to control fleas on animals. Bacteria very sensitive to light and heat. Bacteria doesn’t survive long outside of host. | Antibiotics within 3-5 days after exposure. Vaccine 2-3 doses not for general population. Boosters injection required. |
| Clostridium tetani (tetanus) [bacteria] | Painful muscular contractions, primarily of the neck muscles, then trunk muscles, abdominal rigidity. | Injection-puncture wound with contaminated soil or feces. Also through lacerations, burns, and trivial wounds. Not transmitted human to human. | Toxin extremely potent. 30-90% fatality rate. | Incubation period: 3-21 days, average 10 days. Survival outside the host- manure several days, soil spores survive long periods, textiles-730 days. | Spores resistant to many disinfectant, moderate susceptible to bleach, susceptible to high level with glutaraldehyde. Spores fairly resistant to heat but are destroyed after 15 mins contact with moist heat 1210C. | Tetanus immune globulin (TIG) as soon as following the wound. Vaccination good for 10 years. Need booster dose of toxoid if more than 10 years. |
| Salmonella typhi (typhoid fever) [bacteria] | Fever, headache, malaise, enlarged spleen, constipation; severe abdominal pain, rose spots on trunk. | Ingestion of food or water contaminated by feces or urine containing infectious bacteria. Shellfish contaminated by raw sewage. Raw fruits and vegetables, and milk products by hands of carriers. Files can infect foods. | Infectious dose: 100,000 organisms. | Incubation period depends on the size of infecting dose; usually 1-3 weeks. Survival outside the host: ashes-130 days, dust- up to 30 days, feces up to 62 days, skin 10-20 mins. | Susceptible to many disinfectant. 1% bleach, 70% ethanol, 2% glutaraldehyde, iodines, phenolics, and formaldehyde. Physical inactivation: sensitive to moist heat (1210C for 15 min). | Antibiotics. Thypoid vaccine available. Two vaccines several weeks apart. Reinforcing injections every 3 years. |
| Coccidioides immitis (San Joaquin Valley Fever) [soil inhabiting mold that produces a spore] | Flu-like illness, vague feeling of bodily discomfort, chest pain, inflammation of nasal passages and palate, bronchitis, dry cough, loss of appetite, joint pain, fever, measle-like rash, severe muscular ache of the back and head. | Inhalation of spores from airborne soils. Inhalation of spores from fungal cultures. Accidental inoculation. Not contagious from human to human. | Outer wall of spore is toxic to mammalian cells. | Most signs and symptoms of illness start between 10-14 days. |  | Anti-fungal agents (amphotercin B and ketocona-zole) for six months or longer. Antibiotics. Skin test 1-3 weeks after onset of symp- toms. Blood tests serologic testing. X-rays for scarring. |
| Borrelia burgdorferi (Lyme disease) [bacteria] | Flu-like symptoms fever, chills, muscle aches, joint pains, neck stiffness, headaches, bulls-eye rash. Late cardiac, neurologic or arthritic complications. | Injection from blood feeding ticks (western black-legged tick). No human to human transmission. |  | Incubation period: 3-32 days after tick exposure | Insecticides (permethrins) applied to clothing can control the tick. | Commonly available antibiotics. Vaccine available (Lymerix, 3 shots, booster once a year; protects 80% of people vaccinated). Serological tests can show a rise in anti- bodies. |
| Vibrio cholera (cholera) [bacteria] | Severe diarrhea, vomiting, weakness, leg cramps, rapid loss of body fluids that can lead to weak pulse and shock. | Ingestion of food or water contamin- ated with bacteria. Fecal-oral route. Not contagious with proper hygiene, | 1M-100,000M organisms by ingestion route is the infectious dose. 20-50% of cases are fatal if untreated, cholera toxin causes massive secretion of electrolytes and water into the intestine. | Incubation period: Few hours to 5 days, usually 2-3 days. Liquid stools to shock in 4-12 hours with death in 18 hours to several days. Dust 3-16 days, Feces up to 50 days, Soil up to 16 days, Water survives well in water depends on temperature. | Chlorination of water. Cook foods thoroughly 650C for 10 mins. Acids destroy bacteria. | Antibiotic. Vaccine use to be available. Fluid and electrolyte replacement. Patients that recover have solid immune for 3 years. |
| Hepatitis A [virus] | Fever, malaise, nausea, and abdominal discomfort, followed in a few weeks by jaundice. Mild illness lasting 1-2 wks, rarely to a severely disabling disease lasting several months. | Ingestion of food or water contaminated with fecal contaminated with hepatitis A. Contact with infected persons. |  |  |  | Vaccine available. Two shots, one initially and then a second shot 6-12 month later. Post-exposure prophylaxis. Immune globin (IG) given within 14 days of exposure, is 80-90% effective in preventing the disease. |

Table C: Properties of common disinfectants

Practical Requirements For Use

| DISINFECTANTS | CONCENTRATION OF ACTIVE INGREDIENTS | FOR LIPOPHILIC VIRUS ONLY (contact time in mins.) | BROAD SPECTRUM (contact time in mins.) | TEMPERATURE, 0C | RELATIVE HUMIDITY % |
| --- | --- | --- | --- | --- | --- |
| Quaternary ammonium compounds | 2% | 10 | Not Effective |  |  |
| Phenolic Compounds | 2% | 10 | Not Effective |  |  |
| Chlorine Compounds | 3% | 10 | 30 |  |  |
| Iodophor | 2% | 10 | 30 |  |  |
| Ethyl Alcohol | 85% | 10 | Not Effective |  |  |
| Isopropyl Alcohol | 85% | 10 | Not Effective |  |  |
| Formaldehyde | 8% | 10 | 30 |  |  |
| Glutaraldehyde | 2% | 10 | 30 |  |  |
| Ethylene Oxide | .45 g/l | 60 | 60 | 37 | 30 |
| Paraformalde--hyde | 10 g/m3 | 60-120 | 60-120 | >24 | >50 |

Agent Inactivated

| DISINFECTANTS | VEGETATIVE BACTERIA | LIPOPHILIC VIRUS | HYDROPHILIC VIRUS | BACTERIAL SPORES |
| --- | --- | --- | --- | --- |
| Quaternary Ammonium Compounds | Yes | Yes |  |  |
| Phenolic Compounds | Yes | Yes | Variable results dependent on virus |  |
| Chlorine Compounds | Yes | Yes | Yes | Yes |
| Iodophor | Yes | Yes | Yes | Yes |
| Ethyl Alcohol | Yes | Yes | Variable results dependent on virus |  |
| Isopropyl Alcohol | Yes | Yes | Variable results dependent on virus |  |
| Formaldehyde | Yes | Yes | Yes | Yes |
| Glutaraldehyde | Yes | Yes | Yes | Yes |
| Ethylene Oxide | Yes | Yes | Yes | Yes |
| Paraformalde-hyde | Yes | Yes | Yes | Yes |

Major Characteristics

| DISINFECTANTS | EFFECTIVE SHELF LIFE > 1 WEEK, Protect from light and air | CORRO-SIVE | FLAM-MABLE | EXPLOSION POTENTIAL | RESIDUE | INACTIVATED BY ORGANIC MATTER | LENS COMPATI-ABILITY |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Quater-naryAm-monium Compounds | Yes |  |  |  |  | Yes | Yes |
| Phenolic Compounds | Yes | Yes |  |  | Yes |  |  |
| Chlorine Compounds |  | Yes |  |  | Yes | Yes |  |
| Iodophor | Yes | Yes |  |  | Yes | Yes |  |
| Ethyl Alcohol | Yes |  | Yes |  |  |  |  |
| Isopropyl Alcohol | Yes |  | Yes |  |  |  |  |
| Form- aldehyde | Yes |  |  |  | Yes |  |  |
| Gluatar- aldehyde | Yes |  |  |  | Yes |  | Yes |
| Ethylene Oxide | Not Applicable |  | Yes | Yes |  |  | Yes |
| Paraform- aldehyde | Not Applicable |  | Yes | Yes |  |  | Yes |

Application

| DISIN- FECTANT | WORK SURFACE | DIRTY GLASS-WARE | LARGE AREA DECON | AIR HANDLING SYSTEM | PORTABLE EQUIP., SURFACE DECON | PORTABLE EQUIP., PENETRATING DECON | STATIC EQUIP., SURFACE DECON |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Quater-nary Ammon-ium Compounds | Yes | Yes |  |  | Yes |  | Yes |
| Phenolic Compounds | Yes | Yes |  |  | Yes |  | Yes |
| Chlorine Compounds | Yes | Yes |  |  | Yes |  | Yes |
| Iodophor |  |  |  |  |  |  |  |
| Ethyl Alcohol | Yes | Yes |  |  | Yes |  | Yes |
| Isopropyl Alcohol | Yes | Yes |  |  | Yes |  | Yes |
| Form- aldehyde | Yes | Yes |  |  | Yes |  | Yes |
| Glutar- aldehyde | Yes | Yes |  |  | Yes |  | Yes |
| Ethylene Oxide |  |  |  |  |  | Yes |  |
| Paraform- aldehyde |  |  | Yes | Yes |  | Yes |  |