# DWR Comments on DEIR for Implementation of

- 1995 Bay / Delta Water Quality Plan

The Department of Water Resources has reviewed the Draft Environmental Impact Report for Implementation of the 1995 Bay/Delta Water Quality Control Plan.    The Department has grouped its comments into three categories.    First, we have a set of general comments that relate to how the DEIR is organized and its assumptions.    We make suggestions on ways to restructure the document consistent with the SWRCB's CEQA obligations and to facilitate comprehension by the general public.    Second, we have specific substantive comments beginning on page 4 regarding impact analysis, mitigation measures, and conclusions reached in the DEIR.    We again offer suggestions where appropriate on how to modify the document consistent with the SWRCB's CEQA obligations.    Finally, we have a set of editorial suggestions and minor, technical changes beginning on Page 37 that we recommend be incorporated in the FEIR for accuracy.

### GENERAL COMMENTS

- 1. Alternatives

DWR believes that the SWRCB has fulfilled its CEQA obligation of analyzing a reasonable range of alternatives for the various components of the program.    We agree with the approach in the DEIR of not selecting a preferred alternative for each component at this stage of the process.    We understand the SWRCB will select a preferred alternative in the FEIR.

For the FEIR, we have recommended a preferred alternative for some components of the program and we have offered reasons for this selection in our specific substantive comments below.    For the flow objective alternatives, we are not recommending a preferred alternative at this time.    We continue to support negotiated flow arrangements, and will be submitting testimony about the status of the negotiations at the April 21st workshop.    During the SWRCB hearing, we will recommend a flow alternative to help implement the Plan for non-settling parties consistent with the SWRCB's public interest responsibilities in the Bay-Delta and the obligation to consider all water right holders affecting the Delta.

For the Suisun Marsh, we recommend Alternative 5 with indicated modifications that update the description of the alternative.    We clarify that the actions proposed in the Suisun Marsh Preservation Agreement Amendment III do not include Green Valley Creek flow augmentation nor redirection of Morrow Island drainage to Suisun Bay.    For the Salinity Control Alternatives in the San Joaquin Basin (Vernalis), we recommend a negotiated settlement.    For the South Delta (excluding Vernalis), we recommend alternative 3, which is an implementation of the South Delta flow control structure of the ISDP program.    For the dissolved oxygen objectives, we recommend a combination of alternative 3, ISDP program, and alternative 4, discharge improvements from the Stockton treatment plant.    We believe it is significant to note that the ISDP program may serve as the preferred alternative for two components of the program.    For the joint point, we recommend alternative 5, which authorizes the joint point for all purposes consistent with the existing operating criteria.    The implementation of the joint point will be contingent on DWR and the USBR completing satisfactory agreements that set forth conditions under which the combined use of SWP and CVP points of diversion could occur.    Finally, DWR supports the pending petition before the SWRCB to consolidate the Bureau's place of use, because it will increase the flexibility afforded to the Bureau in managing CVP supplies.

- 2. Impacts associated with Water Quality Objectives

The 1995 Bay/Delta Water Quality Control Plan established objectives to protect the Delta environment.    We believe it is important to stress that the impact associated with establishing the objectives and the environmental impacts associated with these objectives were already fully addressed in the 1995 Environmental Report for the 1995 Bay/Delta Water Quality Control Plan.    This DEIR properly did not reexamine these issues.

- 3. Mitigation

We believe that certain sections of the DEIR pertaining to mitigation measures can be improved.    The reader of the document should be able to determine whether a particular impact is significant or not, whether mitigation measures can be implemented to substantially mitigate or avoid the impact, and what entity will be responsible for carrying out the mitigation measures.    A conclusion that this impact is "significant" should be supported by analysis in the document.    If the conclusion is that a particular impact is "not significant," that should be supported and stated clearly as well.    Conclusions in the DEIR that no mitigation measures are available for certain impacts appear to us to be unsubstantiated.    We have suggested mitigation measures where appropriate for some of these impacts.    It should be noted that because mitigation measures are available, it does not imply that the SWRCB is necessarily responsible for implementation.    CEQA permits the lead agency to find that mitigation measures are under the jurisdiction of other agencies and that those agencies should adopt them.    (Public Resources Code sec. 21081.).    Therefore, the SWRCB will not be responsible for all mitigation measures described in the FEIR.    This is consistent with the programmatic approach taken by the SWRCB for this project.    The following pages in the DEIR deserves your close review consistent with this discussion on mitigation: VI-56 to 59, VI-72 to 78, VI-102 to114, VI-116 to 117, VII-66, VIII-35, IX-19, XIII-20, XIII-38, XIII-53, XIII-60, and XIII-67.    It may be helpful to your readers to create a summary chapter or table in the FEIR of impacts, level of significance, mitigation measures, responsibility for implementation of mitigation measures, and residual impacts after mitigation.

- 4. Appendices

CEQA Guidelines section 15147 provides that technical data be summarized in the body of the document and placed in the appendices for the benefit of the reviewing

agencies and public.    Although this is a question of style, we suggest that FEIR place the following pages into the appendices: VI-82 to 94, VII-28 to 40, XIII-21 to 30, and XIII 53 to 58.

- 5. Banks Pumping Assumption

The DEIR assumes that full Banks pumping capacity is available from July through October.    It is assumed not to be restricted by storage, conveyance, or demand south of the Delta.    In fact, this is not the case.    If the water is available for export after meeting Delta standards, the water may be exported only if San Luis is not full or if the canal capacity is available.    Even if these conditions are satisfied, the demand may not coincide with water availability.    Agricultural users also have found that if the water is not available early in the spring, cropping patterns are set for the year and the water made available later cannot be effectively used.    Some of this water designated as available for export would be of course used, but assuming full capacity at Banks all summer results in a bias of higher export amounts than occur during actual operations.    One way to use this additional water is to have surface storage or direct recharge capacity available south of the Delta.    We address this subject in detail in our specific comments to Chapter V.

- 6. Joint Point of Diversion

DWR is concerned over the assumption used in the development of alternatives 1 & 2.    Neither of these alternatives, which are "base cases," include wheeling of water for Cross Valley Canal, Tracy Golf Course, Musco Olive, or VA Cemetery.    These water users utilize, at full delivery approval, about 150,000 AF.    Not including this wheeling in the base case may mislead the reader about the alternatives.    The alternatives do not "increase" water supply in most cases but more correctly stated enable the wheeling to these current users.    This is discussed below in comment on XIII-7.

- SPECIFIC SUBSTANTIVE COMMENTS

- II-2

In addition to the water quality objectives for Contra Costa Canal, there is also a chloride objective of 250 mg/L for West Canal at mouth of Clifton Court Forebay, Delta-Mendota Canal, Barker Slough at North Bay Aqueduct Intake, and Cache Slough at City of Vallejo Intake.    These objectives are essentially the EPA secondary drinking water standard for chloride.    The model analyses conducted with model runs on VI-8, XII-24, and XIII-20 do not show the impacts on chloride concentrations at these four sites.    Only impacts on chloride concentrations at Contra Costa Canal are shown.    Conversely, the DEIR includes much analysis on impacts on electrical conductivity at sites other than these diversion sites.    How often is the chloride objective of 250 mg/L exceeded at the diversion sites?    Can chloride concentrations at the diversion sites be estimated from electrical conductivity values if the diversion sites were included in the model runs?

- II-2

Contra Costa Canal Pumping Plant No. 1 in Table II-1 is not a reasonable compliance location for the water quality objective for Municipal and Industrial beneficial uses for the SWP and CVP, because water qualities at this location has been shown to be significantly independent of project control.    This problem has been exacerbated by the recent implementation of Contra Costa Water District's Los Vaqueros facilities use of the new intake results in stagnation and worsen water quality at PP No.1.    It adversely affects the State Water Project's and Central Valley Project's ability to exceed the objectives at CCC PP No. 1.

The analysis provided in this report is not representative of current operating conditions and it should also be revised utilizing a more appropriate compliance location along Old River.    We propose an alternative compliance location be established at Bacon Island at Old River to replace CCC PP No. 1 (C-5) and Antioch Water Works Intake (D-12) for the purposes of compliance by SWP and CVP (and other upstream diverters.)

- II-15, paragraph 4

Append to the end of the paragraph, which begins "Alternatives for the two narratives" the following: "In response to SWRCB recommendation, DWR has convened the multi-agency Suisun Ecological Work Group (SEW) to address, among other tasks, the Suisun Marsh narrative standard. The SEW plans to provide its recommendation to the SWRCB in time for the next triennial review."

- II-16, Flow Alternative 3

Expand on the rationale for this alternative where Friant is considered an in-basin project.

- II-18, paragraph 1

We suggest modifying the paragraph relating to Term 91.    We would like it clarified that the Term 91 rationale applies to all inbasin appropriators of unregulated flow regardless of priority date consistent with previous Board Interpretations.    The purpose of Term 91 is to protect DWR's and the Bureau's rights to their previously stored water regardless of the priority date of the inbasin permittees.    In SWRCB Order 84-2 (the motion for reconsideration of D-1594, which made Term 91 a standard permit term in all new permits and those diverters with Term 80), the SWRCB specifically rejected an effort to limit the application of Term 91 based on priority.    The SWRCB determined that the purpose of Term 91 is to protect the project's stored water unrelated to priority of in basin diverters.    We recommend that the FEIR modify this paragraph by eliminating the reference to post-1914 priority as the rationale for Term 91, and substitute the following:    "Term 91 prohibits permittees from diverting water when stored or imported project water is being released to meet Delta water quality objectives.    The purpose of Term 91 is to protect the projects' stored water.    If Term 91 were not in place, in-basin diverters could end up diverting a portion of project water released from storage intended to meet water quality objectives, or could be diverting non-project water at a time when the projects are releasing stored water.    The project operators would have to make additional storage releases to compensate.    Currently, Term 91 is made applicable to all Delta diverters with application dates after 1965 pursuant D-1594.    Under this alternative, Term 91 is made applicable to all appropriative water user in the basin with post-1914 priority dates."

The Department questions the purpose for allocating Term 91 responsibility in 8 blocks (except for modeling purposes).    It would seem both fair and practical to impose curtailments upon individual diverters to make up the excess project reservoir withdrawls.    This method will need to be revised to remove the grouping and specifically identify each appropriate water right holder for curtailment if either alternative three or four is the basis of the SWRCB’s preferred flow alternative.

- II-33 Table II-8

Reference on this to Table II-8 should be clarified.    Not all of the Feather River Entities are SWP contractors

- II-34, Section 2, paragraph 1, sentence 4

Change to: "An exception to this responsibility may be made when: (1) hydrologic conditions are such that even with full bore SMSCG operation and implementation of the Delta outflow objectives the objectives cannot be achieved; or (2) the SMSCG cannot be operated full bore and/or is physically modified in response to regulatory constraints."

- II-35, Section 2a

"The DWR and the USBR take no further action to meet the D-1485 western marsh objectives." (Delete the rest of the sentence.)

- II-36, Section 2e

DWR suggests that a clear distinction be made between what actions were modeled, and what actions are currently planned.    Modeled actions include: (1) Green Valley Creek augmentation up to 20 cfs from the FSSD; (2) consolidation and redirection of Morrow Island managed wetland drainage to Suisun Bay; and (3) September operation of the SMSCG when needed to meet objectives. The currently planned actions are listed in the "Demonstration Document," beginning on 38 (DWR, 1997c), except for Morrow Island drainage redirection which further analysis indicated would not achieve

its stated objective. We suggest that the full list of the eleven actions be listed and a citation of the Demonstration Document be included.

- III-20 1st full paragraph, last sentence

Only two pumping plants (Dos Amigos and Buena Vista) are necessary to serve most of SWP's agricultural users.    (96% of our aqueduct turnout capacity for San Joaquin valley agriculture lies upstream of the Buena Vista Pumping Plant.)

- Table III-5

Although the numbers in the table all add up to the totals in the respective permits, the way the table is structured and some of the permit components lumped together, it perpetuates some misunderstandings about DWR permits.    For A14443, only 1360 cfs of the allowable direct diversion is out of the Feather, the remaining 6185 cfs is direct diversion at Banks.    The allowable storage is 3,500,000 AF out of the Feather and 42,100 AF at Banks to DWR reservoirs south of the Delta.    Any of the water permitted for diversion out of the Feather may also be taken directly at Banks without any initial diversions at Oroville.    The permitted storage season for the Delta diversions is January through December.    The water diverted to storage under A17512 may be diverted to any of DWR south of Delta facilities not just San Luis.    Showing San Luis has led to the misconception by many that 1.1 MAF is the maximum we can store in San Luis in a single year.    This is not the case.    We can store any of the permitted storage quantities in San Luis.

- III-26, 2nd paragraph

The first sentence states that the implementation of the SWRCB's water rights decision "could affect" many of the Central Valley's fish species and communities.    This seems understated.    Because implementing the Water Quality Control Plan, which has been modified to better protect fishery resources, "would be intended to affect" those fishery resources.

- III-27

The statement that late-fall run Chinook salmon wait 1-3 months before spawning should be re-examined.    Late-fall run fish are typically in the most advanced reproductive state of all the races while they are migrating through the Delta, and may be older than 1-3 months.

- III-27

Department of Fish and Game biologists working on Deer and Mill Creeks reported that adult spring-run migration period is February through June.    There is disagreement about the validity of extending the migration period from June through September.    The reference to adult migration period should be re-examined.

- III-27

Presently, summer steelhead are found only in North Coast drainage but winter steelhead are found in the Sacramento River system, the north, south and central coast drainage. (McEwan & Jackson 1996)

- III-32, 2nd paragraph

The second sentence and paragraph five should be updated.    Recent Interagency Ecological Program studies have concluded that the distribution of splittail is not all that different than historically. (Sommer et al. 1997)

- III-32, 6th paragraph

Replace the entire paragraph with the following two paragraphs:

"The abundance of young splittail was significantly reduced during the recent six-year drought.    Nonetheless, it is clear that the drought did not compromise the ability of the stock to rebound. Wet conditions in 1995 resulted in record indices for most of the measures of YOY abundance.    This response is similar to other extreme wet years such as 1982, 1983 and 1986, which also produced high indices.    Initial indices for 1996 and 1997 (both wet years) also suggest improved abundance.    In contrast to young splittail, adult abundance showed no obvious decline during the 1987-1992 drought.    Adult population variation is moderated by the species' long-life span and multiple year classes.

Abundance of young splittail appears to be primarily related to flooding of terrestrial areas such as the Yolo Bypass, which provide spawning, rearing and foraging habitat.    Other factors that may affect abundance include toxic substances, introduced species, predation and sport fishing.    Salvage data since 1979 suggest that water project diversion losses have not had an important population level effect."

- III-39

The DEIR describes significant population growth in the City of Sacramento, as well as other communities in the Sacramento River Region.    Yet, there is no discussion of the impacts of waste discharges from the City of Sacramento.    There are currently plans to expand the Sacramento Regional Wastewater Treatment Plant, and a revised draft Environmental Impact Report (September 1997) for the master plan of the expansion is currently out for public review.    This DEIR does not address the water quality impacts, which may occur due to the Sacramento Regional Wastewater Treatment Plant.    This impact could be considered in the Cumulative Impacts section of the FEIR.

- III-45

In the discussion on the Sacramento River Basin, there is reference to only three waste discharges into the Sacramento River.    These are discharges from the cities of Redding, Red Bluff, and Chico (III-45).    Based on information from a report entitled Sanitary Survey of the State Water Project, which was published in 1990 by Brown and Caldwell on behalf of the State Water Contractors, there are 15 major waste water treatment plants which discharge into the Sacramento River watershed.    A list of these wastewater treatment plants is provided in the included table.    These wastewater treatment plants may also have impacts on dissolved oxygen and electrical conductivity.

- Major Waste Water Treatment Plants Discharging into the

#### Sacramento and San Joaquin Rivers

| Facility | Average Flow (million gallons/day) | Total Average Flow for Basin (mgd) | Basin Location |
| --- | --- | --- | --- |
| Sacramento Regional | 150 | 195 | Sacramento |
| Roseville | 11.8 |  |  |
| Vacaville Easterly | 6 |  |  |
| West Sacramento | 4.5 |  |  |
| Davis | 3.6 |  |  |
| Redding, Clear Creek | 3.5 |  |  |
| Oroville | 3.5 |  |  |
| Chico Main | 3 |  |  |
| University of California | 1.8 |  |  |
| Grass Valley | 1.6 |  |  |
| Red Bluff | 1.2 |  |  |
| Anderson | 1.2 |  |  |
| Placerville, Hangtown Creek | 1.2 |  |  |
| Beale AFB | 1.1 |  |  |
| Olivehurst PUD | 1 |  |  |
| Stockton Main | 29 | 50.9 | San Joaquin |
| Turlock | 8 |  |  |
| Merced | 5.5 |  |  |
| Tracy | 4 |  |  |
| Atwater | 2.9 |  |  |
| EID Deer Creek | 1.5 |  |  |
| Total | 245.9 | 245.9 |  |

- III-90, San Joaquin River

The text states there are no major public recreation facilities on the river's upstream reaches.    This information appears outdated.    Recreational areas on the San Joaquin River below Friant Dam have been expanding in recent years.    The text should reflect creation of the San Joaquin River Conservancy, a State-established regional land conservancy, and recent parkway developments in the Fresno area, such as Lost Lake Park, and the Lewis Moran Bicycle Trail; public land acquisitions such as Rank Island, recreational facilities near Mendota and planned parkway development near Firebaugh should also be included.

- III-98

In the discussion on human health concerns for Delta water as a drinking water source (III-98), the only disinfection by-products mentioned are Trihalomethane.    Another human health concern that should be discussed is the formation of bromate in the finished drinking water when ozonation is used for disinfection of Delta water.    Bromate formation occurs due to the higher bromide concentrations commonly found in Delta water.

- III-104, Section F

The introduction paragraph should be modified to give the latest information about the Suisun Marsh.    We suggest that the 2nd paragraph under Section F2 (Vegetation) be revised and moved to become the second paragraph in the introduction of Section F. A suggestion for the text of the new paragraph follows:

"Under the 1984 Plan of Protection for the Suisun Marsh and the 1987 Suisun Marsh Preservation Agreement to mitigate the effects of upstream water projects on the managed wetlands in Suisun Marsh, the staged construction of extensive water control facilities was planned. To date, the Initial Facilities (Roaring River Distribution System, Morrow Island Distribution System, and Goodyear Slough Outfall), and the Salinity Control Gates on Montezuma Slough have been constructed. These facilities help to ensure that a dependable supply of suitable salinity water is available to preserve managed wetland habitat including food plants for waterfowl."

- III-104

The DEIR states that Suisun Marsh supports 45 species of mammals, 230 species of birds, and 15 species of reptiles, but it does not mention the number of fish species.    We suggest that the DEIR also state that the Suisun Marsh also supports 51 fish species (Matern et al 1996).

- III-104, Section F1, Land Use

To make it more precise, we recommend this section be modified to read.

"The portion of Suisun Marsh within the Suisun Resource Conservation District boundaries includes 52,000 acres of diked, managed wetlands; 6,300 acres of relict tidal marsh; 29,300 acres of bays and sloughs; and 27,000 acres of grasslands including vernal pools and other natural seasonal wetlands. These acreage figures do not include the diked and tidal wetlands adjacent to the Contra Costa shoreline, which are part of the Suisun Ecosystem and under the influence of regulatory standards reviewed in the draft EIR.    The diked managed wetlands within Suisun include 153 privately owned managed wetlands.    The Department of Fish and Game manages 15,000 acres of land, which includes diked wetlands, tidal marsh, and uplands.    Concord Naval Weapons Station owns channel islands (Seal Island, Roe Island, Ryer Island, Snag Island, and Freeman Island) which are undiked tidal marsh set aside as wildlife sanctuary which support a variety of listed species."

- III-104, Section F2, Vegetation & VII-49, Section 4

These sections discuss vegetation occurring within the Marsh and impacts to vegetation associated with salinity.    These sections should include the following information. Vegetation pattern correlates to salinity regime along the estuarine salinity gradient of the San Francisco Estuary on the broadest landscape scale. The functional role of salinity as a causative agent for the distribution and abundance of plant species may differ between and within marshlands and salinity has not been identified as a direct causative factor of plant distribution.    Early observational studies conducted on waterfowl food plants within Suisun Marsh by the Department of Fish and Game some 25 to 30 years ago were not comprehensive in their treatment of this issue, and were not designed to determine causative mechanisms.    Modern scientific studies of wetland plant ecology confirm that the duration of saturation and anaerobic soil conditions often controls the distribution and occurrence of marsh plants.    Competition and positive biotic interactions have been found to determine marsh plant zonation patterns.    A growing body of scientific literature suggests salinity is often not the controlling factor in brackish and salt marsh community structure.    Observations within Suisun tidal marshlands confirm that the structure of these plant communities is not as strongly correlated to salinity regime as was once thought.    (Refer to the Suisun Ecological Workgroup Interim Report to the State Water Resources Control Board, September 1997: SEW Brackish Marsh Vegetation Subcommittee Report (IV 1 - 24) for more detail.)

There is no mention of rare plant species in these sections.    Suisun Marsh supports two endangered plant species (soft haired bird's beak and Suisun thistle) which are both endemic to Suisun Marsh, the rare Mason's lilaeopsis, and several species of concern considered to be in decline due to habitat fragmentation and fill (Delta tule pea, Suisun aster, and Contra Costa goldfields).    We suggest information about these plants be included in the FEIR.

- III-104, Section F3, 1st paragraph, lines 5-6

The reference to "natural food plants" is misleading.    The early studies by the Department of Fish and Game which identified alkali bulrush, fat hen, and brass buttons as the most valuable waterfowl food plants in Suisun were based on food habit studies that are now considered out-dated.    The details of this fact are reported later in this DEIR (p. VII-4).    Alkali bulrush seed production requirements are the basis for the salinity standards in Suisun.    The life history strategy of this plant does not require or even favor seed production for reproduction, as the primary reproduction of the species is vegetative spread of rhizomes.    Alkali bulrush is not a dominant or co-dominant species in any natural tidal marsh of Suisun Marsh.    The historic and modern importance of alkali bulrush to waterfowl in the Suisun tidal basin is unknown. Fat hen is a facultative wetland plant and its importance as a waterfowl food plant within Suisun is questionable.    Brass buttons is an introduced non-native species from South Africa, which should not be considered a keystone species in a Suisun Marsh ecosystem management strategy.    The need for a re-evaluation of salinity standards for the Suisun Marsh by the Suisun Ecological Workgroup was in part promulgated by a concern for the historic single resource (waterfowl) approach and because of the need to review the early studies, which established the original D-1485 standards.

- III-104, Section F3, 1st paragraph, lines 6-7

While soil salinity does influence seed germination and ultimately seed production, there is no evidence to substantiate the claim that channel salinity is directly correlated to soil salinity in managed wetlands of Suisun Marsh.    Years of monitoring within Suisun Marsh managed wetlands show that water management is the single most important factor in soil salinity management. Nearly all holophytic plants known to brackish marshes and coastal salt marshes subject to near seawater salinity require soil profile freshening to germinate.    There is no evidence to substantiate the need for sustained low salinity periods for this germination to occur.    A single rainfall event or even coastal fog drip often provides the freshwater input to trigger germination.    Controlled, scientific experiments to determine the germination and growth requirements of plants native to Suisun Marsh have not been conducted to date.

- III-106

We suggest you include delta smelt, longfin smelt, and splittail as fish species found in the Marsh.    These species are present in Suisun Marsh and were noted as important on III-26.    In addition to being an important nursery area for striped bass, Suisun Marsh is used by delta smelt, longfin smelt, and splittail larvae (DWR 1997).

- IV-3 item 4. Last sentence

The 2,500 cfs during October and November is not a maximum flow restriction.    Our agreement with DFG specifies that if flows remain below 2,500 cfs from October 15 through November 30, the minimum fish flows are 1,700 cfs through March whereas if maximum flows between October 15 through November 30 exceed 2,500 cfs, we can only decrease 500 cfs from that high point.    There are some other caveats like unplanned releases and shortage years.

- IV-5

The maximum pumping rate is not 8,500 cfs under Army Corp of Engineer Public Notice 5820-A as described in the third paragraph.    The 8,500 cfs maximum pumping rate is used for modeling purposes to reflect physical and operational constraints experienced with Skinner Fish Facility screens and Clifton Court Forebay trash racks.    Public Notice 5820-A limits diversion to a 3-day average of 13,250 AF/day plus an additional amount equal to one-third of total flow at Vernalis during December 15 to March 15 when flows at Vernalis exceed 1,000 cfs.

- IV-7 CVP Demands

Although the 1995 level of development described in item 1 may reflect reasonable demands for Contra Costa Canal, the modeling analysis does not reflect the recent implementation of the Los Vaqueros intake at Old River.    Also, analysis should be revised utilizing a more appropriate compliance location along Old River.    We propose a substantiated location at Bacon Island at Old River and Antioch Water Works Intake (D-12).    This is similar to the comment made with respect to section II-2.

- IV-8 Delta Standards

Recent implementation of the Los Vaqueros facilities provides an alternate intake to CCC and is resulting in revised operations by CCWD.    These revised operations adversely affect the SWP’s and CVP’s ability to ensure the M&I chloride objective at Contra Costa Canal is maintained even with the current or increased buffers under item 1. The analysis provided in this report is not representative of current operating conditions and should be revised utilizing a more appropriate compliance location as described in the previous comment.

- IV-11 Section E Aquatic Resource Relationships in the Delta

Section E (Aquatic Resource Relationships in the Delta) provides a good overview of the relationships used to analyze the effects of the alternatives on aquatic species.    Unfortunately, recent studies have shown that many of these relationships have either deteriorated or are not accurate.    The preparers of the DEIR should not be faulted here, as these results are very recent.    This does not mean that the conclusions of the DEIR are invalid, because the results are at least qualitatively sound as discussed in our next four comments.    Most of the following comments are caveats that should be included with the description of the modeling.

An additional concern is that the analysis of effects on upstream habitat may not be very biologically meaningful.    Effects on anadromous species are analyzed based on DWRSIM results for April-June.    This approach may be reasonable for American shad, but is probably not very relevant to Sacramento basin fall-run Chinook salmon, the bulk of which typically migrate as fry during the January-March period.    Inclusion of April may apply to late migrants, but the May-June flows are not informative.    While including other months of the year may not change the conclusions of the DEIR, it will make it more technically sound.

- IV-12, 5th paragraph

Add the following: "As a result of these concerns, the Sacramento River data were recently reviewed by two statisticians, Drs. John Rice (UC Berkeley) and John Newman (Univ. Idaho).    They supported the qualitative conclusion that salmon survival is indeed affected by Delta Cross Channel gate operations, temperature and flow in the Sacramento basin.    However, they found no evidence that exports reduced survival and the model results are not considered predictive.    As a result, the results of the USFWS model should be treated with caution.    In general, alternatives with higher outflow and longer DCC gate closures should be considered superior. "

- IV-13, 2nd paragraph

Add to the end of the paragraph: "However, there is evidence that a number of these relationships have deteriorated somewhat since the introduction of the Asian clam, Potamocorbula."

- IV-13, 3rd paragraph

Delete Sacramento splittail from the species list in sentence 3, then add the following to the end of the paragraph: "The outflow regression equation for splittail are less appropriate as year class strength is probably best associated with inundation of floodplain during the February-May period, which occurs in wet and extreme wet years (Sommer et al. 1997).    The effect of the different alternatives was therefore addressed qualitatively based on expected frequency of high flow events." Note also that the pages indicated for Chapter VI in the last sentence are not accurate.

- IV-14, 3rd paragraph

Add to the end of the paragraph: "Moreover, the striped bass population data were recently reviewed by Kimmerer (1997), who concluded that interannual variability of recruitment once was, but no longer is, related to flow conditions and exports in the

Estuary.    These parameters continue to affect survival in young bass, but this variation is largely suppressed by density dependent mortality after the first summer.

- IV-16 Term 91-Delta Calculation for Flow Alternative 3

Add the basis for using the assumption that carriage water is always zero for salinity control in the Delta.

- V-2, Paragraph 1

The assumption that all Sacramento Valley diverters will contract for a water supply to compensate for deficiencies does not appear to be a valid assumption.    Many Sacramento Valley diverters have access to groundwater supplies and would likely first turn to groundwater to supplement deficient surface supplies.    If Sacramento Valley contractors were to turn to the CVP or SWP, there are many contractual issues, which would have to be addressed through negotiations.    If they do not contract with the projects, there are other potential ways of meeting the deficiencies including crop shifting or fallowing, which were not discussed.    These types of options have been implemented in the past in response to shortages.

- V-2

The second sentence of the last paragraph needs to clarify that limited joint point of diversion occurs in the base case flow alternatives as allowed under D-1485 to recover fish constraints in April and May.

- V-2 Water Deliveries, last paragraph

Consider adding the statement that Alternative 6 has the smallest impacts because the assumption is made that the SWP export facilities are run full capacity all summer.

- V-4 Carryover storage

The assumptions regarding transfer capacity or export opportunities use the full capacity of Banks Pumping Plant.    The DEIR should explain why an assumption that DWRSIM optimization studies include storage and capacity limitations, which are appropriate, but the assumptions regarding transfer capacity or export opportunities are not limited by these constraints

- V-5 Delta Exports

The use of average annual exports as a measure of impacts should be limited by storage and conveyance capacity.    The assumption that Banks is pumping at full capacity all summer overstates the amount of water exported.

- V-6

The estimates of transfer capacity in the SWP and CVP systems are significantly overestimated.    The DEIR has not considered the many limitations on the projects ability to pump water for export even during the more favorable months, including storage space for the transfer water and timing on the available water.    In addition, as the deficiencies increase, the availability of water for transfer decreases.

- VI-6 next to last paragraph, last line

Is the reference to "...when X2 moves upstream about 2.5 miles" correct?    Should it be "2.5 km" instead?

- VI-6

The last sentence of the first complete paragraph should be revised for clarification as follows: "...the majority of the field salinity EC data is measured at the surface. " (Note these data is adjusted to 25oC to provide comparable data.)

- VI-7-8

To help your readers, we recommend that you include a map and a table showing how an X2 position relates to the location of Collinsville, Chipps Island, and other key geographical points in the estuary.    The text might mention here and in appropriate sections of previous chapters that the X2 relationship to the abundance indices of some species is not as strong as it was when X2 was originally evaluated.    Kimmerer (1997) attributes this to the effects of Potamocorbula.    The FEIR should consider the significance of this new information as it relates to the impact analyses on the various species.

- VI-18

Recent implementation of the Los Vaqueros intake is not represented in the modeling analysis performed for this report or described in the first complete paragraph and accompanying figures.    The operation of CCC Pumping Plant No.1 in conjunction with this new facility can adversely exacerbate SWP's and CVP's current difficulties in maintaining compliance with the water quality objectives for chloride identified in Table II-1.    Increased chlorides in January and February months would affect CCWD operations in filling Los Vaqueros Reservoir and may result in additional water costs by the SWP/CVP in attempting to maintain compliance.    However, the standard may be exceeded despite SWP/CVP actions depending upon CCWD operations. The analysis provided in this report should be revised utilizing a more appropriate compliance location along Old River.

- VI-20

The last paragraph dealing with "Delta Outflow" focuses on fish transport and migration.    The text should note that equally important functions of outflow include inundation of floodplain, dilution or mobilization of toxic materials and modifying salinities in the Estuary.

- VI-24, 3rd paragraph

Modest changes in salinity, such as the differences between the alternatives, are unlikely to affect harvest rate.    We suggest that the paragraph be modified with text to this effect.

- VI-30, 1st paragraph

We suggest deleting the second and fourth sentences.    It is not clear to us why an adult smelt that is moving upstream against the flow would be more likely to move with net reverse flow into the south Delta.    Nor are we aware of any evidence that saltwater fish

species are more aggressive than freshwater communities.    Both statements are speculative, at best, and detract from the more supportable outflow relationship.

- VI-30, 4th paragraph

We suggest that Delta diversions be deleted as one of the factors adversely affecting splittail spawning habitat.    They do not significantly affect the amount of land flooded and they are downstream from the most significant splittail spawning areas.    It should also be noted that reservoir flood control operations have a far more significant impact on the availability of flood habitat than the water supply operations.

- VI-30 last paragraph

Delta diversions and SWP/CVP exports have no effect on temporarily flooded habitat.    That is due to flows in integrated streams and flood releases from upstream reservoirs.

- VI-31, 2nd paragraph

Replace the first sentence with: "There is no evidence that water project entrainment has an important population level effect on splittail abundance (Sommer et al. 1997)."

- VI-31, 3rd paragraph and Figure VI24

For reasons discussed in Chapter IV, we recommend deletion of this paragraph and figure.    Substitute the text with: "As noted previously, abundance of splittail appears to be controlled primarily by inundation of floodplain habitat.    This occurs primarily in wet years and depends on the type of flood events.    Alternatives 2 through 7 have somewhat higher mean outflow than Alternative 1, which might provide slight benefit to spawning in the main channels of rivers."

- VI-31, 4th paragraph

The DEIR cites a 1990 estimate of the striped bass population.    We recommend the FEIR use the estimate for 1994.    The striped bass population numbers for 1994 are 1,192,247 for adult bass (3+ years old) and 712,111 for legal sized bass.

- VI-32, 2nd and 3rd paragraphs, Figure VI-25

For reasons described in Chapter IV, we recommend deleting these two paragraphs and the figure.    A new figure should be prepared showing just the YOY portion of the DFG striped bass model.    The results of the YOY model will likely show that Alternatives 2 through 7 are better.    The text should also note that variation in YOY is unlikely to affect adult population levels.    It should also be clearly noted that the DFG model assumes rather than indicates the relationship between outflow and bass population –as in IV-14, the model is not valid for adults.

- VI-33 plot of striped bass model comparison of alternatives.

The text states at VI-31 that striped bass population "plummeted" from 3 million in early 60s to 1.7 in late 60s.    The plot of model results does not indicate such a precipitous decline and further indicates that the "no project" alternative case shows more striped bass than were present in the early 60s.    This suggests that the model does not accurately reflect conditions in the Delta.

- VI-35, 1st paragraph

Delete the sentence that begins.    "First, higher river..." This is probably a reference to the two-layer flow in the Suisun Bay entrapment zone that is now known not to exist.

- VI-35, 3rd paragraph

It is suggested that food limitations may also be important, because the Potamocorbula invasion has increased competition for Neomysis.    It seems this fact should also be mentioned for splittail, striped bass and other such species that feed on Neomysis.

- VI-39, section 5

It has not been established that all the appropriative water rights holders located in the Delta have riparian rights unless SWRCB is accepting assumed riparian status for all diverters within the Delta.    This assumption should be stated if that is the case.    This assumption is particularly significant during critical periods when there may be no natural flow.

- VI-46 through VI-56

The analysis of effects on upstream habitat using April-June flows may not be very biologically meaningful.    To review briefly, this approach may be reasonable for American shad, but is probably not very relevant to Sacramento basin fall-run Chinook salmon, the bulk of which typically leave the river as fry during the January-March period.    The analysis is also not relevant to fall-migrating spring-run and winter-migrating winter-run.    If possible, fall and winter months should be incorporated into the analysis.

- VI-59, VI-66, VI-103

Consistent with our general comments, we believe the conclusions drawn about fisheries, vegetation, recreation, and scenic impacts are incomplete and need additional analysis.    The DEIR concludes these impacts may be significant and are not mitigable. Pursuant to CEQA Guideline section 15261, ongoing reservoirs are exempt from CEQA requirements.    In addition, CEQA case authority provides that environmental impacts associated with reservoir operations are not subject to CEQA.    (Nacimiento Regional Water Management Advisory Com. v. Monterey County Water Resources (1993) 15 Cal.App.4th 200.).    The consequences of this authority are that impacts associated with ongoing reservoir operations on fisheries, recreation, and scenic values are not subject to CEQA requirements.

Assuming for the sake of analysis there is some CEQA responsibility for these impacts, DWR believes that proper analysis for reservoir operations is that the reservoir operators have jurisdiction for mitigation measures and that reservoir impacts associated with the implementation of the Bay-Delta Plan are generally temporary and are mitigable.    For example in the case of fishery impacts at Lake Oroville, DWR has employed in the past numerous measures to enhance fisheries after the lowering of the reservoir, including planting fish and improving fish habitat by planting vegetation.    Similarly, DWR has lowered boat ramps to improve recreational access.    DWR also disagrees that the temporary lowering of the lake level, which results in additional shore exposure, is considered a significant impact for an artificial lake when such lowerings are part of normal operations.

- VI-72 Paragraph 2

This paragraph should be revised because of several factual errors.    There are landscape requirements under the Best Management Practices for Urban Water Conservation in the Memorandum of Understanding for Urban Water Conservation in California, but there are no BMP's requiring xeriscaping.    The statement that most of the urban agencies receiving export water from the Delta have signed the MOU is subject to question.    Approximately 150 agencies out of the 430 that supply over 3,000 acre-feet (that have over 3,000 connections) have signed the MOU.    A more appropriate statement would be that most of the urban water exported from the Delta is delivered by agencies that have signed the MOU.

- VI-78

Statement at the bottom of the section on the energy impacts are not mitigable is ambiguous.    Because the section refers to several types of impacts: hydroelectric, ground water pumping, and fossil fuels it is not clear which impacts are not considered mitigable, why are they not mitigable, and what is the level of the impact.    In our view, these impacts may not be subject to CEQA and, assuming they are, they may not be mitigable.    For energy impacts at reservoirs, these impacts are subject to our earlier comments about ongoing reservoir operations.    Energy impacts associated with ground water pumping may be mitigated through conjunctive use and off-peak pumping operations.

- VI-103-114

The cultural resources section is lengthy, but it does not answer the basic questions whether there are impacts and their level of significance.    In our view, there are no cultural impacts associated with SWP operations resulting from implementation of the Water Quality Control Plan.    We suggest other water project operators will have the same conclusion with regards to their own projects.    We would reduce this section considerably and conclude there are no significant cultural resources impacts.

- VI-116-124

Discussion on subsidence, overdraft, and export areas (ground water overlap) should be consolidated for clarity of the reader.    We believe that the mitigation measures associated with these impacts are identical and the responsibility for mitigation rests with the agency carrying out the ground water extraction.

- VI-124 Export Areas

Consistent with our general comments, the DEIR incorrectly assumes pumping unconstrained by operational factors.    The export impacts contained in this table do not distinguish between SWP and CVP in the export service area.    The 1986 Coordinated Operation Agreement is the current mechanism for allocating responsibilities between the two agencies, and any service area impacts will be allocated in accordance with provisions of COA.

- VI-126 Paragraph 2

We have the same comments as discussed above on urban water conservation.

- VII-1 Section A1, 1st paragraph, 3rd sentence

Care should be taken here that the acreage listed is specific to the SRCD boundary that ends at the Contra Costa County line. The statutory Marsh includes the Contra Costa shoreline.

- VII-2 Section A (MAP)

The location of Station S35 is incorrect. The actual location is approximately 1/2 mile upstream (south) on Goodyear Slough. We suggest that the map should include a general location for the Roaring River Distribution System, Morrow Island Distribution System, and the Goodyear Slough Outfall.

- VII-4, Section A.1.a, 4th paragraph

Studies by Batzer et al (1993, Wetlands 13:41-49) conducted in Suisun Marsh identified a variety of preferred invertebrate foods consumed by waterfowl in Suisun Marsh.    These invertebrates were present in pickleweed (Salicornia virginica).    Pickleweed was once dominant to subdominant in the high and middle marsh zones of Grizzly Island prior to diking of these lands (Mason 1972).    The Department of Fish and Game may have underestimated the value of this historic habitat when they recommended management to eliminate pickleweed in managed wetlands of Suisun (Rollins 1981).    The importance of pickleweed habitat for waterfowl is recognized by some private landowners in Suisun.    This pickleweed habitat also hosts native endangered species such as the salt marsh harvest mouse.

- VII-8, Section A.2, 1st paragraph, 1-3

It is erroneous to continue to report salinity as the variable of "primary importance".    While salinity is controllable and measurable making it a convenient regulatory parameter, the direct relationship of channel salinity to vegetation pattern in managed wetlands is problematic.    Water management controlling the depth and duration of flooding and ultimately soil redox potential is the causative variable in plant distribution patterns in diked managed wetlands.    Natural tidal regimes influencing the depth and duration of flooding in the undiked tidal wetlands are also of primary importance in these natural areas. See the SEW Brackish Marsh Vegetation Subcommittee Report submitted to SWRCB in September 1997 for literature citations which support this statement.

- VII-14, Section B, 1st paragraph

A paragraph should be added describing the Suisun Marsh existing facilities and their salinity impact.    These facilities are Roaring River Distribution System, Morrow Island Distribution System, and Goodyear Slough Outfall, and the SMSCG.    These facilities are operated for salinity control, and their operation could be modified depending on future actions.

- VII-22, 24, 25

The DEIR should be revised to include a more recent description of the proposed SMPA Amendment Three actions, which are the basis for a negotiated settlement for DWR and USBR water right responsibilities in the Marsh.    DWR has submitted to the SWRCB a “Demonstration Document: Suisun Marsh Preservation Agreement III Actions as a Means to Provide Equivalent of Better Protection than Channel Water Salinity Standards at Stations S-35 and S-97" (DWR 1997c), which can be referenced for a description of the proposed actions.    The Demonstration Document, however, includes redirection of Morrow Island drainage, which is no longer proposed in Amendment III because analysis indicated it would not significantly change channel salinities.    In April,

the parties to the Amendment plan to have available for public comment a draft environmental assessment and initial study which will have a comprehensive analysis of the proposed actions.

As discussed in the Demonstration Document, the parties to Amendment III are no longer proposing to augment Green Valley Creek flows.    The parties were unsuccessful at obtaining an agreement with FSSD for use of surplus treated effluent and funding of construction of an intertie for flow augmentation.    In lieu of the increased flows into Green Valley Creek, Amendment III would increase funding for Drought Management activities by $22,000 per year.    This funding is intended to help provide equivalent protection of lands that would have benefited from increased Green Valley Creek flows.    If an agreement for increased flows were obtained in the future, these funds would no longer be provided.

Following are specific changes recommended to the DEIR:

Action number 3, consolidate and redirect Morrow Island drainage - A sentence should be added to the end of the paragraph stating: “This action is no longer proposed as a management action in Amendment III because modeling by DWR determined it did not significantly reduce Goodyear Slough salinity and would not achieve the proposed objectives.”

Action number 6, funding of an FSSD-Green Valley Creek Intertie - Delete the first sentence and the description of Alternative b.    With respect to Alternative a, delete reference to DWR and USBR funding this action.    A sentence should be added to clarify that this is not an action proposed in the SMPA Amendment III and that entities other than DWR and USBR would perform this action.    The first sentence could begin with “Although DWR and USBR are not proposing this action as part of SMPA Amendment III, other entities besides the SMPA parties could fund costs associated with constructing the infrastructure (pipes and pumps) at the FSSD wastewater treatment plant . . . .”

Action number 7, operation of the SMSCG in September to meet October Salinity Objectives, should be revised as follows:

“The DWR and USBR will operate the SMSCG in September when the 7-day running average mean daily high tide salinity in September at any compliance station or at the S-35 Monitoring Station is 17.0 m S/cm or greater.    The running averages for September 1-6 will be determined using salinity data from the last 6 days of August.”

- VII -25, Section C.6, 2nd paragraphs

DWR never actually obtained water from SCWA under the 1995 Agreement.    The last sentence would be more accurate as follows: “In the past, the SCWA has agreed to provide water to agencies, including DWR, by agreement (SCWA Agreements 1992 and 1995).

- VII-27, Section D.1. Alternative 5

The DEIR analysis of environmental impacts of Suisun Marsh Alternative Five should be revised to clarify that the parties to the SMPA Amendment III do not plan to implement this action and that other parties could implement the action.    First sentence should be revised to state: “Same as Alternative 3 plus (1) some of the actions proposed by the

SMPA parties in Amendment III and (2) actions by others to provide flow augmentation into Green Valley Creek from FSSD surplus treated effluent.”

- VII-27, Section D.1.a.

End of first paragraph, add: “Implementation of the SMPA Amendment III is most like modeling of Alternative 3 with the addition of September SMSCG operation, which mostly affects October salinities during dry years.    Modeling of Amendment III actions could not include many management actions and would understate the net benefit that may be expected from implementation of Amendment III alone.

- VII-28, Table VII-4

It is not clear what the "Seasonal Average" column represents. Is it the average of the frequencies for eight months in the row, or is it the 73-year frequency?

- VII-41, Section D.1.b, 2nd and 3rd paragraphs

Change all references to exceedence frequency percent differences to percentage point differences. For example, 2nd paragraph, 3rd sentence: "Under Alternative 5, the annual exceedence frequency is 19.8 percentage points greater than under Alternative 4, and 6.9 percentage points less that alternative 3.

- VII-44, Section D.2.a, 4th paragraph, 3rd sentence

It is misleading to suggest that an October augmentation of 900 cfs would "occasionally" be needed. In fact, 900 cfs is needed on only one month of the 73-year analysis period. It would be more indicative to say that extreme augmentations would be greater than 150 cfs 5 percent of the time up to a high of 900 cfs.

- VII-48, Section D.3.b, 2nd and 3rd paragraphs

The Cordelia-Goodyear Ditch and Goodyear Slough Tide Gate alternatives would result in significant environmental impacts that extend beyond initial construction.    Operation and routine maintenance of these facilities would result in potential significant impacts to endangered fish and salt marsh harvest mice.

- VII-49, Section D.4, 1st-6th paragraphs

"Suisun Marsh alternatives will result in channel water salinities slightly different than historic conditions." While this statement is true, the potential impacts are much broader and more complex than this section suggests.

Temporal and spatial variability in flow and salinity regimes which most closely matches historic (pre-diversion) ranges (including extremes) are the factors necessary to sustain all of the diverse biological resources in Suisun Marsh, including waterfowl resources.    Management of the system for protection of important beneficial uses such as drinking water supply, agricultural water supply, and fish resources in the Delta preclude extreme high salinity events once experienced within the Marsh.    These extreme high salinity periods may be critically important to some species endemic to the Marsh, just as extreme flood events and fresh flow pulses have been identified as important to other resources such as cottonwood regeneration in the Sacramento- San Joaquin Delta.    A less than ideal year for waterfowl food production may be a critical time period for seed bank replenishment of other species such as rare, endemic annual plants.    It is

important to consider the full range of natural variability necessary to sustain this system.

- VII-50, Table VII-11

Soft-haired bird's beak and Suisun thistle are federally listed as endangered (not PE).

California black rail is a species that may be influenced by changes in salinity gradients.    While California black rails occur in the Delta, Suisun, and other North Bay Marshes, they occur at exponentially higher densities in holophytic plant communities of Suisun and North Bay Marshes than in freshwater emergent/willow shrub thickets in the Delta.    A shift to more freshwater conditions could result in a decline in this species, which is at its highest breeding densities in holophytic plant communities.

- VII-51, Section D4.a, 1st paragraph

DWR has concerns about the approach stated in Alternatives 1 and 3 of maintaining the western marsh objectives would be maintained with SMSCG operation and Delta outflow because it would reduce variability and natural salinity gradients.

The 1981 Biological Opinion for the Suisun Marsh Protection Program stated that implementation of the D-1485 standards would likely result in negative impacts to the salt marsh harvest mouse. These impacts should be discussed.

The Suisun Ecological Workgroup Interim Report to the SWRCB (September 1997) cautions that the historic monitoring programs in Suisun have revolved around the original D-1485 waterfowl food based requirements.    The data needed to recommend ecosystem-based standards in Suisun are incomplete and primarily directed towards waterfowl and fish resources.    Information needed to evaluate the impact of these stated alternatives will not be obtainable in a short time frame.    SEW's recommendations to the SWRCB will include elements of a comprehensive ecosystem based monitoring program coupled with support for scientific research to answer these pertinent questions.

- VII-54

In 1994, biologists from the University of California at Davis added four stations to the Suisun Marsh study bringing the total to 21 stations.    Six stations are east of Cutoff Slough.    The four new stations were added to better represent the entire marsh.    Consequently, the sentence "The western stations are considered representative of the marsh as a whole" should be deleted or changed. This information is available in Matern et al 1995.

- VII-54 and 55

The sentence "The four fall-midwater trawl stations...Native fishes." does not belong in this section.    The fall-midwater trawl stations are not part of the UC Davis study in Suisun Marsh.

- VII-55

The chameleon goby was misidentified in the UC Davis study until 1994.    All references to chameleon gobies in the UC Davis study in Suisun Marsh should be changed to shimofuri goby, Tridentiger bifasciatus. This information is available in Matern et al 1995.

- VII-58

The information you provided on the 1993 adult salmon sonic tracking study is correct.    However, the information we gained in the 1994 adult salmon study raised questions about salmon movement around the Suisun Marsh Salinity Control Gates.    Average passage times for salmon in 1994 were not statistically different among different gate configurations, and the passage times were greater than those seen in 1993 were.    This raised the question of whether there was actually a biological significance to the delay.    More information is available in DWR-ESO July 1997 and Edwards et al 1996.

- VII-60

During the Western Suisun Marsh Salinity Control Test fisheries monitoring, staff checked for spawning salmon and redds at four locations, not seven.

- VII-61

Under Alternative 1, impacts to aquatic resources would result from changing salinity and the effect of Suisun Marsh Salinity Control Gate operation.    For example, SMSCG operation may increase the passage time of adult salmon, but the biological significance of this effect is unknown.    More information is available in DWR-ESO July 1997.

Under Alternative 2, the fisheries impact of introducing Sacramento River water may be evident in more than just the northwestern marsh.    During the 1994 adult salmon sonic tracking studies, a tagged salmon released in Montezuma Slough near Nurse Slough was recorded in Cordelia Slough during the WSCT flow augmentation.    Modeling studies have also indicated that the effects of flow augmentation on salinity may be evident as far downstream as Suisun Bay. Suggest a change from northwestern marsh to western marsh.    More information is available in Edwards et al 1996.    The Goodyear Slough Tide Gate may also impact the movement of delta smelt within the slough, may increase the number of predatory fish in the area, and consequently increase predation near the gate.

Under Alternative 5, there may be impacts to Chinook salmon and Steelhead trout from the flow augmentation into Green Valley Creek.    During the WSCT questions were raised by agency biologists as to whether Chinook salmon moved into the unlined ditch because of the presence of Sacramento River water or simply as a result of additional flow.    Consequently, the use of Fairfield treatment plant water may not eliminate possible impacts to salmonids.

- VII-61, Section D.5.d, 8th paragraph

Fish screens would be an integral part of all portable pumps and would minimize fish entrainment.

- VII-62, Section D6, 1st paragraph:

While the current land use in Suisun is 88% diked seasonal wetlands, three major Estuary-wide resource agency planning efforts (San Francisco Bay Area Ecosystem Goals Project, CALFED Ecosystem Restoration Program Plan, and USFWS Tidal Marsh Recovery Plan) are calling for extensive tidal marsh restoration within Suisun Marsh to facilitate the recovery of endangered species and sensitive wetland habitat values.

- VII-62, Section D6, 1st paragraph

The D-1485 waterfowl food based standards have been challenged from a waterfowl feeding ecology, plant physiological ecology, and ecosystem basis.    These limitations have been acknowledged by the SWRCB. Channel salinity may not be the controlling factor in habitat quality or waterfowl use.    The evaluation of potential impacts should not assume the implementation of these standards would result in protection of managed wetland areas.    Waterfowl populations have continued to decline on Grizzly Island despite the maintenance of D-1485 standards in this area.

- VII-62, Section D6

Waterfowl hunting is not the only recreational use of these lands, but it is the only one mentioned in this section.    Non-consumptive recreation such as bird watching, canoeing, hiking, and wildlife observation are increasingly popular in the tidal marsh areas.    Educational programs are conducted in the tidal marshes at Rush Ranch and DFG's Peytonia Slough Ecological Reserve. The Napa-Solano Audubon Society volunteers conduct Christmas bird counts and breeding season surveys in Suisun Marsh.    Recreational boating is increasing in Suisun with the improvements to the Suisun City waterfront and harbor facilities.

- VII-63 and 64

The Western Suisun Marsh Salinity Control Test was not carried out for the entire salinity control season because hydrologic conditions were such that flow augmentation was not needed to meet standards.

- VII-66, Section E, last paragraph

This summary should clarify the difference in Alternative 5 versus SMPA Amendment III.    Revise the second to last sentence in paragraph as follows:    “Alternative 5, however, contains management actions proposed in SMPA Amendment III designed to provide equivalent protection to managed wetland areas (flow augmentation would be provided by others).”

- Figure VIII-1

The proper spelling for the Kaweah River near Visalia should be used on this map, even though it is outside the San Joaquin basin.

- Figure VIII-3

The proper spelling for the Kaweah River is needed.    The dashed boundaries of the drainage problem areas on this map disappear on the easterly boundary from Mendota Pool north to the vicinity of Vernalis.    It is confusing to have this open boundary.    The dashed line of drainage problem areas would be easier to understand if it was drawn in on the alignment of the San Joaquin River to reflect where there is a substantial amount of acreage with problems.

- Figure VIII-3

The location of the Chowchilla River is wrong.    It runs about twenty miles south of the City of Merced, approximately halfway between Merced and Madera.    It looks like what is labeled as the Chowchilla River is probably Bear Creek.

On the same map, there is also an error for the Kaweah River, which is nearly universal.    The Kaweah River runs slightly to the north of Visalia and then becomes Cross Creek, which sweeps north, then turns south and runs about halfway between Hanford and Visalia into the Tulare Lake Bed.    The Kaweah River channel shown within the Valley is really Elk Bayou, which is a relatively minor distributary.

- VIII-7 Salinity Sources

The last sentence of the bottom paragraph says, "Spreadsheet analyses indicate that the historical wetland contribution of salts (from all wetlands) to the San Joaquin River account for approximately eight percent of the total mean annual load." Today, salt from wetland runoff is a significant load, but it is not clear that recent historical salt loads represent pre-CVP conditions.    Long prior to the Central Valley Project, the water on Grasslands wetlands would have originated from the very low-salinity San Joaquin River snowmelt runoff. It was substantially lower in salts than the current Delta-Mendota Canal water source. Furthermore, for many years Grasslands wetland managers used tile drainage water from other CVP service areas, and considerable accumulations of salt developed. Recent changes in wetland water supply and management practices may lessen the wetland-derived salt, whether expressed as a load, or as a concentration.    Accumulations that arose during the past 50 years of minimal and highly saline water supply may be leached out, and wetland runoff salinity and loads may decline. This is properly reflected in the last paragraph of the text on VIII-22.

The present narrative suggests that wetlands are a major contributor to salt load.    The text here should reflect this situation is at least partly a contemporary artifact of water supply constraints that no longer prevail.

- VIII-15, a. Out-of-Valley Disposal, paragraph 1,

Rewrite the first sentence as follows: "Implementation of in basin measures, if the only means used to reduce salt loading to the San Joaquin River, will be effective only for the short-term.    A long-term solution must include disposal of salts outside of the valley, along with continuation of in basin measures as an ongoing means of reducing drainage volumes and salt and trace element loads. At present, the San Joaquin River..."

- VIII-18, e. Subsurface Storage, paragraph 2,

Add the following: "In dry years or a succession of dry years, additional water supplies may be needed for leaching and dilution of drainage in order to maintain soil quality and productivity.    If additional water is unavailable, due to water supply conditions or changes in water rights allocations, and groundwater supplies must be used, additional negative impacts may occur."

- VIII-19

Revise the discussion of land retirement as an on-going salinity management program.    There is additional discussion of this issue on pages VIII-20 and 21 under Salinity Control Alternatives.      The status of the DWR's Land Retirement Program needs to be updated to reflect the fact that the funds ($1,000,000) originally allocated to the interagency agreement with the Wildlife Conservation Board were redirected by the Resources Agency in late 1996 to the Department of Conservation for an unrelated program.    While the DWR has the authority to implement such a program, no funds have been allocated to it, nor are any planned in the near future.    Therefore, paragraph 5 and the bullets under this paragraph on VIII-19 should be deleted.    In addition, the focus of the USBR land retirement program is drainage reduction; protection, restoration and enhancement of fish and wildlife resources; and water acquisition to meet CVPIA obligations.    The primary focus of the DWR land retirement program is on reducing selenium in subsurface drainage.    Neither program has the primary objective of salinity management, especially as it relates to the San Joaquin River.    In fact, the majority of lands identified in the San Joaquin Valley Drainage Report, as "Problem Lands for Retirement" do not drain to the San Joaquin River.    Therefore, consideration by the SWRCB of land retirement as a potential means of salinity management may not be duplicative.

- IX-2

The location of the Grantline Canal barrier is correct for the Temporary Barriers Project.    As proposed for ISDP, the correct location for the Grantline Canal barriers is approximately 5 miles west near the confluence of Grantline Canal and Old River.

- IX-8

The stated purposes of ISDP are incorrect. The DEIR should replace these with the project purposes which DWR and Reclamation have agreed upon. They are: "1) improve water levels and circulation in the south Delta channels for local agricultural diversions, and 2) improve South Delta hydraulic conditions to increase the diversion into Clifton Court Forebay to maximize the frequency of full pumping capability at the Harvey O.    Banks Pumping Plant".

- IX-9

The description of the ISDP preferred alternative is incomplete. It should also include obtaining a permit from the Corps of Engineers to increase diversions into Clifton Court Forebay up to 10,300 cubic feet per second per day on a monthly averaged basis.

- Page IX-19, 3rd paragraph, Mitigation

Add the following sentences to the paragraph:

- Active Raptor Nests

Avoidance measures have been recommended to protect active raptor sites.    Measures include rerouting truck traffic, pre-construction surveys by a DFG approved biologist, and levee inspections prior to dredging.    During construction, on site biological monitoring shall continue.

- San Joaquin Kit Fox

Avoidance protocols to protect the kit fox include: pre-construction surveys, avoidance of kit fox dens, speed limits, avoidance of night construction, avoidance of entrapment, and sensitivity training of workers.

- Swainson's Hawk

For the Old River and Middle River sites, if sites are found to be active, reroute traffic no closer than 1/2-mile from the project area.    Trucks will remain rerouted until the biological monitor has determined the young Swainson's hawks have fledged.    At the

Grant Line site, special measures are recommended since a site is within 1,000 feet of the construction site.

Mitigation for the loss of 5.8 acres of foraging habitat shall comply with the DFG management conditions as specified in the Staff Report Regarding Mitigation for Impacts to Swainson's Hawks    (Buteo swainsonii) in the Central Valley of California.

- Giant Garter Snake

Avoid direct impacts to any giant garter snakes and loss of potential garter snake habitat by avoiding placing dredge material in the agricultural drains of Victoria Island.

- Western Pond Turtle

To avoid the loss of western pond turtles and eggs due to construction related activities, prior to construction, DWR and Reclamation should exclude turtles from suitable nesting sites by installing plastic fencing at all of the construction sites.

- Mason's lilaeopsis

Avoidance of Mason's lilaeopsis, to the greatest extent possible, in final location of the barrier on Old River at Tracy will be incorporated into the final design of the facility.    Currently, there is no established mechanism for broad scale mitigation of impacts to Mason's lilaeopsis (Shaffer, pers. comm.).    Such a program is desirable given the continual influx of projects affecting the plant, including water projects and levee maintenance.    In consultation with DFG Endangered Plant Program botanists, DWR/USBR should enter into an agreement to assist in the development of a recovery plan for the species.    Participation will include funding of staff time to develop the plan or funding of specific research needs as identified by DFG.    The limit of DWR/USBR responsibility in the program will be determined by relative costs of other commonly proposed mitigation such as transplanting and monitoring replacement populations.    Preconstruction surveys will also be required.

Prior to grading or construction, DWR and Reclamation shall consult with DFG and request consultation regarding impacts to Mason's and a permit or memorandum of understanding for the transplantation of Mason's, pursuant to DFG Code 2081.    A plan for transplanting masons will be developed with DFG.    Preconstruction surveys will also be required.

- Permanent Habitat Loss

The construction of the northern intake and the fish and flow control structures will result in the loss of habitat.    To compensate for the loss, DWR and Reclamation will create habitat at another south Delta location using mitigation ratios listed below.

| Habitat | Permanent Loss Acres | Mitigation Ratio | Mitigation Acres |
| --- | --- | --- | --- |
| Riparian (Willow) Scrub | 1.36 | 3:1 | 4.08 |
| Freshwater Marsh | 0.61 | 3:1 | 1.83 |
| Blackberry Scrub | 0.61 | 1:1 | 0.61 |
| Wildrye Grassland | 8.86 | 1:1 | 8.86 |
| Cropland | 5.8 | 1:1 | 5.8 |

- IX-20

The SWRCB's description of the Head of Old River fish barrier accurately describes the facility as conceived in the Draft EIR/EIS for ISDP. However, the facility presently planned by DWR will be similar to the other agricultural flow control structures, with radial gates instead of liftgates and a boat lock in lieu of docking facilities, stairs and a jib crane.

- IX-22

The schedule for temporary barriers operation is outdated and the SWRCB may wish to consider adding operation of the Head, Middle River and Old River near Tracy barriers from April 16-30 and operation of the Grantline Canal barrier from June through September, as authorized in the current Corps permit for the temporary barriers.    For the permanent barriers operation, the SWRCB should delete the Head of Old River in September and add Grantline Canal barrier operation in September.

- IX-23

The Grantline Canal barrier is in the wrong location for the permanent barrier as noted previously (comment for section IX-2 above).

- IX-42 and IX-43

The DEIR states that "Closure of the barriers...(Will result in).... water that previously had been diverted to the pumps at Old River would instead be diverted from the central Delta through channels such as Turner Cut and Columbia Cut."    The primary barriers causing this effect are the Head of Old River barrier and the Grantline Canal barrier.    The Middle River and Old River near Tracy barriers by themselves do not produce this effect.    The DEIR should clarify this.

- IX-47

The description of the Head of Old River barrier should be updated as noted in our comment for section IX-20 above.

- IX-48 (Summary)

The DEIR states that "Potential significant impacts to water levels and salinity.... as a result of barrier construction and operation (under Alternative 3) are identified." DWR does not understand the basis for this conclusion and believes it is erroneous. The barriers significantly improve minimum water levels upstream of each barrier with an accompanying slight reduction in water levels downstream of the barriers. We believe the slight reduction in water levels immediately downstream of each barrier is not significant, particularly since the Delta channels downstream of the barrier are significantly deeper than those upstream of the barriers and the siphons and pumps are generally located deeper in the water column. This factor, in conjunction with the net improvement in minimum water levels upstream of the barriers constitutes an overall net benefit to water levels in the south Delta area, not a potential significant adverse impact. With regard to water quality, IX-41 correctly states that "No significant impacts to water quality from operation of the barriers were identified."    The short-term water quality impacts from barrier construction are also not significant.

- IX-49 (Bottom)

DWR believes that the SWRCB should select Alternative 3 as the preferred alternative. First, we do not agree with the DEIR that the barriers may cause significant impacts to water levels. Second, the objectives being considered by the SWRCB in this chapter are to protect the beneficial uses of south Delta agriculture. The South Delta Water Agency represents these same interests and has negotiated a draft settlement agreement with DWR and Reclamation calling for the construction and operation of permanent barriers.    The SDWA also sees the barriers as one of the key solutions to providing irrigation water of acceptable quantity and quality.    We have achieved a negotiated settlement with regard to South Delta water quality objectives, and believe the SWRCB should select this as the preferred alternative.

- X-1

This chapter provides good descriptions of causes of DO problems within the lower section of the San Joaquin River.    Consider using metric units throughout.    In the text, dissolved oxygen is properly expressed in mg/L, but water depth (X-3) is measured in feet rather than meters, and water temperature (X-4) is measured in Fahrenheit rather than centigrade.

- X-1, 4th paragraph

References should be provided for the descriptions of the range of DO levels described in the second to the last sentence in paragraph 4.

- X-3, 2nd paragraph

The use of the term "assimilation capacity" is unclear.

- X-3, 3rd paragraph

The description provided in this paragraph for the San Joaquin River near Stockton is not unique to the San Joaquin River but holds true for all Delta waterways. It is probably a combination of these factors plus the discharges, deep channels, resultant poor assimilative capacity.    It is the fact that this part of the channel is a migratory corridor for fall-run Chinook that makes the DO concentrations in this area significant.

- X-5

In the discussion on the San Joaquin River Basin (III-73), there should be some reference to waste discharges to the San Joaquin River.    While some NPDES dischargers are listed on X-5 (NPDES Dischargers in the San Joaquin River Between Mossdale and Stockton), a list of six major wastewater treatment plants which discharge to the San Joaquin River is provided in the included table (see comment on III-45).    These wastewater treatment plants may also have impacts on dissolved oxygen and electrical conductivity.

- X-13

Based on the succeeding discussion and results of the model runs, DWR recommends that the best alternative for addressing the chronic DO problem is a combination of Alternatives 3 (ISDP) and 4(Reduced BOD Loading from Stockton). Each alternative is feasible by itself and a combination of both does not present any physical,

environmental or institutional conflicts. Therefore, a fifth alternative (combining alternatives 3&4) should be evaluated as the preferred alternative in the FEIR.

- X-13, 1st paragraph

The importance of flow on water temperature should be mentioned here.    In the Delta, water temperature decreases with inflow in both the Sacramento and San Joaquin Rivers. Using our continuous monitoring data, we have calculated a negative correlation between temperature and Sacramento and San Joaquin River flow.

- X-14, 2nd paragraph

There is concern about the implications of using 1957-1961 hydrology. There have been significant changes in the estuary physically since 1961, which make these model results unrealistic for current conditions. In addition, they are not valid years to use biologically. In 1977 we had a shift in climate in the eastern Pacific. This shift brought with it changes in the timing and intensity of rain and snowfall, air/water temperature and wind velocity. These changes combined can influence all ecosystem processes in the estuary, including those that influence dissolved oxygen concentrations.

- X-14 through X-28

These generally display model run results and provide a narrative description of the model runs.    In many instances, the narrative descriptions are absolute even when comparing small differences among the alternatives.    For some of the differences, the DEIR concludes they are significant.    These types of absolute descriptions can be misleading if model limitations, estimates of relative accuracy and estimates of relative error were not previously described for the model results being analyzed. No such descriptions were provided in either Chapter X or in Chapter IV, which describes the models used.    Either model limitations should be sufficiently described or the narrative analysis should take sufficient care to not describe results in absolute terms particularly when differences between alternatives result in relatively small magnitudes.    We suggest that the FEIR reexamine the conclusions it reached in these sections.

- X-18 through X-20

Alternative 1 (1978 Bay Delta Plan) which theoretically provides the least dissolved oxygen benefits is often higher than Alternative 2 (1995 Water Quality Control Plan) during the late summer and early fall months. The FEIR should provide a brief explanation for this.

- X-31

Flow (circulation patterns), algal blooms and water temperature are probably the key factors in regulating dissolved oxygen concentrations. Other alternatives addressing these factors could be useful. For instance, blooms near the bottom of the turning basin produce high organic matter and low dissolved oxygen concentrations that influence the rest of the river. Keeping the turning basin flushed, increasing mixing to reduce water column stratification or reducing the organic matter input (e.g. petroleum products) so that these blooms are smaller in the turning basin may reduce the incidence of the oxygen sag.

Although the Chapter acknowledges the general impact that algal blooms can have on dissolved oxygen levels in the eastern Delta, it does not emphasize the unique, local conditions that can occur as a result of blooms within portions of the Stockton Ship Channel.    Compliance monitoring has shown that late summer and fall phytoplankton blooms periodically occur within the Stockton Turning Basin (at the extreme eastern end of the Stockton Ship Channel).    Dissolved oxygen levels can exceed 14.0 mg/L (supersaturation) in the surface bloom area, and approach 0.0 mg/l (total anoxia) near the bottom as dead or dying algae settle out of the water column and accumulate at the bottom.    The impact of these blooms can "spill over" into the main Channel.

Because of the inverse relationship between water temperature and dissolved oxygen levels (as water temperature increases, dissolved oxygen level decreases) the text should acknowledge that flow manipulations alone may not accomplish dissolved oxygen levels above 6.0 mg/L in the Stockton area under any conditions.    August and September water temperatures in the Stockton Ship Channel can approach 26oC.    At that water temperature, dissolved oxygen levels in a completely saturated (100 %) system approach 8.0 mg/L. A typical aquatic ecosystem with moderate mixing and biochemical oxygen demand would experience 75 per cent saturation and have dissolved oxygen levels approaching 6.0 mg/L.    Thus, the eastern Stockton Ship Channel and the San Joaquin River in the Stockton area should not expect levels significantly higher than 6.0 mg/L when late summer and early fall water temperatures are at their highest, mixing is low, and BOD is high, no matter what flow manipulations are conducted.

- XI-1, Paragraph 3

Increased unreliability means that farmers will be less likely to plant high-input, high- valued crops such as vegetables and tree and vine crops.    Lenders will also be less likely to make production and land development loans for all types of crops, particularly for crops with high investment costs and a potential for high losses.    This, as well as the ability of the market to absorb a significant increase in production in crops for which California is a major or sole producer (e.g. vegetable, tree, and vine crops) without a collapse in prices, is a major limitation to the use of changing cropping patterns as mitigation for increases in unreliability.

Decreased groundwater storage due to substitution of groundwater for surface supplies no longer available will not only increase pumping costs (as cited) but also potentially limit cropping choices because of lower quality and cause subsidence (increasing the chance of flooding in some cases).    It also has the potential to constrain farmers' ability to market water as a means of enhancing their incomes and make groundwater substitution less useful as a drought mitigation measure not only for (i.e. conjunctive use) for themselves but also for potential buyers of transferred water.

While these factors might not have a serious impact on the overall number of acres in production, they have the potential to put specific farmers in financial jeopardy by increasing their costs, lowering their returns, and reducing their collateral (i.e. land value).

- XI-1, Paragraph 4

Because farmers are not necessarily landowners, producer's net income impacts can occur outside of the region (in the case of absentee landlords).

- XI-1, Paragraph 5

The "impact on agriculture" is understated by excluding operating costs since these expenditures are usually made within the region and support employment in agricultural businesses and, consequently, the local infrastructure important to the maintenance of agricultural productivity.

- XI-2, Paragraph 1

This analysis framework overlooks the problems caused by the higher probability of consecutive years of reduced deliveries and, consequently, the cumulative impact of higher production costs and lower incomes.    Farmers' financial resources can dwindle rapidly under these conditions and render them insolvent.

- XI-2, Paragraph 3

Modeling the average for each year type can be another reason impacts may be understated.    The impacts are not likely to be linear even within the grouped year types, particularly in extreme shortage situations.

- XI-4, Paragraph 1

Wet years are not typical.    The FEIR should explain why this condition was apparently used as a base.

- XI-4, Paragraph 2

The explanation of why the value of water is higher during shortages depends on the marginal crops affected within a region, not necessarily the proportion used on high valued crops, particularly where local water trading is common as it is among farmers throughout the Central Valley.    Also, if the shortage affects the ability to make a final irrigation to "bring in the crop", the value of that water can be very high, even for crops typically described as "low valued."

- XI-5, Paragraphs 1 through 7

Net revenues are a misleading measure of the impact on the local agricultural economy because the excluded farm operating expenditures support all the directly agriculturally related businesses in the region.    Although these impacts are discussed later in the regional impact section, their importance to the local economies of the Central Valley and maintaining the local agricultural infrastructure warrants their identification early on.

- XI-8, Paragraph 1

The fact that "these impacts are comparable to recent fluctuations" is not particularly relevant if they serve to reorient these fluctuations around a lower median point.    Financially stressed agricultural communities could be severely affected if this is the case.    The fact that tax income from farmland reduced in value could be significantly lower would only serve to make the situation more serious for those communities.

- XI-8, Paragraph 4

Major urban areas are already planning for extensive use of contingency water transfers during shortages.    Further reliance on this management option may be compromised by conveyance capacity or concern about third-party impacts that may result in more locally imposed restriction in export areas.

- XI-8, Paragraph 5

The statement that "...the only options...are additional water transfers and rationing..." is more likely to be true for urban areas outside of the South Coast Region.    In the South Coast Region, increased recycling is much more likely to be available as an option to replace imported supplies no longer available.    Because recycling is effectively 100% reliable, the replacement would not need to be on a one-for one basis, however.    The most economically efficient replacement ratio depends on the region's capacity for carry-over storage and the availability of contingency transfers.    Because of recycling's high cost, it likely that a combination of additional recycling and additional risk of rationing would be most cost-effective.

- XI-9, Paragraph 1

The impact on overall regional water management planning is not taken into account using this method of analysis.    Without understanding the operational opportunities afforded by the availability of imported and local surface supplies, local groundwater supplies, local carry-over storage and groundwater banking capacity and local contingency water transfers to manage reliability, any estimate of the economic impacts of changes in surface water availability are likely to be seriously biased.    DWR uses a yearly time-step local water management simulation framework based on a least-cost planning objective to estimate the economic consequences of changes in the availability of imported water supplies in view of the cost and effectiveness of all feasible local long-term and shortage contingency water management options.    This approach is also being used for CALFED storage and conveyance facilities screening.

- XI-9, Paragraph 4

For evaluations of the benefits of urban water service reliability, DWR uses a continuous loss curve which produces average shortage costs of about $1,750 per acre foot for a 10% shortage and about $2,650 per acre foot for 20% shortage.    Shortages of 5% are valued at about $1,200 per acre-foot, 15% at about 2,200 per acre-foot, and 25% at about $3,000 per acre-foot.

- XI-11, Paragraph 3

UC research staff should be contacted about the contention that the agricultural labor force remains highly mobile.    Recent studies may have shown that this is no longer true.    The ongoing shift to high-input, high-valued crops will result in more labor employed in agriculture, but (as stated earlier) this trend can be jeopardized by reductions in water reliability.    In addition, for some communities, there may be threshold effects that result in very serious hardship.

- XI-11, Paragraph 6

Impacts reported on a regional basis can mask serious localized impacts to already stressed communities.    The effect of reduced land values on community income from property taxes makes this possibility more likely.

- XI-15, Paragraph 1

Because of the ongoing shift to high-input, high-valued crops, the use of the 21% ratio may not be appropriate for impacts occurring in the future, leading to an underestimate of impacts.

- XII-3, CVPIA

This section refers to a Trinity River flow study to be completed by September 30, 1996.    The section needs to be updated with the results of the study, or its current status, if not completed.    The draft administrative proposal on allocation of the 800 TAF for environmental purposes should be described and its impacts briefly assessed.

- XII-4

The section on conjunctive use seems to be a description of the groundwater portion of the Supplemental Water Purchase Program.    It should be rewritten to indicate that DWR is cooperatively exploring the potential to increase SWP dry year supply with local agencies.    Surface water would be made available from the SWP for in lieu recharge in wet years and in dry years the local agencies would release a portion of their surface water to the SWP and substitute stored groundwater.    The reference to a DEIR should be deleted.    Although, conjunctive use projects could inadvertently result in limited land subsidence, this should probably not be seen as a cumulative impact.

- XII-5&6

The section on the Folsom South Canal Connection Project needs revision to reflect the currently proposed project.    The project also includes alternatives for joint diversion and conveyance facilities with the City and County of Sacramento that could have impacts beyond the EBMUD only alternative.    The project description is also out of date and should be taken from EBMUD's DEIR.    This section may also be an appropriate place to discuss the potential impacts of EBMUD's proposed conjunctive use project in San Joaquin County.

- XII-6

The purposes of ISDP as stated in the DEIR are not the ones stated in the draft EIR/EIS for ISDP. The FEIR should be revised to state ISDP's purposes as: "The DWR, Reclamation, and the U.S. Army Corps of Engineers (Corps) have identified the following two project objectives or purposes: 1) improve water levels and circulation in south Delta channels for local agricultural diversions; and 2) improve south Delta hydraulic conditions to increase diversions into Clifton Court Forebay to maximize the frequency of full pumping capacity at Banks Pumping Plant.    These project objectives or purposes were developed following consultation between the three agencies."

- XII-7, 1st paragraph, 1st line

Edit this line as following and add it to the end of last paragraph on page XII: The DWR and USBR are proposing the installation of three permanent flow structures and one fish control structure through the ISDP.

- XII-7, 1st paragraph

Edit the remaining portion of the paragraph as following: The program also calls for operating the SWP pumps at full capacity; installing additional forebay intake structures; limited channel dredging along 5-mile reach of Old River.

- XII-7, 3rd paragraph

- Change the word "barriers" to "flow control structures"

- XII-7

This section on Los Banos Grandes creates the inaccurate perception that this is an active DWR project and that numerous other south-of-the Delta projects are being investigated by DWR.    This is not the case and this section should be deleted.    This discussion would be better placed in the section on CALFED, which is considering south of the Delta storage.

- XII-9

The program described as the purchase of Delta Islands is more correctly called the West Delta Program. One component of the program, which is not described in the SWRCB's DEIR, is that the purchased lands can be developed into habitat for mitigation of future SWP facilities construction and operation.

- XII-13, Cumulative Impact Assessment

As discussed previously, the assumption that the SWP and CVP diversion is limited only by the physical capacities of the pumping plants is likely to inflate the transfer capacity of the projects.    Assuming the SWP can use the Banks Pumping Plant July through October at 10,350 cfs is going to result in some very large and misleading numbers.    On XII-14 the DEIR states that "Most of the reduced export capacity can be made up through increased transfers." The reliance on transfers that are based on such large export rates underestimates the actual impacts in the export areas because the actual amount of transferred water will be less.

- XII-16, Transfer Capacity

Same comment as above.    Assuming that Banks and Tracy will operate at full capacity July through October inflates the transfer capacity.

- XII-19, Figure XII-3

First two bars are not distinguishable from each other in the legend/graph.

- XII-35 & 36 Section a

Urban Water Conservation section should be updated to reflect the BMP revisions.    The BMP revisions were developed and approved by the CALIFORNIA URBAN WATER CONSERVATION COUNCIL in 1997.

- XII-35 & 36 Section b

This section should discuss the agricultural MOU's completion in 1996, the agricultural Water Management Council's formation in 1997, and its current status.

- XII-36, Water Transfers

Transfers are described as "the most promising way of closing the gap between water demands and dependable water supplies." The additional restrictions on export facilities, and the difficulty in avoiding impacts to prior water right holders is making the transfer of water across the Delta even more difficult.    Transfers, although a useful tool that will continue to be used, should not be viewed as a panacea for all Delta problems.

- XII-38, Combined SWP/CVP Points of Diversion in the Delta

This section needs to be updated.    Since CVPIA and other recent developments, the USBR does not have the "excess water supply north of the Delta."    This section also discusses "excess capacity in the SWP facilities" that could be used without impairing the SWP.    As our previous comments have pointed out, this is an oversimplification of the present situation.

- XII-39, Offstream Storage Projects

Los Vaqueros Reservoir has been completed and is now operating.

- XII-39

The description of why ISDP is being undertaken is incomplete and should be modified to note that it is being undertaken to improve conditions for local diverters, in addition to increasing SWP yield and operational flexibility.

- XIII-5

The assumption used in development of alternatives 1 & 2 overstate the exports occurring as a result of the Joint Point.    Neither of these alternatives, which are "base cases," include wheeling of Cross Valley Canal, Tracy Golf Course, Musco Olive, or VA Cemetery by the SWP.    These water users use, at full delivery approval, about 150,000 AF.    Not including this wheeling in the base case causes a misrepresentation in evaluation of the alternatives.    The alternatives do not "increase" water supply in most cases but more correctly stated, enable the wheeling to those users.    This is discussed below in comment on XIII-7.

- XIII-6 2nd line

The statement that "The maximum pumping rate under Notice 5820-A is 8,500 cfs." is incomplete.    Refer to comment on IV-5 for a complete description.

- XIII-7 bottom paragraph

This states "...the average annual water supply increase for the CVP ranges from 75 TAF to 247 TAF." The actual water supply increase is more likely from 0 to 100 TAF, because both alternatives 1 and 2 do not include SWP wheeling for Cross Valley Canal, Tracy Golf Course, Musco Olive, or VA cemetery.

- XIII-34, 4th paragraph

See the previous comments about recent results showing that exports are not a significant factor affecting smolt survival.    This should not affect the model results, as the differences between alternatives are primarily attributable to DCC operations.    A sentence or two to this effect should be added to the text.

- XIII-34, 5th paragraph, and XIII35, 1st and 2nd paragraphs

As noted, recent studies indicate that the striped bass model is no longer valid for adults.    It is reasonable to conclude that export changes will affect juvenile abundance levels, but the text should make it clear that this will not necessarily affect adult abundance.

- XIII-38, 2nd paragraph and Figure XIII50

As discussed in Volume I, Chapter 4, the outflow relationship is less appropriate for Sacramento splittail.    The text is appropriate, but the figure should be deleted.

- XIII-38 Net Reverse Flows

Statement that revere flows may disorient fish causing increased straying is largely conjectural.    Channel flows resulting from tidal filling and draining of the Delta occur four times daily and at flow rates sometimes orders of magnitude larger than the "net" flow.

- XIII-48 and XIII-49 and Tables XIII-26 through XIII-31

In DWR's comments on Chapter VI we expressed concern that this analysis of effects on upstream habitat using April-June flows may not be very biologically meaningful.    As stated previously, this approach may be reasonable for American shad, but is probably not very relevant to Sacramento basin fall-run Chinook salmon, the bulk of which typically leave the rivers as fry during the January-March period.    The analysis is also not relevant to fall-migrating spring-run and winter-migrating winter-run.    If possible, fall and winter months should be incorporated into the analysis.

- XIII-62 through 64

Over a 73-year period, there will be essentially no difference in hydroelectric generation by SWP or CVP unless the alternatives change the amount of floodwater "spilled".    (Only a minor change might be seen due to changes in generation head during some months).    There may well be some minor change in net hydroelectric generation available to the California grid due to changed pumping loads of the various alternatives.

- XIII-73

As we suggested earlier, we do not expect cultural resources impacts to occur as a result of the Oroville drawdowns.

- XIII-78 through 79

Of alternatives 2 through 8, only alt 4 and alt 5 would be acceptable to the SWP.    The indications of additional reservoir drawdown stated herein are accurate and attributable to the JPOD.    The CVP is the party receiving benefits from the JPOD and the SWP will not agree to wheeling for the CVP if it directly affects the SWP water supply.    Lake Oroville change in minimum storage for the alternatives is restated below:#1=base, #2=-2', #3=-12', #4=+1', #5=+1', #6=-28', #7=-45', #8=-47'

- EDITORIAL COMMENTS

- II-16, 2nd paragraph

As written the last sentence could incorrectly be taken to suggest that the SWP and CVP are responsible for meeting the D1422 objectives.    We suggest the meaning be clarified by adding at the "...and the CVP is solely responsible for meeting the objectives required by D1422."

- II-35, Section 2b, 4th sentence

"For the purposes of this analysis, the plan is assumed to consist of flow augmentation up to 80 cfs in Green Valley Creek with North Bay Aqueduct water and construction..."

- II-35, Section 2c

"The DWR and the USBR take no further action to meet the D-1485 western marsh objectives." (Delete the rest of the sentence.)

- II-36, Section 2d, 4th sentence

"For the purposes of this analysis, the plan is assumed to consist of flow augmentation up to 80 cfs in Green Valley Creek with North Bay Aqueduct water and construction..."

- II-36, Section 2e, 2nd sentence

"...take management actions to protect the beneficial uses of the managed wetlands of the western marsh..."

II-36, Section 2eChange all occurrences of the word "water master" to "water manager."

- II-36, Section 2e, 2nd sentence

Strike the phrase "...but not limited to..."

- III-1

The Aquatic Resources section of Chapter III is concise, well written and mostly accurate. "delta smelt" is the scientifically accurate spelling, not "Delta smelt".

- III-2

Spelling of Kaweah River is not "Kawaeh".

- III-15 last paragraph

Statement that figure III-8 shows SWP deliveries from '67 to '96 is incorrect.    This does not include deliveries in the Feather River basin including the water rights settlement agencies.

III-16

The map incorrectly portrays location of South Bay Aqueduct facilities.    Water for the South Bay Aqueduct is first pumped at Banks PP.

- III-17 bottom line

Substitute the following sentence: Del Valle Reservoir provides 40,000 AF of storage capacity for water delivery and conservation.    The Lake is designed for 77,000 AF storage, but all storage above 40 TAF is floodwater encroachment.

- III-28, 2nd paragraph

We suggest adding the following sentence to the end of the paragraph: "However, data from Bennett and Howard (1997) suggests that many older bass move to the ocean during warm El Nino events (e.g. 1976 - 1977)."

- III-30, 2nd paragraph

Change the last three sentences to read: "During the recent six-year drought the center of delta smelt abundance was the western Delta.    However, during high outflows in water years 1993, 1995 and 1997 their distribution shifted into Suisun Bay and areas further downstream.    Delta smelt are captured seasonally in the channels of Suisun Marsh."

- III-30, 5th paragraph

Change the second sentence to read: "...between 1982 and 1992 their abundance was relatively low in most years." The 1991 Fall Midwater Trawl Adult Abundance Index was actually pretty strong.    The FEIR should mention that the 1993 and the 1995 indices were the 6th and 7th highest on record.

- III-31, 1st paragraph

Consider concluding this paragraph by mentioning that the SWP and CVP are now operated under the USFWS' 1995 Biological Opinion.

- III-31, last paragraph

Add the following after the third sentence about longfin smelt: "Abundance has improved substantially during recent wet years."

- III-32, 3rd paragraph

Change the first sentence about splittail to read: "...begin spawning at 2 years of age." The third sentence should read: "Adults spawn from February through May in the Delta, upstream tributaries, Napa and Petaluma rivers and Suisun Bay and Marsh."

- III-51 and 53

The statement that Sacramento perch still persist in the Sacramento River may not be accurate.    Moyle (1976) and Moyle et al (1995) report that Sacramento perch no longer exist within the Sacramento River but have been introduced into reservoirs and lakes.

- III-52 last paragraph, line 3+

The reference to "interim bypass operation" should be modified.    The temperature control structure is in place and was operating through the summer.

- III-70, Figure III-11

The map of the San Joaquin River Region has Camanche Lake misspelled.    Also Stanislaus is misspelled near Stan/Merced County Line.

- III-73, 2nd paragraph

This paragraph cites some principal dams on tributary streams. On the Mokelumne, only Pardee is cited, although Camanche stores significantly more water. It would be helpful if the text described the basis for selecting the dams that were worthy of being mentioned.

- III-82, Table III-15

- The California mastiff bat genus Eumops is misspelled

- III-83, 1st full paragraph

The Grasslands Water District no longer accepts tile drainage flows in the Grasslands area for wetland use.    Since passage of the CVPIA, water for these wetlands has been made available from the Delta Mendota Canal or tailwater supplies.    Selenium remains a concern, because the Grasslands area has significant residues of selenium from local tributary streams, soils, and accumulated loads from past use of tile drain water.

- III-88

Recreation at O' Neil Forebay also includes a large numbers of windsurfers.    DWR tries to maintain high water surface elevations as operational needs allow to provide a safer windsurfing area.

- III-92 2nd paragraph, last 2 sentences

Modify this section to reflect that there are 11 fishing access sites along the California Aqueduct.    Bikeway mileage should be modified to 97 miles.

- III-104, Section F2 (Vegetation)

The paragraph on vegetation appears outdated. We believe that a list of endangered and candidate species should be provided similar to Table III-18 for Delta wildlife species.

- III-104, Section F3, 6th sentence

Remove the word "proper."

- III-106, Section F3, 6th sentence

Revise sentence to read: "...which is affected by salinity of applied water and land management.

- III-106, Section F3, 7th sentence

Revise sentence to read: "Freshwater flows from the Delta and tributary creeks into Suisun Bay..."

- III-134

Modify first full paragraph, third sentence to read: "The completion of the Coastal Branch of the SWP in 1997 has lessened the reliance on ground water supplies in San Luis Obispo and Santa Barbara counties.    The Coastal Branch facilities transport . . . "

- III-106, Section F3, 6th sentence

Remove the word "proper."

- IV-9, Section B, 1st sentence

Revise the sentence to read: "DWRDSM is a mathematical computer model..."

- IV-9, Section B, 1st paragraph

Make a distinction between the two versions of the DWRDSM model used in the DEIR. Delta alternatives are analyzed using DWRDSM1, which uses the Martinez tide as the downstream tidal boundary condition.    DWRDSM Suisun Marsh Version is used for Suisun Marsh alternatives, and uses the Golden Gate tide for its downstream boundary condition.    Both versions are variants of the Fisher Delta Model.

- IV-9, Section B, 2nd paragraph, 2nd sentence

Change the second sentence to "The equations are solved numerically using the Method of Characteristics for flows, stages, and velocities at discrete locations."

- IV-9, Section B, 3rd paragraph, 1st sentence

Revise the sentence to read: "The transport of dissolved water quality constituents..."

- IV-9, Section B, 4th paragraph, 3rd sentence

Revise the sentence to read: "... and a one to five minute time step..."

- VI-2 line 3

- Change "eggs, smelts, larvae" to "eggs, smolts, and larvae"

- VI-2 line 3 VI-26, 1st paragraph

Replace the last two sentences with the following: "As noted in Chapter I, the models have some important limitations, so it is probably best to compare the results qualitatively.    Given the fixed temperatures used in the model, the best survival can be expected with higher flows and Delta Cross Channel Closure."

- VI-26, 2nd paragraph

After the first sentence, add the following: "Note, however, the previous comments about the limitations of the model."

- VI-29, 3rd paragraph

Replace this paragraph with: "Outflow controls the location of the entrapment zone, an important part of the habitat of delta smelt.    There is a weak, positive correlation between abundance and the number of spring days that the entrapment zone remains in Suisun Bay (IEP 1997).    Reduced outflow shifts the distribution of smelt into upstream channels, reducing the available habitat and increasing risks of predation, entrainment and contaminant effects."

- VI-30, 3rd paragraph

Change the second sentence to read: "...and typically begin spawning at 2 years of age..."

- VI-35, 2nd paragraph

Change paragraph to read..."C. franciscorum spawn in the winter and early spring and densities are correlated with outflow during this time (DFG 1993). In low flow years, the distribution of C. franciscorum is further upstream and exposes them to entrainment at the PG &E power plant. Large numbers of C. franciscorum are entrained by the PG&E plant in a wet year, and the numbers may be higher in dry years (IEP 1996). C. franciscorum populations may be adversely affected by the reduced availability of phytoplankton food since 1986, when the exotic clam, Potamocorbula amurensis, became established in the estuary and reduced chlorophyll a levels by a factor of 10 in Suisun Bay."

- VI-35, 2nd paragraph

Add to the end of this paragraph.    "C. franciscorum distribution is further upstream in low flow years which exposes them to entrainment at the PG&E power plant. Large numbers of C. franciscorum were entrained by the PG&E plant in a wet year and the numbers may be higher in dry years."

- VI-35, 2nd paragraph

The last sentence in this paragraph makes it sound as if Potamocorbula eat Neomysis. Modify last paragraph to read: "C. franciscorum populations may be adversely affected by lower phytoplankton food availability since the 1986 invasion of P. amurensis, which reduced chlorophyll levels by a factor of 10."

- VI-35, 3rd paragraph

Change the paragraph to read...."The amount of shallow, brackish water habitat seems to be a key population factor for this species.    Shallow water provides physical refuge for juvenile C. franciscorum from predators and adult shrimp; Crangon are cannibalistic (IEP 1996)."

- VI-35, 3rd paragraph

Modify this sentence to read: Shallow water habitat provides physical refuge for juvenile C.franciscorum from predators and adult shrimp; Crangon are cannibalistic."

- VI-35, 4th paragraph

Use the word "juvenile" rather "immature."

- VI-35, 4th paragraph

Change the last line... other alternatives and may be due to higher outflow.

- VI-35, 5th paragraph

Change the paragraph entirely to read..."Neomysis. Neomysis. Neomysis mercedis, a native mysid shrimp, is an important food source for many estuarine fish and feeds upon phytoplankton, rotifers and copepods (SWRCB 1995). The life span, survival, size and abundance of N. mercedis is regulated by outflow, water temperature and food availability. The SWP and CVP pumps may export large numbers of N. mercedis in low outflow years when N. mercedis are farther upstream. Water temperature affects the rate of development and size at maturity. Food supply is probably the most important limiting factor for N. mercedis, which have decreased with the decline of phytoplankton chlorophyll a concentrations since the 1970s (Orsi and Mecum 1996)."

- VI-35, 5th paragraph

Remove statements about entrapment zone per earlier comment.

- VI-36, 1st paragraph

Modify end of first sentence to read: "lowered phytoplankton chlorophyll concentrations"

- VI-36, 2nd paragraph

Replace the last 1/3 of the paragraph with... "Increased flow and reduced diversions are believed to increase phytoplankton biomass, increase potential habitat and push P. amurensis populations downstream where they compete less with N. mercedis for phytoplankton food."

- VI-36, 2nd paragraph

Modify "abundance" to "phytoplankton chlorophyll" and in other parts of the text.

- VI-36, 4th paragraph

Entrainment by diversions and residence time is probably important factors affecting copepod abundance in the Delta (IEP 1996).

- VI-36, 5th paragraph

Change the entire paragraph..."Phytoplankton.    Phytoplankton are very small, usually microscopic, algae that are suspended in the water column and drift with the currents.    The major phytoplankton groups in the Bay/Delta Estuary are diatoms.    Phytoplankton convert solar energy into food through photosynthesis and comprise an essential part of the food web in the Estuary.    Phytoplankton productivity, biomass, density and species composition are influenced by several factors, including light, temperature, nutrients, residence time, inflow and grazing by aquatic organisms (SWRCB 1995)."

- VI-36, 6th paragraph

Change the entire paragraph...."Light limitation through turbidity and depth affects phytoplankton growth rates in the Estuary (USBR 1997a). In general, phytoplankton are light limited due to the high turbidity in the estuary. Net production is consistently negative in the channels of the Delta, where most of the phytoplankton occur in light-limited conditions below the surface.    Only in the shoal areas, like those in Suisun Bay, where the phytoplankton cells are frequently mixed into the surface waters, can net production be positive; phytoplankton growth rate is about ten times higher in the shoals than the channels of Suisun Bay (SWRCB 1995)."

- VI-37, 1st paragraph

Remove reference to the entrapment zone. Phytoplankton was high in shoals of Suisun Bay until introduction of the clam in 1986 that decreased chlorophyll a concentrations by a factor of 10.

- VI-37, 2nd paragraph

Change the entire paragraph..."Entrainment is important to phytoplankton variability in the Delta (IEP 1996).    Export pumping was negatively correlated with chlorophyll a concentration and influences phytoplankton community composition. Diversions and Delta outflow together account for 86 percent of the variability in chlorophyll a concentrations in Suisun Bay (SWRCB 1995). Habitat for phytoplankton in the Delta is also influenced by residence time, which varies with flow conditions (SWRCB 1995)."

- VI-37, 2nd paragraph

Use "Suisun Bay" instead of "entrapment zone."

- VI-37, 3rd paragraph

Change the entire paragraph...."Lehman 1996 demonstrated chlorophyll a concentrations were highest for the Delta during intermediate (normal) water-year types.    Higher chlorophyll a concentrations occur in Suisun Bay during wetter years, but extremely high outflows decrease phytoplankton biomass by flushing phytoplankton out of the estuary."

- VI-71

Second sentence should be modified to state: "Water is one of many factors influencing growth in a region but does not, by itself, cause the growth of a region."

- VII-3, Section A.1, 3rd paragraph, line 4

"...uplands" should be changed to "...annual and perennial grasslands and vernal pools adjacent to the marsh were classified as the secondary management zone."

- VII-4, Section A.1.b: (Title)

- Revise title to: "The Suisun Marsh Preservation Agreement"

- VII-5, Section A.1.b, 3rd paragraph

Add the following sentence after the first sentence of paragraph 3: "The Initial Facilities (phase 1) were constructed in 1980 including the Roaring River Distribution System, Morrow Island Distribution System, and Goodyear Slough Outfall.

- VII-5, Section A.1.b, 3rd paragraph, 2nd sentence

Change "The first and most..." to "The second phase and most..."

- VII-5, Section A.1.b, 3rd paragraph, 4th sentence

Change the word "salty" to "more saline."

- VII-7, Section A.1.b, 3rd paragraph, 4th sentence

Change "...augmentation in Green Valley Creek, were conducted in 1994." to "...augmentation in Green Valley Creek, were conducted in 1994 with North Bay Aqueduct water."

- VII-7, Section A.1.b, 4th paragraph, 2nd sentence

Change "The discussions are nearly complete, and the..." to "Execution of Amendment Three is pending completion of the CEQA/NEPA and CESA/ESA environmental documentation process, and the..."

- VII-7, Section A.1.b, 4th paragraph, 3rd sentence

"...soil salinity conditions in all years on managed wetlands than would strict adherence to the salinity objectives."

- VII-8, Section A.1.c, 2nd paragraph, 11th sentence

Revise the sentence to read: "Lastly, the plan recommended that the DWR form a multi-agency Suisun Marsh Ecological Work Group (SEW)."

- VII-8, Section A.1.d, 1st paragraph, 4th sentence

Revise the sentence to read: "The SMPA deficiency standards, as applied in water short years..."

- VII-8, Section A.1.d, 1st paragraph, 4th sentence

Delete the word "relaxed".

- VII-8, Section A.2

Regarding list of factors affecting salinity, item 4, "...Salinity Control Gate operation (beginning October 31, 1988)"

- VII-14, Section B.1, 2nd paragraph, 4th sentence:

Revise the sentence to read: "Green Valley Creek can influence flows into Cordelia and to a lesser extent Goodyear Slough and can therefore..."

- VII-15, Section B.1

Add Cordelia Road to the map of Figure VII-4.

- VII-16, Section B.1, paragraph 4, 1st sentence

- Remove the phrase "northeast of the other two reservoirs"

- VII-16 North Bay Aqueduct

It is not correct to assume that because the entire capacity of North Bay is not being used right now that it is available to help meet Marsh standards.

- VII-16, Section B.2, 1st paragraph, 2nd sentence

Revise the sentence to read: "The capacity of the NBA is 174 cfs..."

- VII-18, Section B.2, 2nd paragraph

Add the following sentence to the end of the paragraph: "The city plans to construct a storm discharge retention pond where the ditch is located."

- VII-19, Section B.3, 4th paragraph

Append to the end of the 4th paragraph: "In a letter to the DWR dated April 23, 1997, the FSSD responded that there are too many obstacles to proceed with the proposal at this time."

- VII-19, Section B.4, 2nd paragraph, 1st sentence

- The Figure referred to as "VII-5" should be "VII-5b"

- VII-20, Section C, 1st paragraph, 1st sentence

Revise the sentence to read: "...the DWR and the USBR will operate the Initial Facilities and SMSCG when..."

- VII-21, Section C.1, 1st paragraph, 3rd sentence

Revise the sentence to read: "D-1485 outflow objectives are in effect and the Initial Facilities and SMSCG are in place..."

- VII-21, Section C.1, 2nd paragraph, 1st sentence

Revise the sentence to read: "...plans to meet the western Marsh objectives under these base case hydrology conditions, and if the SWRCB..."

- VII-21, Section C.2, 1st paragraph, 3rd sentence

Revise the sentence to read: "...objectives are in effect and the Initial Facilities, and SMSCG are in place..."

- VII-21, Section C.2, 1st paragraph, 4th sentence

Revise the sentence to read: "...of the Cordelia-Goodyear Ditch and an associated tide gate structure, and through flow augmentation in Green Valley Creek with NBA water (DWR 1993).

- VII-21, Section C.2, 3rd paragraph, 1st sentence

Revise the sentence to read: "...and described in the Western Suisun Marsh Salinity Control Project Screening Report (DWR 1993)."

- VII-22, Section C.2, 5th paragraph, 3rd sentence

Revise the sentence to read: "The inlet and outlet gates would be operated in conjunction."

- VII-22, Section C.5, 1st paragraph, Append to 2nd sentence

Revise the sentence to read: "The following management actions, as recommended by the parties to the SMPA Amendment III, are implemented as described in the "Demonstration Document" (DWR 1997c).

- VII-22, Section C.5

Change all occurrences of the phrase "Water Management Program" to " Water Manager Program" in this Section.

- VII-23, Section C.5, Figure VII-6

The location of Station S35 is incorrect. The actual location is approximately 1/2 mile upstream (south) on Goodyear Slough.

- VII-25, Section C.5

Change "(Draft SMPA Amendment Three, March 26, 1997)" to "(Draft SMPA Amendment Three, September 9, 1997)."

- VII-27, Section D.1.a, 3rd sentence

Remove the word "averaged."

- VII-46, Section D.2.d, 1st paragraph, 1st sentence:

Revise the sentence to read: "The NBA has a capacity of 174 cfs from Barker Slough..."

- VII-48, Section D.3.c, 1st paragraph, last sentence

Revise the sentence to read: "...being prepared by the DWR, USBR, DFG, and SRCD."

- VII-52, Section D.4.c, 1st paragraph

Add a sentence regarding FSSD electing not to proceed at this time with the Green Valley Creek flow augmentation action.

- VII-52, Section D.4.c, 2nd paragraph

Change "water master" to "water manager."

- VII-52, Section D.4.c, end of 2nd paragraph

Add DFG and SRCD to list of environmental documentation participants.

- VII-58, Section D.5.b, 6th paragraph

"... a sonic tracking study was conducted in the fall of 1993 and 1994.    Add citations for two DFG reports on Salmon migration studies.

- VII-58, Section D.5.b, 8th paragraph

"The study was repeated in 1994."(Add citation) The adult salmon migration studies are now available.    Add citations for these reports.

- VII-63, Section E, 2nd paragraph

Add a sentence regarding Initial Facilities (Phase I) and change SMSCG to Phase II

- VII-63, Section E, 2nd paragraph

SMSCG operation began October 31, 1988.

- VII-63, Section E, 2nd paragraph

Delete the word "always" and insert "with minor exceptions in water year 1991."

- VII-63, Section E, 23d paragraph

Change "Western Salinity Control Plan" to "Western Suisun Marsh Salinity Control Project"

- VII-64, Section E, end of 3rd paragraph

"...to resident or migratory endangered species, and high naturally occurring winter and spring creek flows."

- VII-64, Section E, end of 5th paragraph

Add "...or better protection to the managed wetlands. DWR and USBR petitioned the SWRCB to extend the compliance date for S-35 and S-97 to enable the SMPA parties to finalize Amendment III. The SWRCB granted a 180 day extension on October 30, 1997."

- IX-10

The permanent agricultural flow control structures proposed under ISDP would be operated from April through October, not April through September as stated in the DEIR.

- IX-12

"Fifteen areas of concern...." should be replaced with "Fifteen areas of potential impact...". Not all of these areas were determined to have significant environmental impacts or be of concern.

- IX-12, 3rd Paragraph

Change the dates...."An initial Study ... prepared by DWR Division of Planning in 1995 (DWR 1995)."

- IX-12, 3rd Paragraph

Delete the last sentence,      "Based on the initial study..."      and add the following sentence taken from the 1995 Initial Study page 36:

"As part of the ongoing environmental analysis for TBP, a USACE jurisdictional wetland delineation survey was prepared for DWR by a consultant.    DWR prepared a biological assessment required as part of the endangered species process, which discussed potential impacts of the project on listed species and species proposed for listing.    At the same time, DFG staff prepared an assessment of non-endangered species including assessments of impacts of fish, wildlife, and plant community resources.    The studies did not specifically identify any other significant adverse impacts due to the proposed TBP installations.    They did identify some possible adverse impacts, however, and concluded that it could not be determined that there were no significant impacts based on available data."

- IX-31

Figure IX-17 appears to be incomplete for Alternatives 2 and 3.

- IX-42

Table IX-2 should be updated to reflect the changes in both temporary barriers and permanent barriers operation as recommended above.

- XII-20, Last sentence of paragraph on striped bass

Modify this sentence to read: "...the effect of a stocking program on smelt...", rather than "...program for smelt..."

- XIII-1 bottom paragraph

"...The SWP can operate to its physical capacity of 10,400 cfs under limited conditions established in the agreement. ..." DWR's water right is for 10,350 (as stated on XIII-5) not 10,400.

- XIII-8 2nd paragraph

Statement is true as presented but there are potential benefits to fisheries possible from combined points of diversion, because pumping could be shifted from one plant to the other.

- XIII-14 mid-page

This states that the "net Delta outflow pattern is the opposite of the export pattern.    Generally, net Delta outflow decreases from July through January and increases in February and March." The concept is right but it is not obvious from Tables XIII 10 through 13.    Conceptually, NDOI should increase over alternative 1 for February through June except in critical periods when the system may run short of water to meet needs.

- XIII-19 mid-page

This states that minor differences in X2 for the alternatives are expectable because NDOI varies little among them.    Table XIII-10 & 11 indicates a significant difference in the February to mid-June period when X2 is a concern.    For the 73 year period, NDOI changes vary as follows: Feb 962 cfs, Mar 597 cfs, Apr 1109 cfs, May 810 cfs, June 577 cfs.    These seem like significant variations of NDOI among the alternatives.