April 1, 1998

Victoria A. Whitney, Chief

Bay-Delta Unit

Division of Water Rights

State Water Resources Control Board

P.O. Box 2000

Sacramento, CA      95812-2000

Re:	Comments on Draft EIR

Dear Ms. Whitney:

The East Bay Municipal Utility District (EBMUD) appreciates the opportunity to comment on the November, 1997 “Draft Environmental Impact Report for Implementation of the 1995 Bay/Delta Water Quality Control Plan” (Draft EIR).    This letter summarizes the District’s comments on (1) the Mokelumne River baseline conditions used in the Draft EIR; (2) the Draft EIR’s reliance on water transfers as a solution to shortages caused by the SWRCB flow alternatives; and (3) the Draft EIR’s omission of article X, section 2 reasonable use considerations.    The enclosed Attachment A presents a detailed discussion of EBMUD’s more technical comments on the EIR.    The District’s comments are limited to those portions of the Draft EIR that deal with allocation of responsibility for meeting the flow objectives of the 1995 Bay/Delta WQCP.    As requested in the SWRCB’s January 26, 1998 Notice, we are submitting two copies of EBMUD’s comments.

Environmental Reference/Baseline Conditions for the Lower Mokelumne River

In its description of the 1984-1994 “baseline” environmental reference conditions on the lower Mokelumne River, and in its description of the instream flow assumptions used in the SWRCB’s analysis and modeling,    the Draft EIR uses inappropriate flows for the Mokelumne River.        (On page II-14, footnote 1, the Draft EIR describes the 1984-1994 baseline flow    conditions for the Mokelumne River as being the “new flows required by the    FERC.”    On page IV-4, the Draft EIR states that Mokelumne River flows are set pursuant to the new FERC agreement.)    The SWRCB’s use of the proposed Federal Energy Regulatory Commission (FERC) Mokelumne River settlement flows as the baseline condition is inaccurate for the reasons discussed below.

The existing release requirement    for EBMUD on the lower Mokelumne River is set pursuant to a 1961 Agreement,    as amended, between EBMUD and the California Department of Fish & Game (CDF&G).    That 1961 Agreement constitutes EBMUD’s release requirement for the lower Mokelumne River pursuant to both EBMUD’S SWRCB water rights and its FERC license,    and was the    release requirement during the entire 1984-1994 baseline/environmental reference condition period.

However, rather than use the 1961 Agreement as the 1984-1994 baseline condition, the Draft EIR treats proposed FERC settlement flows as the baseline.    In FERC    proceeding 2916-004 on the lower Mokelumne River,    a Joint Settlement Agreement (JSA) has been reached which contains proposed new requirements for EBMUD releases to the lower Mokelumne River.    While it is EBMUD’s expectation that those settlement flows will ultimately be approved by FERC, FERC has not yet taken final action on those flows.    Thus, it is inaccurate for the Draft EIR to assert those proposed flows are in effect as the current release requirements.    Moreover,    it is    erroneous to describe those flows as the 1984-1994    baseline condition, because those flows were not required and did not exist during the baseline period.

In defining the 1984-1994 baseline conditions on the Mokelumne River, the Draft EIR should be revised to refer to the release requirements of the 1961 Agreement which constitute the actual, existing 1984-1994 legal standard.

The practical effect of the Draft EIR’s treating the proposed JSA flow regime as the baseline is to grossly underestimate the significant difference between existing baseline requirements (the 1961 Agreement), the proposed JSA flows, and the proposed SWRCB flow alternatives.      In fact, the difference between the existing requirement    and the JSA is substantial,    and is shown on the attached Figure 1.      For purposes of accurately comparing the SWRCB’s proposed flow alternatives to baseline conditions, the Draft EIR should use the 1961 Agreement as the lower Mokelumne River baseline condition.

SWRCB’s Proposed Water Transfer Solution

In describing the economic impacts of the SWRCB flow alternatives, the Draft EIR concedes that the alternatives will affect EBMUD.    (p. XI-8.)    The EIR then states that water utilities, such as EBMUD, will likely respond to water deficiencies caused by the SWRCB alternatives by taking a number of actions, including arranging water transfers.    This reference to water transfers as a solution to real deficiencies caused by the SWRCB flow alternatives glosses over the complexity and uncertainty of water transfers.    EBMUD supplies water to a major metropolitan area consisting of domestic, industrial and institutional users.    EBMUD relies on the Mokelumne River as its primary water source, which provides on average 95% of EBMUD’s supply.    SWRCB flow alternatives 3, 4 and 5 will seriously impact EBMUD’s Mokelumne River water supply, as the EIR acknowledges, thereby impacting a dependable, high quality water supply.    The Draft EIR’s solution - water transfers - introduces significant uncertainty and is a significantly less reliable source of water.

The history of water transfers shows that, even given a willing seller and buyer, institutional and legal obstacles may arise that can impede the transfer.    For example, transfers require approval by the SWRCB with respect to changes in the point of diversion, place of use, purpose of use and related issues.    The approval process involves staff investigations, opportunity for protests, and a hearing.    This process    takes considerable time, and approval of a proposed transfer is by no means assured.    Unless the SWRCB is able to assure approval of water transfers necessary to make up for supply losses    caused by the SWRCB alternatives, it cannot be assumed that transfers are a feasible means to protect against resulting shortages.    This issue is of paramount importance given the extremely limited availability of high quality source water to replace Mokelumne River water that would be taken by SWRCB alternatives 3, 4 and 5.

In essence, SWRCB alternatives 3, 4 and 5 would be taking a reliable supply, the Mokelumne River, and offering as a substitute for a portion of that supply a highly speculative water transfer solution.      A large    urban suppler such as EBMUD must have a dependable supply of water to provide reliable service within its service area.    Given the enormous uncertainty associated with seeking alternative water supplies through transfers, it is not reasonable to assume that water transfers to make up shortages are readily available.      The Draft EIR must acknowledge the water supply, service area and economic impacts of alternatives 3, 4 and 5 as significant impacts on EBMUD.    These impacts are potentially unmitigatable because the Draft EIR’s water transfer “solution” may not be real, given the uncertainty that water transfers will actually result in delivery of sufficient substitute supplies.    Thus, the Draft EIR should identify these impacts as significant and potentially unmitigatable.

Consideration of Reasonable, Beneficial Use

In the Bay-Delta water rights hearing for which the Draft EIR was prepared,    the SWRCB proposes to take quantities of water from various water rights holders in order to meet water quality standards.    This could potentially result in significant impacts to water users as their supply is diminished and/or made less reliable.    In taking such a significant action affecting water rights, the SWRCB must adhere to the constitutional provisions of article X, section 2.    That provision states that water use must be for a reasonable and beneficial use, and it applies to water right decisions of the SWRCB.    In exercising its duties under article X, section 2, the SWRCB customarily examines a number of factors to determine if a given right holder’s use is reasonable.    This review can include an examination of whether the user has implemented conservation, reclamation and other water use efficiency actions to ensure efficient delivery and use of water. This requires an examination by the SWRCB of each affected water right holder’s reasonable    use as part of the SWRCB’s    balancing of factors in the course of making its decision.

The Draft EIR, however, does not incorporate consideration of water users’ reasonable use into the proposed implementation of the SWRCB flow alternatives.    Instead, it appears that the SWRCB proposes to implement flow alternatives 3, 4 or 5 without regard to article X, section 2 reasonableness factors.    For example, consider the imposition of SWRCB alternatives 3 or 4 on two diverters with equal seniority dates.    One diverter is making reasonable use of its water, employing proactive conservation, reclamation and water efficiency measures, while the other diverter has not implemented such measures.      Under SWRCB alternatives 3 and 4, the two users would be treated    equally as their diversions would be cut off at the same time.    This approach is both inequitable and contrary to article X, section 2 because the exercise of water rights would be significantly curtailed with no consideration whatsoever of reasonable use.

The SWRCB must take into account    the reasonableness of use made by water right holders.    This reasonable use factor must be taken into consideration by the SWRCB as it balances factors in making its decision.

Impacts of SWRCB Flow Alternatives

SWRCB flow alternatives 3, 4 and 5 would cause significant impacts to the Mokelumne River anadromous fishery, Pardee and Camanche Reservoir fisheries and recreation resources, and to EBMUD’s service area and customers.    These impacts are not adequately addressed in the DEIR.    They are discussed in more detail in the enclosed Attachment A.

Thank you again for the opportunity to comment on the Draft EIR.    If you have any questions, please contact me or Jon Myers , (510) 287-1121, at the District.

Very truly yours,

Fred S. Etheridge

Attorney

Attachment

# Figure 1

ATTACHMENT A

DETAILED COMMENTS OF THE EAST BAY MUNICIPAL UTILITY DISTRICT ON THE

STATE WATER RESOURCES CONTROL BOARD’S

DRAFT ENVIRONMENTAL IMPACT REPORT

FOR THE IMPLEMENTATION OF THE

1995 BAY/DELTA WATER QUALITY CONTROL PLAN

March 31, 1998

Table of Contents

I. PROJECT DESCRIPTION	1

II. UPSTREAM AREA IMPACTS	1

A. LOWER MOKELUMNE RIVER FISHERY	2

Impacts of SWRCB Alternatives on Lower Mokelumne River Fishery	2

Camanche Reservoir Discharge Temperatures	5

Conclusion	7

B. RESERVOIR FISHERIES AND RECREATION	7

Reservoir Fishery	7

Pardee Reservoir Fishery	7

Camanche Reservoir Fishery	8

Reservoir Recreation	9

Pardee Recreation Area	9

Camanche Recreation Area	10

Impacts of SWRCB Alternatives on Reservoir Fisheries and Recreation	10

III. CUSTOMER AND SERVICE AREA IMPACTS	Error! Bookmark not defined.

A. EBMUD SERVICE AREA	12

B. WATER TRANSFERS	12

C. CONSERVATION AND RATIONING	12

IV. MISCELLANEOUS	17

V. CONCLUSION	18

LIST OF TABLES

TABLE 1.  Fish Species Known to Occur in Pardee Reservoir	8

TABLE 2.  Fish Species Known to Occur in Camanche Reservoir	8

LIST OF FIGURES

FIGURE 1.  Lower Mokelumne River Fall-Run Chinook Salmon Escapement, 1940-1997	3

FIGURE 2.  Lower Mokelumne River Fall-Run Chinook Salmon Escapement, 1990-1997 Natural Spawners	4

FIGURE 3.  EBMUD Water Supply Facilities and Service Area	14

FIGURE 4.  Accounts vs. Consumption, 1961-1997	16

ATTACHMENT A

East Bay Municipal Utility District (EBMUD or District) submits these detailed comments on the Draft EIR as part of the State Water Resources Control Board’s (SWRCB) review process.    EBMUD’s comments are limited to those portions of the Draft EIR that deal with the allocation of responsibility for meeting the flow objectives established in the Bay/Delta Plan.

# I.    PROJECT DESCRIPTION

The seven alternatives evaluated in the Draft EIR focus solely on water supply, while assuming fixed water demands. That approach is far too narrow, ignoring the entire water demand side of the equation, and so omits required California Constitution Article X, Section 2 considerations. Instead, the Draft EIR should include an analysis of the reasonableness of use of entities which divert water from the Bay/Delta basin, as required under Article X, Section 2.    A list of reasonableness factors that could be included in the analysis of urban water use are included in the Memorandum of Understanding Regarding Urban Water Conservation in California, which is summarized in the Draft EIR (page XII-34).    A list of reasonableness factors was also provided by the Joint California Water Users in their January 29, 1996, comments to the SWRCB, pages 10-12.    Based on this analysis, the Draft EIR should examine water users’ reasonable use and demand management.    The EIR list of flow allocation alternatives must be modified to take into account users’ demand management and reasonable use, so that flow allocation alternatives are not implemented without consideration of reasonable use.

# II. UPSTREAM AREA impacts

Section VI. C of the Draft EIR describes environmental effects of the SWRCB flow alternatives on upstream areas.    The Draft EIR identifies only recreation impacts for the Mokelumne River; hydrology, aquatic resources riparian habitat, and vegetation and wildlife    impacts are ignored.    The Draft EIR does not address any of the impacts on the lower Mokelumne River resident or anadromous fishery or the Camanche and Pardee Reservoir fisheries, and inadequately addresses the impacts on Mokelumne reservoir recreation.    Most significantly, temperature impacts on the lower Mokelumne River anadromous fishery are not taken into account, and impacts on the warmwater and coldwater fisheries and recreation in Pardee and Camanche Reservoirs are also ignored or grossly understated.    These impacts are addressed below.

SWRCB flow Alternatives 3, 4 and 5 would cause significant, adverse impacts on the lower Mokelumne River anadromous fishery.    Subpart A, below, describes that existing fishery and summarizes the impacts SWRCB flow Alternatives 3,4, and 5 would have upon the Mokelumne fishery.

## A.    Lower Mokelumne River Fishery

The lower Mokelumne River supports a significant anadromous fishery.    Figure 1 shows the historical annual escapement of fall-run Chinook salmon returning to the lower Mokelumne River from 1940 through 1997.    Estimated annual run sizes during this period have ranged from 100 fish in 1961 (before Camanche Reservoir was constructed) to 15,900 fish in 1983.

In recent years, the annual runs have been equal to or exceeded the long-term average. Although these annual runs are supplemented by Mokelumne River Fish Hatchery production, there has also been a commensurate increase in the numbers of in-river spawners (Figure 2).

In order to enhance and sustain these resources, EBMUD has performed some of the most extensive monitoring and research activities on anadromous fish and their habitats in the Central Valley.    EBMUD has made a significant commitment and contribution to improving the technical understanding of the river’s anadromous resource through intensive in-river scientific investigations and the development of measures to improve the resources based on the results of those investigations.

In summary, the lower Mokelumne River has a significant adadromous fishery which is doing well, as recent returns illustrate.    SWRCB Alternatives 3,4, and 5 could, however, adversely impact that fishery.

### Impacts of SWRCB Alternatives on Lower Mokelumne River Fishery

SWRCB flow Alternatives 3,4, and 5 would substantially deplete storage levels in EBMUD’s Pardee and Camanche Reservoirs in some years.    For example, Tables V-3 and V-4 show the carryover storage at Pardee Reservoir under the Draft EIR alternatives.    The carryover storage reported in Table V-4 for Pardee Reservoir, for Alternative 5 is incorrect.    Operation under Alternative 5 would draw Pardee Reservoir down to 76 TAF in the critical period, not the 95 TAF shown in Table V-4.    Depleting carryover storage levels increases the risk that water supply will become unavailable for both instream and consumptive uses, should dry conditions persist.    In addition to this risk, there is an increased likelihood that the hypolimnion (cold water) portion of those reservoirs would be lost.    Without the ability to release cold water to the lower Mokelumne River, an important tool in fisheries management is eliminated with the result that the fishery can be significantly, adversely affected, as explained below.

FIGURE 1.    Lower Mokelumne River Fall-Run Chinook Salmon Escapement, 1940-1997

FIGURE 2.    Lower Mokelumne River Fall-Run Chinook Salmon Escapement, 1990-1997 Natural Spawners

Camanche Reservoir Discharge Temperatures

Over the past several years, EBMUD has developed a flexible Pardee/Camanche Reservoir management system under which operating decisions are guided by monitoring of actual field conditions.    This approach allows EBMUD to maximize its ability to provide appropriate temperatures in the lower Mokelumne River, and also to minimize unnecessary releases from Pardee Reservoir that do not support beneficial uses of the river.    Appropriate temperatures can usually be maintained in Camanche Reservoir releases when stratification is established in the Spring and maintained in both Camanche and Pardee Reservoirs through the summer and fall until normal overturn in November. Camanche Reservoir needs to remain stratified to provide a cold water source that can be used for releases to the lower Mokelumne River; Pardee Reservoir needs to remain stratified to provide a cold water reserve that can be used to maintain stratification in Camanche Reservoir.

EBMUD’s analysis indicates that the higher flow requirements of SWRCB Alternatives 3, 4, and 5, will cause Pardee and Camanche Reservoir storage to be depleted in drier years, so that the ability to maintain stratification and to make cold water releases to the lower Mokelumne River is reduced.    This will result in discharges of warmer water to the river, with corresponding, adverse impacts on the anadromous fishery.

Examination of seasonal water temperature patterns on the lower Mokelumne River has shown that temperatures are generally suitable for both Chinook salmon and steelhead during the period from approximately November to April.    The period of greatest concern for adverse temperature effects on Chinook salmon and steelhead occurs from May through October.    EBMUD has implemented, as part of its fisheries protection and enhancement efforts for the lower Mokelumne River, a temperature management strategy which relies on management of releases seasonally from the upper and lower outlet structures on Camanche Dam in addition to management of coldwater storage seasonally between Pardee and Camanche Reservoirs.    The success of the water temperature management strategy is based, to a large extent, on the seasonal management of cold water within the hypolimnion of Camanche Reservoir.

Reservoir operations significantly influence cold water pool management and water temperature conditions within the lower Mokelumne River during the period of juvenile salmon and steelhead rearing.    Temperature conditions influenced by reservoir operations are particularly significant during the late spring period of juvenile Chinook salmon smolt emigration (May and June), and during the period of juvenile steelhead oversummering.    As a result of the natural seasonal patterns in water temperature conditions, and the influence of reservoir operations on water temperatures below Camanche Dam, the primary emphasis of these analyses on the impacts of alternative reservoir operational strategies on fisheries is focused on water temperature conditions suitable for emigration and juvenile rearing for both salmon and steelhead during the period from May through October.

The occurrence of elevated water temperatures relative to the thermal tolerance of each of the life stages of fall-run Chinook salmon suggest that the most sensitive time periods for the species occur during the fall period of adult upstream migration and early spawning (e.g., October through mid-November), and during the latter part of the spring juvenile emigration period (e.g., mid-May through June).

Exposure to adverse temperature conditions during May through October has the potential to adversely impact juvenile steelhead rearing and oversummering within the lower Mokelumne River, and juvenile steelhead and salmon rearing throughout the spring and summer months within the Mokelumne River Fish Hatchery.    Exposure to elevated water temperatures during September and October has the potential to adversely impact migrating adult Chinook salmon, in vitro development of salmon eggs and milt, and salmon egg incubation.    A significant portion of EBMUD’s plan for protecting and enhancing lower Mokelumne River Chinook salmon and steelhead populations relies on the production of yearling salmon and

steelhead within the Mokelumne River Fish Hatchery.    These fish would be vulnerable to impacts resulting from exposure to elevated water temperatures and increased turbidity levels throughout the May    through October time period.

Exposure to adverse elevated water temperature conditions over an extended period of time within a year (e.g., over a number of consecutive months) has the potential to adversely impact the production of an entire Mokelumne River year class.    Adverse temperature conditions occurring over a number of consecutive years has the potential to result in significant population level impacts that affect multiple year classes.    For example, Chinook salmon and steelhead typically have a 3- to 4-year life cycle, and hence adverse temperature impacts in three or more consecutive years would significantly impact the entire Mokelumne River population.

Impacts on a single year class and, more significantly, impacts on multiple year classes resulting from consecutive years of adverse conditions, have the potential to deplete Mokelumne-origin Chinook salmon or steelhead populations.    Rebuilding of the populations from these adverse conditions could take a number of generations.    In the event that adverse population-level impacts occur to lower Mokelumne River salmon or steelhead populations over a long time period, environmental conditions may be inadequate for population recovery.

#### Conclusion

Operating Camanche and Pardee Reservoirs under the Water Right Priority Alternative 3, the Water Right Priority Alternative 4, or the Watershed Allocation Alternative 5 would lead to substantially reduced storage in those reservoirs which in turn would increase the likelihood of losing the hypolimnion in Pardee and/or Camanche Reservoirs.    Loss of the hypolimnion would result in increases to the temperature of water released to the lower Mokelumne River. This would have a significant adverse impact on the quality of available habitat on the lower Mokelumne River, and introduce a significant risk of population-level impacts to the lower Mokelumne River salmon and steelhead stocks.    Thus, the SWRCB’s alternatives have the potential to damage what is now a healthy fishery in the lower Mokelumne River.

## B.    Reservoir Fisheries and Recreation

SWRCB flow alternatives would adversely impact both the fisheries within and recreation provided at EBMUD’s Pardee and Camanche Reservoirs.    While the Draft EIR does discuss reservoir recreation impacts, its discussion of impacts upon Pardee and Camanche Reservoirs is incomplete and contains some inaccurate analyses.    This section begins with a description of EBMUD’s Pardee and Camanche Reservoir resources, then summarizes likely impacts on those resources from SWRCB Alternatives 3,4, and 5.

### Reservoir Fishery

Pardee and Camanche Reservoirs support naturally sustaining populations of fish, including fish native to the Mokelumne River.    Stocking programs include rainbow trout and Kokanee salmon at Pardee Reservoir, and rainbow trout and largemouth black bass at Camanche Reservoir.    A detailed discussion of the fisheries at Pardee and Camanche Reservoirs is presented below.

#### Pardee Reservoir Fishery

Pardee Reservoir provides habitat for a wide variety of coldwater and warmwater fish species.    The warmwater fishery includes largemouth, spotted and smallmouth “black” bass; several species of sunfish; catfish; crappie; big-mouthed minnow, and warmwater non-game species such as carp, goldfish, and suckers.    The coldwater fishery includes rainbow trout, brown trout, and Kokanee salmon.    Table 1 lists the fish species known to occur in the reservoir.

TABLE 1.    Fish Species Known to Occur in Pardee Reservoir

| Fish Species |  |  |
| --- | --- | --- |
| Kokanee (Sockeye) Salmon | Redear Sunfish | Brown Bullhead |
| Rainbow Trout | Green Sunfish | Carp |
| Brown Trout | Big-mouthed Minnow | Goldfish |
| Largemouth “Black” Bass | White Crappie | Golden Shiner |
| Smallmouth “Black” Bass | Black Crappie | Sacramento Sucker |
| Spotted “Black” Bass | Channel Catfish | Prickly Sculpin |
| Bluegill | White Catfish | Mosquitofish |

Note:    Based on angler catch statistics and biological surveys.

The District and the California Department of Fish and Game (CDFG) maintain a significant rainbow trout stocking program at Pardee Reservoir.    In addition, Kokanee salmon fry are planted in Pardee Reservoir by CDFG and by a private organization, Project Kokanee.

#### Camanche Reservoir Fishery

Camanche Reservoir also provides habitat for both coldwater and warmwater fish species.    Table 2 lists fish species known to occur in Camanche Reservoir. Warmwater fish in Camanche Reservoir include largemouth, smallmouth, and spotted “black” bass; black and white crappie; redear, bluegill, and green sunfish; big-mouthed minnow, channel and white catfish; and white sturgeon.    Coldwater fish in Camanche Reservoir include rainbow trout and Kokanee salmon.    Forage fish include threadfin shad, inland silversides, golden shiner, and prickly sculpin.    Native non-game fish include hitch, Sacramento blackfish, and Sacramento sucker.

TABLE 2.    Fish Species Known to Occur in Camanche Reservoir

| Fish Species |  |  |
| --- | --- | --- |
| Kokanee (Sockeye) Salmon | Big-mouthed Minnow | Carp |
| Rainbow Trout | White Crappie | Goldfish |
| Largemouth “Black” Bass | Black Crappie | Golden Shiner |
| Smallmouth “Black” Bass | White Sturgeon | Threadfin Shad |
| Spotted “Black” Bass | Channel Catfish | Inland Silversides |
| Bluegill | White Catfish | Hitch |
| Redear Sunfish | Brown Bullhead | Sacramento Sucker |
| Green Sunfish | Sacramento Blackfish | Prickly Sculpin |
| Mosquitofish |  |  |

Note: Based on angler catch statistics and biological surveys.

The District and CDFG maintain fish stocking programs at Camanche Reservoir similar to those at Pardee Reservoir.    The District plants approximately 1,200 pounds of catchable rainbow trout per week from October through June.    Also, Channel catfish and Florida-strain largemouth black bass fry are planted in Camanche Reservoir.    The CDFG plants rainbow trout and Kokanee salmon.

### Reservoir Recreation

EBMUD has developed extensive recreation facilities at Pardee and Camanche Reservoirs, which it operates in accordance with Federal Energy Regulatory Commission (FERC) regulations.    These District facilities support a wide variety of recreation opportunities, including fishing, boating, camping, hunting, and recreational vehicle use.    In addition, there are 200 permanent mobile homes established at Camanche North and South Shore Recreation Areas, that are operated I accordance with California State Department of Housing and Community Development regulations.

There are four developed recreation facilities at Pardee and Camanche Reservoirs.    Combined, the two reservoirs accommodated over 500,000 visitors in 1995, and, in the six year period between 1991 and 1996, approximately 2.8 million people visited the two reservoirs.    Camanche Reservoir receives the majority of its visitors from May through August;    Pardee Reservoir receives the majority of its use from March through June.    Pardee Reservoir is closed to the public from November through January to provide a sanctuary for migratory waterfowl.

#### Pardee Recreation Area

The Pardee Reservoir Recreation Area is located in the Sierra Nevada foothills 28 miles northeast of Stockton.    Situated in the northwestern wing of Pardee Reservoir, the recreation area has been open to the public since 1958.    The recreation area currently offers both land and water oriented recreation activities. Site-specific facilities at Pardee Reservoir include: full-service marina with 10-lane boat ramp and bait and tackle shop; 1 low water boat ramp; bait and tackle shop; 5 designated campgrounds (100 campsites); parking lots for 400 vehicles; 7 day-use areas for picnicking; 2 swimming pools with restroom/shower complex; 97 short-term/monthly/seasonal RV sites with full hook-ups for sewage, water, and electricity; boat-trailer storage area; and other ancillary facilities including flush rest rooms, fish cleaning facilities, an office, maintenance shop, restaurant/store, laundry facility, gasoline station, and entry kiosk.

Pardee Reservoir supports both day and overnight use during the recreational season.    Most of the recreational activity is related to fishing.

#### Camanche Recreation Area

Camanche Reservoir is located downstream of Pardee Reservoir.    Camanche Reservoir has two regional recreation areas, Camanche North Shore and Camanche South Shore, which were developed and opened to the public in 1964.    These recreation areas are open year-round and provide for a wide variety of water-oriented activities.    Recreation activities allowed at the reservoir include swimming, water skiing, jet skiing, windsurfing, fishing and boating.    Recreation facilities include campsites, motel units, rental cottages, launch ramps, marinas, covered berths, playgrounds, tennis courts, flush rest rooms, showers, mobile home areas, stores, laundries, a restaurant, service stations, and maintenance shops.    In addition to these facilities, the recreation areas also offer group campsites, recreational trails, and a dry storage yard at Camanche North Shore; and fishing ponds, an amphitheater, a recreation hall, recreational trails, and RV sites at South Camanche Shore.

All amenities and recreation activities provided at Camanche are operated by a master concessionaire, under contract to the District.    The business operated by the concessionaire is dependent on the utilization of the recreation areas on a 365-day per year basis.

EBMUD leases property surrounding the Rabbit Creek arm of Camanche Reservoir to the Camanche Hills Hunting Preserve (CHHP), which operates a 1,500 acre public hunting area.    The provision of hunting at Camanche was a condition of the Camanche Reservoir project approval by the federal government.    The CHHP is a licensed Pheasant Club, Domesticated Migratory Game Bird Shooting Area, and Domesticated Game Breeder.    All licenses are issued by the California Department of Fish and Game.    The hunting club is open year round.

### Impacts of SWRCB Alternatives on Reservoir Fisheries and Recreation

The key indicator of impacts on reservoir fisheries and recreation is reservoir water level.    When extreme drawdown of reservoir waters occurs, Pardee and Camanche Reservoirs are subject to destratification, causing loss of the coldwater pool, resulting in temperatures throughout the reservoir that are adverse to coldwater fish species such as trout and Kokanee salmon in the reservoir and anadromous salmonids in the lower Mokelumne River.    Reservoir warmwater species can also be impacted as littoral habitat disappears.

Recreational facilities are also adversely impacted by extremely low reservoir levels.    Boat ramps, mooring and marina facilities and services that are normally accessible by boats become inaccessible when reservoir levels drop.

Severe drawdowns of EBMUD’s Pardee and Camanche Reservoirs have occurred only occasionally in the past, and then only during naturally occurring drought periods.    Additional increases in demand associated with implementation of the SWRCB’s proposed flow allocation Alternatives 3,4, and 5 could substantially increase the frequency and magnitude of extremely low reservoir elevations.    Severe drawdowns would result in significant impacts to existing reservoir fisheries and public recreational use, similar to impacts experienced at the reservoirs during the 1976-77 and 1988-92 droughts.

Pardee and Camanche Reservoirs both support a put and take recreational fishery supported by public restocking programs, and naturally sustaining populations of over twenty species of fish, including fish native to the Mokelumne River Watershed Basin.    Extreme drawdowns of Pardee and Camanche Reservoirs would impact available coldwater storage and the ability to maintain coldwater species within the reservoirs, downstream at the Mokelumne River Fish Hatchery and in the lower Mokelumne River.    Lowered reservoir elevations would also result in the loss of habitat, which would affect the littoral life stages of various fish species, especially the more vulnerable fry and young-of-year.

Recreational use at Pardee and Camanche Reservoirs is substantial, with millions of visitors in the past few years.    Lowered reservoir elevations would substantially decrease opportunities for public recreational use, as occurred at Pardee and Camanche during past drought periods.    Extreme drawdowns would force the closure of marinas and boat launch ramps, resulting in a loss of boat access and increased safety hazards.    These conditions would in turn reduce visitor use levels, as well as revenues from recreational use which are necessary for facility maintenance, ultimately resulting in reduced opportunities for public recreation.

In addition, there is a likelihood of successive years of drawdowns under SWRCB Alternatives 3,4, and 5. The occurrence of successive years of drawdown is significant because of the potential cumulative effects of such events.    Extreme drawdowns over multiple years could result in significant population-level impacts for various fish species, particularly coldwater species which depend on the presence of coldwater for survival.    Extreme drawdowns over multiple years would also likely have a compounding effect on concessionaire operations due to the cumulative economic impact of reduced visitation, as well as significant reduced public opportunities for recreation.

The Draft EIR, on page VI-67 and in Tables VI-51a, b, and c and VI-52, also fails to identify impacts to the shallow water, mid-water, and open-water habitat for waterfowl at Pardee and Camanche Reservoirs.    This habitat would be significantly affected by the extremely low reservoir levels which would result under SWRCB flow Alternatives 3, 4, and 5.    The EIR should note these waterfowl impacts created by SWRCB flow alternatives.

It is inaccurate to suggest, as the Draft EIR does in the Mitigation section beginning on page VI-94, that the adverse reservoir impacts will be partially offset by increased river recreation opportunities.    Not only is the Pardee and Camanche reservoir recreation usage several orders of magnitude greater than the current or any potential increase in recreational use of the lower Mokelumne River, additional river flows, as proposed by some of the draft EIR’s proposed alternatives, may well create hazardous recreational conditions along the lower Mokelumne River, thereby resulting in fewer, not greater, recreational opportunities on the river.

# III.    CUSTOMER AND SERVICE AREA IMPACTS

As the Draft EIR acknowledges, SWRCB flow alternatives will have an impact on EBMUD’s customers and service area.    (Page XI-8.)    However, the Draft EIR states that urban suppliers such as EBMUD can adjust to shortages caused by the SWRCB alternatives by water transfers and increasing conservation and the rationing of its customers.    In reality, those solutions may not alleviate the shortages to EBMUD caused by the SWRCB alternatives.    Outlined below is a summary of EBMUD’s service area, and EBMUD’s existing, proactive conservation and rationing programs.    Following that is an explanation of why the Draft EIR’s proposed water transfer, conservation and rationing “solutions” may not mitigate impacts on EBMUD.

## a.    EBMUD Service Area

The East Bay Municipal Utility District (EBMUD or District) serves approximately 1.2 million people and numerous industrial, commercial, and institutional water users in the San Francisco East Bay region with water obtained primarily from the Mokelumne River Basin in the Sierra Nevada Mountains.

EBMUD’s service area covers two counties, 20 cities, and 15 unincorporated communities within a 325 square-mile area. The District’s Mokelumne River Project supplies 95 percent of the water used by the 1.2 million people in the service area. EBMUD customers include residential, industrial, commercial, and institutional water users within the service area.    EBMUD’s service area and the Ultimate Service Boundary are shown in Figure 3.

## b.    Water Transfers

The Draft EIR’s reliance on water transfers to make up for water supply shortages caused by SWRCB flow alternatives is misplaced, as explained in EBMUD’S cover letter (attached).

## c.    Conservation and Rationing

The Draft EIR states that urban water suppliers can use conservation and rationing to make up for water shortages caused the SWRCB alternatives. (Page XI-8.)    But the ability of a supplier to save water through conservation is affected by what it is doing now in regard to    conservation and rationing opportunities.

EBMUD has been nationally recognized for its efficient water use practices.    District practices which optimize the use of its water supplies are incorporated into its future water supply planning and are summarized as follows:

- A strong drought management program which is based on a rationing level of up to 25 percent in drought years in addition to the long-term conservation and reclamation savings described below.

- Water service regulations to encourage efficient water use and prohibit its waste.    These include waste water regulations , landscape standards for new developments, and non-residential plan reviews.

- Use of an inclining rate structure to encourage conservation.

FIGURE 3.    EBMUD Water Supply Facilities and Service Area

- Corel Draw Map Image

- A proactive water conservation program designed to achieve water savings of 34 million gallons per day (MGD) by the year 2020.

- An extensive water reclamation program, with existing water savings of 5 MGD, and a plan to achieve an additional 8.3 MGD savings prior to the year 2020.

- An ongoing program to minimize distribution system water losses.    This program is based on corrosion control, leak detection and regular pipe replacement.

In short, EBMUD has implemented a comprehensive conservation and reclamation program.    EBMUD’s program has worked.    As the attached Figure 4 shows, since 1976 -- despite a steadily increasing account base – EBMUD’s total water consumption has remained relatively the same.    This demonstrates the effectiveness of EBMUD’s conservation (and reclamation and water use efficiency) programs.

Thus, when faced with water supply shortages caused by the SWRCB alternatives, EBMUD does not have the same ability to easily implement conservation measures to alleviate those shortages as would a supplier who has not already implemented a comprehensive conservation and reclamation program.

This means that the EIR’s conservation “solution” to shortages may, in reality, be no solution at all for EBMUD because it has already implemented such a program.

Finally, EBMUD has built into its water supply planning an assumption of a 25% mandatory rationing level.    EBMUD believes this is among the most aggressive rationing levels of urban water suppliers, and is evidence of EBMUD’s already existing commitment to rationing.    It also means EBMUD’s customers are currently shouldering a significant burden under dry

year conditions, and reduces the ability of the Draft EIR’s rationing solution to help EBMUD cope with shortages imposed by the SWRCB alternatives.

In conclusion, the SWRCB flow alternatives would cause excessive water shortages to EBMUD’s customers with the attendant economic impacts caused by such shortages.    Thus, the EIR must be revised to acknowledge the significant EBMUD customer and service area impacts of SWRCB Alternatives 3, 4, and 5, and to acknowledge that those impacts are potentially unmitigatable.

FIGURE 4.    Accounts vs. Consumption, 1961-1997

# IV.    MISCELLANEOUS

The Draft EIR identifies several irreversible impacts on the environment that could occur as a result of the proposed water right decision.    Another should be added.    As shown above, under the Water Rights Priority and Watershed Allocation alternatives (draft EIR Alternatives 3, 4, and 5) there is a significant risk of population level adverse impacts to the lower Mokelumne River Chinook salmon and steelhead stocks.    These population level impacts could threaten the abundance of lower Mokelumne River salmon and steelhead stocks.

On page XII-6, in the section on future actions with potential for cumulative effects, the description of EBMUD’s Folsom South Canal Connection Project incorrectly cites that, “The American River diversion would present risk to fish of impingement and entrainment at diversion facilities.”    Since delivery to EBMUD of its contract entitlement by the US Bureau of Reclamation through the Folsom South Canal would use existing USBR facilities, no additional impact to fish is anticipated.

Page XII-9 of the Draft EIR describes EBMUD’s alternative Pardee Reservoir Enlargement Project.    The project description should be supplemented to note that no environmental documentation is planned to occur on this project in the near future.    The second sentence of the description of potential project impacts should be revised to read:    “These exports may decrease overall Delta inflows from the Mokelumne River; however, minimum instream flows for the lower Mokelumne River would not decrease, but would be expected to increase due to the gain-sharing provision of the Mokelumne River Joint Settlement Agreement approved by the Resource Agencies and EBMUD.”

The Draft EIR describes major features of the environment in most of California, since most areas of California will be affected by this water right decision.    This description is overly generalized and incomplete. These areas are addressed below.

Page III-26, Paragraphs 5 and 6 of the Draft EIR, regarding Central Valley fish habitats, inaccurately generalize the reasons for differences in productivity between Central Valley lakes and reservoirs, and appears to be an attempt to justify any adverse impacts that the Draft EIR alternatives may have on water supply reservoirs.

The first complete paragraph on Page III-30 states that Delta smelt spawn in the Mokelumne River.    This is not an accurate statement.    Delta smelt may spawn in Delta channels of the Mokelumne River but not in the riverine portion (29.6 miles) of the River (54 percent of the Mokelumne River below Camanche Dam lies in the Delta and is tidally influenced).

Page III-34, Paragraphs 3 and 4, mischaracterizes the tradeoffs in recreation benefits between storage reservoirs and their downstream riverine environments.    It is not appropriate or accurate to compare the large variety and economic value of recreation at Central Valley Reservoirs, especially large, low elevation impoundments, to some minimal, potential increase in riverine recreation in low gradient rivers with slow meandering water.    This is especially true for rivers that have limited public access.    The description of the lower Mokelumne River, paragraphs 4 and 5 on Page III-91, is characterized incorrectly.    The lower Mokelumne River from Camanche Dam to the Delta is 29.6 miles, not 40 miles, as stated in the Draft EIR.    American shad are not found in the riverine portion of the Mokelumne River.

# V.    CONCLUSION

EBMUD has expended considerable time, effort, resources, and water in protecting and enhancing the anadromous fisheries and ecosystem of the lower Mokelumne River. EBMUD’s fisheries program is comprehensive, encompassing study, monitoring, and enhancement actions. The lower Mokelumne River anadromous fishery is doing well; recent years have seen average or above-average natural river escapement (returns) of adult salmon, good spawning and rearing conditions, and strong returns to the Mokelumne River Fish Hatchery.    Thus, the lower Mokelumne River, as an eastside    tributary to the Delta, is a healthy fishery.

The State Water Resources Control Board Water Rights Priority alternatives (Alternatives 3 and 4) and its Watershed Allocation alternative (Alternative 5) would cause significant adverse impacts to the lower Mokelumne River anadromous fishery. These fishery impacts would result from increased risk of water shortage for instream purposes and from increased frequency of warm water discharges from Camanche Reservoir, due to depletion of the Camanche and Pardee Reservoir hypolimnions. Those alternatives would also cause significant adverse impacts to fisheries and recreation at Pardee and Camanche Reservoirs.    And, importantly, some of these impacts on the lower Mokelumne River anadromous fishery are population level impacts.

In addition, the draft EIR alternatives would result in unreasonable impacts to EBMUD’s 1.2 million customers and service area economy in the San Francisco East Bay Area. These customer impacts would occur despite EBMUD’s existing and comprehensive conservation and reclamation program and its 25 percent mandatory rationing assumption, and the resulting customer burden, that EBMUD has already built into its long-term water planning.

In trying to implement the WQCP to benefit one part of the ecosystem (the Bay/Delta), the State Board must not simultaneously damage another part of the ecosystem (such as upstream tributaries).    The State Board’s proposed Water Rights Priority and Watershed Allocation alternatives (Alternatives 3, 4, and 5) would do precisely that.    They could significantly impact the lower Mokelumne River ecosystem, the reservoir fisheries and recreation values, and EBMUD’s customers.    The Draft EIR concludes that the effects of the SWRCB flow alternatives are virtually indistinguishable from each other.    (As the Draft EIR, on page VI-6, concludes, “The effects of Alternatives 2, 3, 4, and 7 on X2 are virtually indistinguishable from each other.”    And on page VI-19 it concludes, “In summary, there is no clear environmentally superior alternative in terms of the effects on salinity at specific stations in the Delta.” )    This demonstrates that there would be no significant differences in Delta water quality between the various alternatives.    But, as noted above, three of those alternatives (Alternatives 3, 4, and 5) could adversely impact the Mokelumne River ecosystem.    It therefore makes little sense for the SWRCB to implement    flow Alternatives 3, 4, or 5 because they would significantly impact the Mokelumne River ecosystem, recreation, and EBMUD customers, while providing no distinguishable Delta benefits over other flow alternatives that do not cause such adverse impacts.

EBMUD, the United States Fish and Wildlife Service (USFWS) and CDF&G initialed a Joint Settlement Agreement (JSA) in the FERC proceeding on the lower Mokelumne River, FERC proceeding No. 2916-004.  That initialed agreement was submitted to FERC in June, 1997.  FERC then engaged in Federal Endangered Species Act (ESA) consultation proceedings on the JSA.  At the conclusion of that consultation, the JSA was fully executed and just recently (in March 1998) submitted to FERC with a request to amend EBMUD’s FERC license to incorporate the flow regime contained in the JSA.  EBMUD anticipates FERC will render a decision in the FERC proceeding incorporating new release requirements upon EBMUD later in 1998.  At that time,  those newly imposed flows will constitute the then-existing requirement.   But at this time, FERC has not issued a decision in Proceeding No. 2916-004.  Thus, the existing release requirement remains the 1961 Agreement, discussed above.

While the JSA flows can not be considered as the Mokelumne River baseline, it would be appropriate to consider the JSA flows as part of the “No Project” alternative because, in the absence of SWRCB approval of this Project, FERC approval of the JSA flows could reasonably be expected to occur in the foreseeable future.  (EBMUD made this distinction earlier, in its July 3, 1996 comments to the SWRCB, pp. 1-2.)

This is not a new issue.  In its March, 1996 comments on the revised Notice of Preparation (NOP) for this proceeding, the District stated that because the “environmental reference condition” identified in the NOP was the hydrologic conditions in the Delta, upstream areas and  export areas during water years 1984-1994,  the existing regulatory requirement in effect during that period must be used.  That requirement is the 1961 Agreement.  (Please see EBMUD comments on the revised NOP, March 1996, pp. 1-2.)

Furthermore, the Draft EIR’s assumptions on the costs of transferred water to EBMUD are inaccurate.  The analysis in the Draft EIR assumes that costs and impacts of water transfers to EBMUD will be equivalent to those of the Santa Clara Valley Water District (SCVWD).  This comparison is not valid because EBMUD’s existing water treatment facilities are significantly different from those of SCVWD.  Treatment costs associated with water transferred to EBMUD could be substantially in excess of the 4250 to $350/AF referenced in the ties a, depending on the source of water transferred to EBMUD.

There appears to be a typographical error in the third sentence of the opening paragraph in this section of the draft EIR (page VI-40).  The sentence should be revised to read, “The evaluation of the environmental effects in the upstream areas …(emphasis added).

Impacts on Lower Mokelumne River Fishery

The Draft EIR states on page VI-46 that  “there is no reliable method available for most of the watershed to predict temperatures resulting from the changes in the flow alternatives.  Therefore, the effects of the flow alternatives on temperature are not analyzed.” (Emphasis added.)  This is a major oversight, first, because temperatures can be predicted for the Pardee/Camanche/lower Mokelumne system.  Second, because higher temperatures can have a significant, adverse impact on fisheries, as described below.