April 1, 1998

Victoria Whitney, Chief

Bay/Delta Unit

Division of Water Rights

State Water Resources Control Board

P. O. Box 2000

Sacramento, CA 95812-2000

This letter presents the comments of the State Water Contractors on the

State Water Resources Control Boardís November 1997 Draft Environmental

Impact Report for Implementation of the 1995 Bay/Delta Water Quality

Control Plan.    To avoid duplication and to provide focused and useful input

to the SWRCB, our comments are intended to supplement the comments of the

Department of Water Resources.

GENERAL COMMENTS

Regarding the alternatives presented in the DEIR for meeting the Bay-Delta

flow objectives, the State Water Contractors do not at this time have a

preferred alternative.    However, we do not support Alternatives 2, 6 and 7,

which would allocate sole responsibility for meeting Delta flow objectives

to the SWP and CVP.    We continue to work actively to develop, with other

parties, various settlement agreements that are expected to deal with the

potential responsibilities of various upstream water users with respect to

the obligation to meet Bay-Delta standards.    The State Water Contractors

expect to recommend a flow alternative during subsequent SWRCB water right

hearings with respect to non-settling parties.    We expect any allocation to

be consistent with the Racanelli decision and the State Constitution.

Apart from the issue of flow, the State Water Contractors do prefer certain

alternatives that will resolve other specific issues raised in the DEIR.

First, regarding Suisun Marsh, we recommend Alternative 5 with

modifications that include an update of the actions proposed in the Suisun

Marsh Preservation Agreement Amendment III (the SMPA Amendment does not

include Green Valley Creek flow augmentation nor redirection of Morrow

Island drainage to Suisun Bay).    Second, we strongly support the Vernalis

Adaptive Management Program (VAMP), which sets forth flows, fishery

investigations and other measures that are supported by a broad group of

regulatory agencies, fisheries experts, water interests and other

stakeholders.    Regarding the South Delta we support implementation of the

South Delta flow control structures as set forth in DWRís Interim South

Delta Program (ISDP).    We also support DWRís recommendations with regard to

the dissolved oxygen objectives.

The DEIR also addresses the proposed ìjoint point of diversionî for SWP and

CVP diversions from the Delta.    We support alternative 5, which authorizes

implementation consistent with the existing operating criteria.    As

indicated in DWRís comments, such implementation is subject to development

by DWR and the USBR of agreements setting forth the conditions under which

use of both diversion facilities will occur.    We also support the proposed

consolidated place of use for the CVP.

As identified in our specific comments, which are presented below, the

draft Environmental Impact Report (DEIR) should be critically reviewed and

revised to address general concerns with regard to:

Improved documentation and discussion regarding the uncertainties,

constraints, and lack of specific scientific data necessary to

comprehensively evaluate various aspects of the proposed alternatives. The

draft EIR text would be improved by including additional discussion

regarding the inherent variability in many of the relationships upon which

analyses and models presented in the EIR are founded, the absence or

weakness in available scientific information, and the degree of confidence

that can be placed in the resulting indices or predictions regarding

differences among alternatives;

Many of the analyses presented in the draft EIR should be updated to

reflect the most recent information available.    In a number of instances

the information used in developing predictions for the EIR and for

comparisons among alternatives rely on biological data collected through

the 1980ís or early 1990ís and have not been updated to reflect results of

the most recent scientific investigations;

Throughout the text, statements are presented as fact, often citing other

references as the source of these findings, many of which are either draft

or have not been peer-reviewed, which would more legitimately be

characterized as hypotheses or speculation.    The text should be critically

reviewed and revised to clearly articulate those areas which represent a

broad consensus regarding factual conclusions of specific investigations

and studies versus those areas which have not been substantiated through

detailed scientific peer-reviewed analyses or investigations.    The text

should also clearly note those areas where there is currently scientific

controversy or uncertainty regarding results, analysis, and interpretation

of information from various investigations.    The disclosure of areas of

controversy and an expanded discussion regarding the strength and weakness

of available information presented as the technical and scientific

foundation for analyses presented in the draft EIR should be included in

the final document;

Key individuals within DWR and other organizations involved in the South

Delta Barrier Program (particularly those individuals involved in

evaluating the biological issues and benefits of the Head of Old River

barrier), the lower San Joaquin River salmon smolt survival investigations

(VAMP), and other program areas being evaluated in the EIR, should be

consulted to incorporate the latest technical information, results of

recent scientific investigations, and recent changes in these proposed

programs regarding experimental design, testing and investigations, or

mitigation obligations to reflect the most contemporary information

available.    The draft EIR reflects information that is several years old

regarding these important issues.

Our comments are in a format, which cites DEIR page number and/or text,

followed by our specific remarks.

SPECIFIC COMMENTS

Comment (pages II-16 through II-18):    Flow Alternative 3 is described as an

alternative that assigns responsibility for meeting Bay/Delta flow

objectives to all water right holders based on the water right priority

system. Under Alternative 3, junior appropriative right holders would be

required to cease diversions to meet objectives before more senior right

holders. As the text further acknowledges, pre-1914 appropriators and

riparian right holders also could be affected under severe conditions when

cessation of diversions by post-1914 right holders is alone insufficient to

achieve Bay/Delta requirements.    The State Water Contractors have several

concerns with regard to this discussion.

First, we share the concern expressed in the comments submitted by DWR that

the discussion mischaracterizes the purpose and effect of Term 91.    The

purpose of Term 91 is to protect the previously stored (or imported) water

of the SWP and CVP from in-basin diversion when that water is released to

meet Delta water quality standards, and that purpose applies without regard

to the relative priority of in-basin diverters.    We strongly endorse the

substitute language proposed by DWR, for inclusion in the FEIR in order to

correct the misimpression conveyed by the DEIR in this regard.

Second, we agree that it is important to recognize the potential

responsibilities of pre-1914 appropriators and riparian right holders to

meet Bay/Delta flow objectives. We are concerned, however, that the

analysis contained in the DEIR may not fully take those responsibilities

into account. For example, in evaluating, the potential impacts of

Alternative 3, the DEIR focuses on post-1914 appropriative right holders

and assumes a mechanism for curtailing diversions among these right holders

based on a modified Standard Water Right Permit Term 91 approach. By

emphasizing potential modifications to permit terms and conditions as the

principal means for assuring compliance with Bay/Delta standards, the text

fails to address the SWRCB's authority to regulate water users who are not

required to have permits, i.e., pre-1914 appropriators and riparian right

holders.

Finally, while we appreciate that an analysis of impacts on pre-1914

appropriators and riparians may be difficult, we believe that the text in

this section should be revised to clarify the SWRCB's authority to require,

by regulation or other means, all water users to contribute as necessary

toward compliance with Bay/Delta flow objectives, consistent with Article

X, Section 2 of the State Constitution and other applicable provisions of

law.

Comment (page II-29):    We agree with the statement concerning the lack of

hydraulic continuity with the Delta of Putah and Cache creeks, at least

during periods when Delta flow standards are controlling on SWP and CVP

operations. We are working with parties in both watersheds on stipulations

that recognize this lack of continuity and relieve the parties of the

obligation to contribute to Delta outflow to meet flow objectives.

Comment (page IV-6):    Reference is made to ìdraft Monterey Agreement

criteria.î    Most SWP water contractors have signed the ìMonterey Amendmentî

which amends their water supply contracts with the State to include

provisions of the ìMonterey Agreement.î    The so-called ìcriteriaî are

provisions of the water supply contracts.

Comment (pages IV-14 through IV-23):    There is a discussion on these pages

regarding the methodology used to allocate responsibility for meeting the

Delta flow objectives for Alternatives 3 and 4.    We are concerned with

three aspects of this methodology:    (1) the apparent arbitrary use and

designation of priority groups;    (2) the frequency and magnitude of the

outflow shortfalls (unallocated water) resulting from the proposed

methodology;    and (3) the presumption that it is the responsibility of the

SWP and CVP to make flow releases to meet these shortfalls.

The water right priority system methodology arbitrarily chose eight water

right priority groups as a means for allocating responsibility to water

right holders for meeting Delta outflow objectives. The coarseness of the

gradations among priority groups leaves significant outflow shortfalls,

which are presumed to be an additional project obligation. The amount of

these shortfalls can exceed    600,000 acre-feet per year, while the average

shortfall is 230,000 acre-feet and 282,000 acre-feet per year (both

calculated over a 73-year period of hydrologic record), respectively for

Alternative 3 and Alternative 4.

For example, under Alternative 3, in July 1925, it was determined that

water right holders were diverting 512,000 acre-feet beyond their in-basin

rights. Under the proposed allocation methodology, water right holders

included in priority groups 1 through 7 would have been directed to cease

diverting during this period. Such action would have supplemented Delta

outflow by 289,000 acre-feet, leaving an outflow shortfall of 223,000

acre-feet.    If priority groups 1 through 8 had been directed to cease

diverting, the amount of supplemental outflow generated by this action

would have exceeded the required 512,000 acre-feet.    In other words, only

some of the members of priority group 8 would have been required to cease

diverting in order to properly allocate responsibility for meeting Delta

outflow standards.    Because fewer than all of the diverters in priority

group 8 were required to cease diverting, under the Boardís methodology

none of the members of priority group 8 are required to cease diverting,

thus leaving the remaining 233,000 acre-feet of required outflow to be

provided by the projects.    This particular ”on-off-switch” methodology

results in an enormous, unwarranted burden on the SWP and CVP.

For purposes of Alternatives 3 and 4, we recommend directing water right

holders to curtail diversions one-at-a-time, rather than by priority

groupings, such as done for the Vernalis flow objective, to minimize

outflow shortfalls and avoid any unfair reallocation of burdens to the SWP

and CVP.

Comment (page VI-1):	“…flow, salinity, and the condition of fish and

wildlife are closely correlated in the delta.”

The relationship between flow, salinity, and the biological response of

various fish and invertebrate populations is both complex and highly

dynamic.    The statement overly simplifies the strength of the correlation

between these environmental parameters and the corresponding abundance

and/or condition of either fish or invertebrate populations within the

Delta.    Very little quantitative information has been developed on the

relationship between flow, salinity, and ”wildlife” populations within the

Delta, other than indirect effects on vegetation and other habitat

parameters.

Comment (page VI-2):	”The gates are required to be closed in the winter

and spring to reduce the diversion of eggs, smelt, and larvae into the

central Delta where survival is generally reduced.”

Closure of the Delta Cross-channel gates is intended to reduce the movement

of planktonic fish eggs and larvae from the Sacramento River into the

Delta.    Closure of the gates is also intended to improve survival of

downstream migrating Chinook salmon smolts.    Closure of the Cross-channel

gates was not identified as a major management action designed to protect

“smelts.”

Comment (page VI-6):	Within the discussion of the effects of various

alternatives on the location of X2 the observed differences in average

location among the various alternatives are not likely to be biologically

significant.    The location of    X2 varies substantially based on daily tidal

dynamics and variation in freshwater outflow occurring within a month.

Although the modeling results provide a convenient tool for comparing

alternatives, the relatively small resulting difference in the location of

X2 should be interpreted in context with the known variation occurring in

X2 location within and between years within the system.    Given the

magnitude of naturally occurring variation, differences predicted by the

models in the location of X2 (Table VI-11) are not likely to be

biologically significant.

Comment (page VI-18):    There is a discussion concerning modeling

assumptions with regard to meeting the chloride objectives at Contra Costa

Canal Pumping Plant No. 1.    The modeling analysis needs to take into

account the recent implementation of the intake to Contra Costa Water

District’s Los Vaqueros Project, which significantly reduces the flow in

channels between Old River and Pumping Plant No. 1.    The influence of local

agricultural drainage and the low net flow toward Pumping Plant No. 1 will

at times mean that no actions taken by the SWP and CVP can control chloride

concentrations at that location.    Accordingly, we support moving the

location of this objective to a location where quality is within the

control of those assigned such responsibility.

Comment (page VI-20):	“Approximately 80% of California’s commercial

fishery depends on species that inhabit or migrate through the estuary

(USBR 1997a).”

This statement should be checked and verified.    Given the extensive

commercial fishery for species such as rockfish, Dungeness crabs, and other

marine coastal fisheries extending from Southern to Northern California, I

seriously doubt the statement that 80% of these fisheries are dependent

upon the estuary.

Comment (page VI-21):	”Currently SWP and CVP exports can reach

approximately 10,000 cfs most of the year with higher levels possible in

the winter.”

This statement is not correct given the conditions imposed on SWP and CVP

exports through the Delta Accord.    Additional restrictions imposed by the

USFWS Delta Smelt Biological Opinion also serve to limit SWP/CVP exports,

particularly during the critical spring period.    The text should be

modified to accurately reflect export conditions, particularly during the

spring (February - June) rather than giving the impression that SWP/CVP

exports are 10,000 cfs throughout this period in all years.

Comment (page VI-21):	“Agricultural diversions, which peak between April

and August (with an estimated combined flow of 4,000 cfs)    . . .”

A citation should be provided for the technical basis used to estimate that

agricultural diversions from the Delta are approximately 4,000 cfs.    In

earlier discussions with representatives of DWR and others it was stated

that agricultural diversions, although not quantified, might be equal to

the diversion volume from the SWP and CVP combined during spring period.

Comment (page VI-22):	The discussion of QWEST, although accurate, is

confusing.    The text currently states that as QWEST decreases reverse flows

increase.    Results of model output show that QWEST increases when compared

with a base case, during the spring, which would be beneficial.    The

difficulty arises in the choice of the words increase and decrease in QWEST

value.    The text should be reviewed to determine whether alternative

language might help clarify the concepts and results presented from these

analyses.

Comment (page VI-24):	“Many contaminants naturally accumulate in the

entrapment zone of the Estuary, which is preferred by many Delta organisms,

increasing exposure.”

References should be cited providing the technical data to support this

statement.    References to the scientific investigations, which demonstrate

the longitudinal gradient of contaminant concentrations with an

accumulation of contaminants within the entrapment zone should be

documented as part of the background information presented in this section

of the EIR.

Comment (page VI-25):	The discussion of the status of Chinook salmon

should be updated to reflect the most recent proposal for listing various

races of Central Valley salmon under the Federal Endangered Species Act by

NMFS.

Comment:	“Higher flow rates increase salmon populations by

increasing smolt migration survival rates, reducing exposure to diversions,

and maintaining favorable water quality conditions during migration (USBR

1997a)” (page VI-25).

The effects of flow on Chinook salmon habitat for spawning, egg incubation,

juvenile rearing, and outmigration, in addition to the effects of flow

variation on survival are complex and dynamic.    Salmon production (juvenile

salmon abundance within a year) generally is higher with increased flow,

which may reflect a variety of factors.    Exceptionally high flows within

the upstream spawning areas may also be detrimental to salmon production as

a result of scouring eggs from redds.    The statement in the EIR should be

expanded to provide additional information regarding the dynamics and

variability in the effect of flow on salmon within the freshwater

environment for various lifestages, and should not simply rely on general

statements contained in USBR (1997a) regarding these key issues.

Comment (page VI-25):	”Losses of Chinook salmon at the SWP and CVP Delta

export facilities typically range from 400,000 to 800,000 per year (USFWS

1995).”

The text should also explain that the Four-Pump Mitigation Agreement for

the State Water Project and the Tracy Agreement for the CVP identify

entrainment losses of juvenile Chinook salmon and provide for mitigation of

direct losses.    The EIR text should also explicitly note that the estimated

losses are for salmon fry (<2 inches in length) and smolts (<4 inches in

length), to avoid any confusion that these could be adult fish.

Comment (page VI-26):	”Models that calculate expected survival of

Sacramento River fall-run, late fall-run, and winter-run Chinook salmon,

and survival of San Joaquin River fall-run salmon were generated for this

analysis.”

The text of the EIR discusses the use of multiple-regression techniques and

coded-wire tag smolt survival studies as the basis for establishing the

salmon smolt survival models.    We are not aware of any such detailed

investigations or analyses that would support the development of a survival

model, specifically for either late fall-run or winter-run Chinook salmon.

The technical data upon which these models have been based should be

documented as part of the EIR, and made available for peer review.    In the

event that data from fall-run Chinook salmon studies, conducted during the

spring, were used as a surrogate for developing estimates of late fall- and

winter-run survival on the Sacramento River, this should be noted and

documented as part of the EIR.    The existing regression models for fall-run

salmon are strongly influenced by water temperatures during the spring.

Migration times for other races such as winter-run, occur during periods of

the year when temperature is not thought to be a key issue, and hence the

development of a model based on fall-run coded-wire tag survival studies

would not be representative or an appropriate surrogate for use in these

analyses.    The EIR should contain additional documentation on the actual

data and formulation of the models, as described.

Furthermore, there is currently considerable debate and discussion

regarding the effects of flow on salmon smolt survival from the San Joaquin

River within the controllable range being evaluated as part of the EIR.

The effects of flow on San Joaquin salmon smolt survival is part of the

underpinning for the proposed VAMP testing program.

Comment (page VI-28):	“Delta smelt are important food source for

commercially valuable predator species like salmon and steelhead.”

The scientific studies used to determine the importance of delta smelt as a

food resource for salmon and steelhead within the Bay-Delta system should

be referenced.    Delta smelt are a component of the diet of species such as

sub-adult and adult striped bass.    Since the majority of Chinook salmon

emigrate through the Delta as fry and smolts, they would not be expected to

prey on delta smelt in substantial numbers.    Yearling Chinook salmon and

steelhead may prey on delta smelt, however the residence time of these

yearlings within the Delta environment is relatively short.    Adult salmon

and steelhead may also forage on delta smelt incidentally, however the

importance of delta smelt in the diet of these adult fish is not expected

to be significant.

Comment (page VI-29):	“Reduced Delta outflow is thought to have greater

effects on smelt abundance than entrainment (USBR 1997a).”

The basis for this statement should be critically reviewed.    After

extensive analysis and discussion, the delta smelt Recovery Team was unable

to quantitatively prioritize the significance of changes in Delta outflow

versus entrainment losses as factors influencing delta smelt population

abundance.    The data and analyses upon which the statement presented in the

EIR are based should be referenced and subject to scientific peer review.

Comment (page VI-29):	“Contaminants have also been found to affect delta

smelt abundances.”

The use of correlation analyses between copper application and delta smelt

abundance in the fall mid-water trawl is suggestive, but not conclusive

evidence, of a link to contaminants.    Although many of the investigators

working on delta smelt population abundance speculate that contaminants may

be a significant factor, the actual scientific data necessary to support

such a finding are not yet available.

Comment (PAGE VI-30):	“Adequate flows reduce predation because the smelt

are more dispersed, the water is more turbid, and it increases food

production (USBR 1997a).”

Throughout much of the text statements are presented as fact, however these

statements are not supported by rigorous technical analysis or scientific

data.    In most cases the strong and positive statements made in the text,

many of which were attributable to USBR (1997a), would be better

characterized as hypotheses or speculation.

Comment (page VI-30):	The paragraph starting with “29A significant

Positive…” should be revised to delete “29.”

Comment (page VI-30):	Results presented in Figure VI-23 for longfin smelt

simply reflect results of a linear regression between an index of abundance

and December - May Delta outflow.    Within this context, any alternative

that has higher flow during the specified period will produce a higher

index of abundance.    The analysis, although useful for comparing

alternatives, fails to address issues such as variation in the regression

analysis (e.g., we would speculate that none of the model results would be

significantly different from each other if 95% confidence intervals were

placed around each abundance estimate), and the fact that a variety of

other factors interact with spring Delta outflow to influence the abundance

of longfin smelt.    These other influences cannot be taken into account as

part of the modeling performed in support of the EIR.

Comment (page VI-30):	”Delta diversions, including SWP and CVP pumping

facilities, coupled with upstream storage and reservoirs, may adversely

affect spawning adults by reducing freshwater flow and the availability of

temporarily flooded habitat.”

There does not appear to be any information to support the contention

presented in the EIR that Delta diversions at the SWP and CVP facility

would adversely affect the availability of temporarily flooded habitat.

Operation of the SWP/CVP diversions would have minimal effect on water

surface elevations within Delta regions potentially used as spawning

habitat.    Tidal action is a dominant factor influencing water surface

elevations, rather than diversions at the SWP/CVP facilities.    Furthermore,

the incremental magnitude of diversions at the SWP/CVP during periods when

flood flows create temporarily inundated habitat used by splittail for

spawning in upstream rivers of the system tend to be reduced both in

absolute magnitude and as a percentage of Delta inflow.    The scientific

data and logic used to support the statement presented in the EIR should be

made explicit or the statement should be modified.

Comment (page VI-31):	Figure VI-24 on the relationship between March -

May Delta outflow and splittail abundance is subject to the same concerns

as addressed above regarding Figure VI-23.    This comment applies to most of

the analyses presented in this section of the draft EIR.

Comment (page VI-32):	”This remains their primary food source until their

second year when they are large enough to feed on Bay shrimp and small

forage fish.”

Yearling striped bass actively forage on a variety of organisms, in

addition to mysid shrimp.

Comment (page VI-32):	Using DFG’s data, U.C. Davis researchers (Botsford

1994) developed a model to predict striped bass abundance (DFG 1992a).”

Concerns regarding the application of the striped bass model have been

addressed in earlier comments.    The development of a striped bass model,

based on relatively few parameters such as Delta outflow and SWP/CVP

exports, although a useful tool for comparing alternative operational

strategies fails to take into account the dynamic nature of a variety of

interacting factors affecting the Bay-Delta striped bass population.

Although the results of the modeling effort seem to be reasonable with

regard to the comparison among alternatives, caution should be used in

giving too much weight to the absolute results developed by this model.

Comment (page VI-32):	The discussion of explanations for reduced American

shad abundance at lower Delta outflows presented on this page and

attributable to USBR (1997a), as discussed earlier, represent hypotheses

and speculation rather than scientifically supported factual explanations.

Comment (page VI-34):	”Starry flounder spawn in winter and early spring,

and abundance is correlated to outflow during the same period (DFG 1993).”

The relationship developed between indices of starry flounder abundance and

freshwater outflow primarily were derived using data from the 1980ís. Have

these relationships been updated since the DFG (1993) report to include the

additional data collected from the Bay studies through the 1990ís?    For

many of the earlier relationships, the inclusion of data from more recent

years has shown either substantial increases in variability in the earlier

reported relationship or, for some species, a deterioration of the

fundamental relationships reported in the earlier analyses.    The most

recent and comprehensive data available should be used as the basis for

analyses comparing alternatives within the draft EIR.    This same concern

exists regarding Crangon and other species for which flow-based analyses

are presented.    As discussed in earlier comments, variability about the

regression analyses, and the influence of a variety of other factors on

actual population dynamics of these species, should also be incorporated as

part of the discussion and an interpretation of results presented from

these analyses.

Comment (page VI-46):	“Unscreened diversion entrain eggs and larvae.”

Results of monitoring conducted on the Sacramento River have demonstrated

that in addition to eggs and larvae, juvenile, and in some cases adult

salmonids are also entrained at unscreened water diversions.

Comment (page VI-46):	“…there is no reliable method available for most of

the watershed to predict temperature resulting from changes in flow

alternatives.”

Detailed water temperature models are available for the upper Sacramento

River, Yuba River, Feather River, American River, and San Joaquin River and

its tributaries.    Water temperature models have also been developed for the

Mokelumne River.    These available models could be used, if needed, to

provide additional information regarding the potential effects of various

flow alternatives on seasonal water temperature conditions.

Comment (page VI-46):	“It can be reasonably assumed, however, that

temperature and water quality conditions are positively affected by

increases in flow.”

The relationship between water temperature and flow is considerably more

complex than suggested by the draft EIR.    In general, within a year, water

temperature does decrease within a stream segment as a function of

increased instream flow releases; the relationship, however, is

considerably more dynamic when viewed in context with: (1) the effects of

various flow releases on upstream reservoir limnological conditions, (2)

the availability of a cold water pool, (3) the location of the release

point with regard to water surface elevation within the reservoir, (4)

seasonal timing, and (5) the effects of increased instream flows in one

year affecting carryover storage and subsequent limnological conditions

within the upstream impoundments in subsequent years.

Comment (page VI-47):	“Chinook salmon support a commercial and sport

fishery and are listed as endangered under both the State and federal

ESA’s”

This sentence should be revised to more accurately reflect that only

winter-run Chinook salmon are listed under the ESA’s, while fall-run

Chinook salmon, many of which are hatchery produced, are the dominant race

supporting the sport and commercial fisheries.

Comment (page VI-47):	“Steelhead in the Central Valley Evolutionarily

Significant Unit were proposed for listing as endangered by the NMFS in

July 1996 (NMFS 1996).”

NMFS has made a final determination regarding the listing of steelhead

within the Central Valley system as of 1998.    The draft EIR text should be

updated to reflect the most current status of steelhead and other species

considered in these analyses.

Comment (page VI-49):	”Within the range of flows recommended in the

working paper, higher flows are, in general, considered to provide better

quality riverene habitat and are assumed to increase habitat availability

for Chinook salmon, steelhead trout, American shad, and green and white

sturgeon.”

Habitat preference curves for many of the lifestages of Chinook salmon and

steelhead show that higher flows do not necessarily provide better habitat.

Given the relatively broad ranges of flows shown in Table VI-29, the

assumption in the draft EIR that higher flows are considered to provide

better quality riverene habitat and to increase habitat availability, may

be invalid.    These relationships should be supported by additional

life-stage specific analyses within the range of flows being considered.

Comment (page VI-49):	”The April through June period was selected for

this summary analysis because:    (1) it is the most biologically important

period on the river…”

It should also be pointed out in the draft EIR that there remains

controversy regarding the recommended flow ranges for most of the river

systems and species included in the AFRP working paper, as summarized in

Table VI-29.    Although these recommended flow ranges provide a convenient

tool for comparing alternatives, it would be premature and inappropriate to

conclude that the proposed actions are inadequate simply because they do

not meet a specific recommended flow contained in the AFRP documents.

Comment (page VI-57):	Methods of Analysis for largemouth bass spawning

and juvenile habitat within upstream reservoirs are based on a relatively

complex reservoir habitat index value.    The technical bases upon which many

of the parameters used in this habitat index do not appear to be based on

specific scientific studies or data, but rather appear to be derived from

assumptions.    Although the impact assessment of the proposed alternatives

on upstream fisheries population, and particular warmwater species such as

largemouth bass, is extremely important, it is difficult to evaluate the

resulting habitat index.    Alternative approaches have been used in other

water supply and water transfer EIR documents, which provide simpler

methods for analyzing potential impacts of downstream water projects on

warmwater fisheries habitat within a reservoir.    Examples of these

reservoir habitat impact criteria can be found in EIRs prepared for the

American River related to flood control operations, the Fazio water

transfer (1997), and the EBMUD 1997 supplemental water supply project.

Comment (page VI-59):	”These impacts cannot be mitigated.”

Impacts related to reservoir drawdown may potentially be mitigated through

activities such as reservoir habitat improvement.    In a number of Central

Valley reservoirs, habitat improvement projects including the placement of

brush piles and additional habitat features, over a range of water depths,

has been used to provide refugia and to improve habitat for warmwater

species as the reservoirs are drawn down.    This mitigation does not address

problems associated with dewatering of nests due to reservoir drawdown, but

does provide some additional benefits for juvenile and older lifestages.

Comment (pages VII-22, 24 and 25):    We agree    with DWRís recommendations

regarding the description of Suisun Marsh Preservation Agreement proposed

Amendment III.

Comment (page IX-42):	”If San Joaquin River flow exceeds 5,000 cfs, the

temporary Head of Old River Barrier is removed.”

Current information used in planning the VAMP program suggests that the

temporary Head of Old River Barrier cannot be installed when San Joaquin

River flows exceed 5,000 cfs, however once installed the temporary barrier

can functionally operate up to a San Joaquin River flow level of

approximately 7,500 cfs.    Potential discrepancies regarding barrier

installation and operations, and the assumptions used in Table IX-2, should

be reconciled with representatives of the Department of Water Resources.

Comment (page IX-42):	”The proposed barriers would affect neither Delta

inflow nor Delta outflow, and would not affect outflow/abundance

relationships discussed in Chapter 6.”

There does not appear to be a technically sound basis for the underlying

assumption that the proposed barriers would not affect outflow/abundance

relationships.    For example, installation of the Head of Old River Barrier

during the spring is expected to influence the relationship between San

Joaquin River flow or Delta outflow and the survival and abundance

relationship for San Joaquin origin fall-run Chinook salmon.    Installation

of the barrier may also affect the survival and abundance of splittail

produced in the lower San Joaquin River as observed in recent years.

Comment (page IX-43):	”Operation of the Old River and Middle River

permanent barrier in the first part of April and the Head of Old River

Barrier in late April coincides with upstream migration of American shad,

sturgeon, Delta smelt, and longfin smelt....”

In general the upstream migration of species such as delta and longfin

smelt occurs earlier than April.    Care should be taken in these analyses to

insure that the seasonal timing of the various life history stages of

targeted species coincides appropriately with the period included in the

analysis of alternatives.    Similarly, the reference to the upstream

spawning migration of adult late-fall-run salmon (page IX-43) should be

critically reviewed with regard to the April time frame included in these

analyses.    The report also makes reference to migration for species such as

winter-run Chinook salmon and spring-run Chinook salmon, both of which

occur on the Sacramento River and its tributaries.    Since the Delta

Cross-channel would be closed during April, a serious question arises

whether winter-run Chinook salmon and spring-run Chinook salmon would be

adversely affected by changes in Delta hydraulics during April as a result

of barrier operations.

Comment (page IX-43):	“Virtually all the species are considered to be

present during June, July, and August in some years when the Grantline

Canal permanent barrier is operated.    Operation of the barrier during this

period may cause the same problems as in April.”

Since this paragraph follows the identification of a variety of fish

species including, but not limited to, fall-run, spring-run, winter-run,

late fall-run Chinook salmon and steelhead, it is implied that these

species would also be vulnerable to potential impacts associated with

Grantline Barrier operations during the summer (June-August).    As a result

of significantly elevated water temperatures in the lower San Joaquin River

and southern Delta during the summer, conditions in this area are not

suitable for either juvenile salmon or steelhead.    There is no scientific

evidence to suggest that species such as juvenile winter-run Chinook salmon

or other salmon and steelhead would be present in the area during June,

July, or August as suggested in the DEIR text.    The seasonal timing and

geographic occurrence of the various fish species and lifestages used in

this analysis should be critically reviewed to insure that they are

appropriate.    The text should be modified, as appropriate, to explicitly

identify the species and lifestages to be included in the analysis at

various time periods during the year.

A similar issue arises in the next paragraph on page IX-43, which states

that barrier operation in October coincides with the “peak upstream

migration” of adult Chinook salmon and steelhead.    Although fall-run

Chinook salmon do migrate into the Sacramento and San Joaquin river systems

in October, there is no evidence to suggest that the peak period of adult

steelhead migration in either the Sacramento or San Joaquin river systems

occurs in October.    Generally the peak of steelhead migration is thought to

occur later in the winter.    Since the seasonal timing of adult upstream

migration for species such as steelhead is incorrect in this paragraph, the

statement that “the additional operation of these two barriers also has the

potential to cause blocked passage, straying, and increased entrainment

problems for these species”, is incorrect in October.

Comment (page IX-44):	”The permanent barrier project is considered to

have potentially significant adverse impacts with no identifiable benefits

for all of the species mentioned above, with the possible exception of San

Joaquin fall-run Chinook salmon.”

The level of analysis that is documented as part of the draft EIR is

inadequate to support such a finding for ìall of the species mentioned

aboveî.    The potential significance of changes in Delta hydraulics

associated with barrier operations, particularly in combination with

changes in Delta Cross-channel gate operations, SWP/CVP exports, and

modifications to flows requires substantially more analysis in order to

evaluate potential impacts for various fish species, particularly those

which primarily migrate into the Sacramento River.    The frequency,

magnitude, and potential significance of fishery impacts to the target

species identified in this analysis cannot be assessed based on the

information presented in this section of the draft EIR.

Representatives of the California Department of Water Resources involved in

preparing the fisheries impact analysis for the Interim South Delta Program

(ISDP) should be consulted regarding the analysis and interpretation of

potential fisheries impacts to be addressed as part of the SWRCB draft EIR.

The draft EIR/EIS for the ISDP has undergone public review and comment and

is currently being revised by the Department of Water Resources.    The most

recent analysis of key issues related to potential fisheries impacts and

benefits should be synthesized and incorporated into the SWRCB draft EIR.

In addition to the consultation regarding the evaluation of potential

fishery impacts and benefits, the Department of Water Resources and other

individuals involved in the evaluation of the ISDP should also be consulted

regarding the identification of appropriate mitigation measures.    Results

of these discussions should be incorporated into this section of the SWRCB

draft EIR to reflect the most recent scientific data and analysis available

regarding these issues.

The draft EIR should also note that, as part of lower San Joaquin River

Chinook salmon smolt survival studies, experiments are continuing to be

conducted to assess and evaluate fisheries benefits and impacts associated

with the Head of Old River Barrier.    The Head of Old River Barrier has been

identified as a key element in the study design of VAMP.

As discussed above the SWRCB draft EIR contains a number of proposed

mitigation actions, including fisheries monitoring in the lower San Joaquin

River, as part of the South Delta barrier project.    Although these

discussions occurred in the draft ISDP environmental document, they may or

may not be included as part of a final project.    Rather than rely on the

draft ISDP environmental work, the Department of Water Resources should be

consulted for the most recent information regarding proposals for

monitoring and mitigation associated with the South Delta barrier project.

Please call me if you have any questions about any of these comments.

Sincerely,

Steve Macaulay

General Manager

C:    	Member Agencies

Bay-Delta Workgroup